W7500x

Reference Manual

Version 1.1.1

http://www.wiznet.io

Table of Contents

1	Table of Contents	2

2	List of table	19

3	List of figures	21

4	Documentation conventions	23

4.1	Glossary	23

4.2	Register Bit Conventions	25

5	System and memory overview	26

5.1	System architecture	26

5.2	Memory organization	27

Introduction	27

Memory map	28

6	System configuration controller (SYSCFG)	29

7	Interrupt and events	29

7.1	Nested vectored interrupt controller (NVIC)	29

NVIC main features	29

SysTick calibration value register	29

Interrupt and exception vectors	29

7.2	Event	30

8	Power supply	31

8.1	Introduction	31

8.2	Voltage regulator	31

8.3	Low-power modes	31

Sleep mode	32

Peripheral clock gating	32

9	System tick timer	33

9.1	Introduction	33

9.2	Features	33

9.3	Functional description	33

9.4	Registers (Base : 0xE000_E000)	34

System Timer control and status register (SYST_CSR)	34

SysTick Reload Value Register (SYST_RVR)	34

SysTick Current Value Register (SYST_CVR)	35

SysTick Calibration Value Register (SYST_CALIB)	35

10	Booting Sequence	36

11	Embedded Flash memory	37

11.1	Flash main features	37

11.2	Flash memory organization	37

12	Clock Reset generator (CRG)	39

12.1	Introduction	39

12.2	Features	39

Reset.......	39

Clock.......	40

12.3	Functional description	41

External Oscillator Clock	41

RC oscillator clock	42

PLL.........	42

Generated clock	42

12.4	Registers (Base address : 0x4100_1000)	43

OSC power down register (OSC_PDR)	43

PLL power down register (PLL_PDR)	43

PLL frequency calculating register (PLL_FCR)	43

PLL output enable register (PLL_OER)	44

PLL bypass register (PLL_BPR)	44

PLL input clock source select register (PLL_IFSR)	45

FCLK source select register (FCLK_SSR)	45

FCLK prescale value select register (FCLK_PVSR)	46

SSPCLK source select register (SSPCLK_SSR)	46

SSPCLK prescale value select register (SSPCLK_PVSR)	47

ADCCLK source select register (ADCCLK_SSR)	47

ADCCLK prescale value select register (ADCCLK_PVSR)	47

TIMER0CLK source select register (TIMER0CLK_SSR)	48

TIMER0CLK prescale value select register (TIMER0CLK_PVSR)	48

TIMER1CLK source select register (TIMER1CLK_SSR)	49

TIMER1CLK prescale value select register (TIMER1CLK_PVSR)	49

PWM0CLK source select register (PWM0CLK_SSR)	50

PWM0CLK prescale value select register (PWM0CLK_PVSR)	50

PWM1CLK source select register (PWM1CLK_SSR)	51

PWM1CLK prescale value select register (PWM1CLK_PVSR)	51

PWM2CLK source select register (PWM2CLK_SSR)	52

PWM2CLK prescale value select register (PWM2CLK_PVSR)	52

PWM3CLK source select register (PWM3CLK_SSR)	53

PWM3CLK prescale value select register (PWM3CLK_PVSR)	54

PWM4CLK source select register (PWM4CLK_SSR)	54

PWM4CLK prescale value select register (PWM4CLK_PVSR)	55

PWM5CLK source select register (PWM5CLK_SSR)	55

PWM5CLK prescale value select register (PWM5CLK_PVSR)	56

PWM6CLK source select register (PWM6CLK_SSR)	56

PWM6CLK prescale value select register (PWM6CLK_PVSR)	57

PWM7CLK source select register (PWM7CLK_SSR)	57

PWM7CLK prescale value select register (PWM7CLK_PVSR)	58

RTC High Speed source select register (RTC_HS_SSR)	58

RTC High Speed prescale value select register (RTC_HS_PVSR)	59

RTC source select register (RTC_SSR)	59

WDOGCLK High Speed source select register (WDOGCLK_HS_SSR)	60

WDOGCLK High Speed prescale value select register (WDOGCLK_HS_PVSR)	60

WDOGCLK clock source select register (WDOGCLK_SSR)	61

UARTCLK source select register (UARTCLK_SSR)	61

UARTCLK prescale value select register (UARTCLK_PVSR)	62

MIICLK enable control register (MIICLK_ECR)	62

Monitoring Clock source select register (MONCLK_SSR)	63

12.5	Register map	64

13	Tcp/ip core Offload Engine (TOE)	66

13.1	Introduction	66

13.2	Features	66

13.3	Functional description	67

13.4	TOE Memory map	67

Common register map	69

Socket register map	69

Memory........	70

13.5	Common register (Base : 0x4600_0000)	72

VERSIONR (TOE Version Register)	72

TCKCNTR (Ticker Counter Register)	72

IR (Interrupt Register)	73

IMR (Interrupt Mask Register)	73

IRCR (Interrupt Clear Register)	74

SIR (Socket Interrupt Register)	75

SIMR (Socket Interrupt Mask Register)	75

MR (Mode Register)	76

PTIMER (PPP Link Control Protocol Request Timer Register)	77

PMAGICR (PPP Link Control Protocol Magic number Register)	77

PHAR (Destination Hardware Address Register in PPPoE)	77

PSIDR (Session ID Register in PPPoE)	78

PMRUR (Maximum Receive Unit Register in PPPoE)	79

SHAR (Source Hardware Address Register)	79

GAR (Gateway Address)	80

SUBR ( Subnet Mask Register)	80

SIPR (Source IP address Register)	81

NCONFLR ( Network Configuration Lock Register)	81

RTR (Retry Time Register)	82

RCR (Retry Counter Register)	83

UIPR (Unreachable IP address Register)	84

UPORTR (Unreachable Port Register)	85

13.6	Socket register (Base : 0x4601_0000 + 0x0004_000 x n)[n=0,…7, where n is socket number]	85

Sn_MR (Socket n Mode Register)	85

Sn_CR (Socket n Command Register)	88

Sn_IR (Socket n Interrupt Register)	90

Sn_IMR (Socket n Interrupt Mask Register)	90

Sn_ICR (Socket n Interrupt Clear Register)	91

Sn_SR (Socket n Status Register)	92

Sn_PNR (Socket n Protocol Number Register)	94

Sn_TOSR (Socket n IP Type of Service Register)	95

Sn_TTLR (Socket n TTL Register)	95

Sn_FRAGR (Socket n Fragment offset Register)	96

Sn_MSSR (Socket n Maximum Segment Register)	96

Sn_PORTR (Socket n Source Port Register)	97

Sn_DHAR (Socket n Destination Hardware address Register)	97

Sn_DPORTR (Socket n Destination Port Number Register)	98

Sn_DIPR (Socket n Destination IP address Register)	99

Sn_KATMR (Socket n Keep Alive Timer Register)	100

Sn_RTR (Socket n  Retry Time Register)	100

Sn_RCR (Socket n Retry Counter Register)	101

Sn_TXBUF_SIZE (Socket n TX Buffer Size Register)	101

Sn_TX_FSR (Socket n TX Free Size Register)	102

Sn_TX_RD (Socket n TX Read Pointer Register)	103

Sn_TX_WR (Socket n TX Write Pointer Register)	104

Sn_RXBUF_SIZE (Socket n RX Buffer Size Register)	104

Sn_RX_RSR (Socket n RX Received Size Register)	105

Sn_RX_RD (Socket n RX Read Pointer Register)	106

Sn_RX_WR (Socket n RX Write Pointer Register)	107

14	Random number generator (RNG)	108

14.1	Introduction	108

14.2	Features	108

14.3	Functional description	108

Operation RNG	109

14.4	Registers (Base address : 0x4000_7000)	110

RNG run register (RNG_RUN)	110

RNG SEED register (RNG_SEED)	110

RNG clock select register (RNG_CLKSEL)	110

RNG manual mode select register (RNG_MODE)	111

RNG random number value register (RNG_RN)	111

RNG polynomial register (RNG_POLY)	112

14.5	Register map	113

15	Alternate Function Controller (AFC)	113

15.1	Introduction	113

15.2	Features	113

15.3	Functional description	113

15.4	Registers (Base address : 0x4100_2000)	116

PA_00 pad alternate function select register (PA_00_AFR)	116

PA_01 pad alternate function select register (PA_01_AFR)	116

PA_02 pad alternate function select register (PA_02_AFR)	117

PA_03 pad alternate function select register (PA_03_AFR)	117

PA_04 pad alternate function select register (PA_04_AFR)	118

PA_05 pad alternate function select register (PA_05_AFR)	118

PA_06 pad alternate function select register (PA_06_AFR)	119

PA_07 pad alternate function select register (PA_07_AFR)	119

PA_08 pad alternate function select register (PA_08_AFR)	120

PA_09 pad alternate function select register (PA_09_AFR)	120

PA_10 pad alternate function select register (PA_10_AFR)	121

PA_11 pad alternate function select register (PA_11_AFR)	121

PA_12 pad alternate function select register (PA_12_AFR)	122

PA_13 pad alternate function select register (PA_13_AFR)	122

PA_14 pad alternate function select register (PA_14_AFR)	123

PA_15 pad alternate function select register (PA_15_AFR)	123

PB_00 pad alternate function select register (PB_00_AFR)	124

PB_01 pad alternate function select register (PB_01_AFR)	124

PB_02 pad alternate function select register (PB_02_AFR)	125

PB_03 pad alternate function select register (PB_03_AFR)	125

PB_04 pad alternate function select register (PB_04_AFR)	126

PB_05 pad alternate function select register (PB_05_AFR)	126

PB_06 pad alternate function select register (PB_06_AFR)	127

PB_07 pad alternate function select register (PB_07_AFR)	127

PB_08 pad alternate function select register (PB_08_AFR)	128

PB_09 pad alternate function select register (PB_09_AFR)	128

PB_10 pad alternate function select register (PB_10_AFR)	129

PB_11 pad alternate function select register (PB_11_AFR)	129

PB_12 pad alternate function select register (PB_12_AFR)	130

PB_13 pad alternate function select register (PB_13_AFR)	130

PB_14 pad alternate function select register (PB_14_AFR)	131

PB_15 pad alternate function select register (PB_15_AFR)	131

PC_00 pad alternate function select register (PC_00_AFR)	132

PC_01 pad alternate function select register (PC_01_AFR)	132

PC_02 pad alternate function select register (PC_02_AFR)	133

PC_03 pad alternate function select register (PC_03_AFR)	133

PC_04 pad alternate function select register (PC_04_AFR)	134

PC_05 pad alternate function select register (PC_05_AFR)	134

PC_06 pad alternate function select register (PC_06_AFR)	135

PC_07 pad alternate function select register (PC_07_AFR)	135

PC_08 pad alternate function select register (PC_08_AFR)	136

PC_09 pad alternate function select register (PC_09_AFR)	136

PC_10 pad alternate function select register (PC_10_AFR)	137

PC_11 pad alternate function select register (PC_11_AFR)	137

PC_12 pad alternate function select register (PC_12_AFR)	138

PC_13 pad alternate function select register (PC_13_AFR)	138

PC_14 pad alternate function select register (PC_14_AFR)	139

PC_15 pad alternate function select register (PC_15_AFR)	139

PD_00 pad alternate function select register (PD_00_AFR)	140

PD_01 pad alternate function select register (PD_01_AFR)	140

PD_02 pad alternate function select register (PD_02_AFR)	141

PD_03 pad alternate function select register (PD_03_AFR)	141

PD_04 pad alternate function select register (PD_04_AFR)	142

15.5	Register map	143

16	External Interrupt (EXTI)	146

16.1	Introduction	146

16.2	Features	146

16.3	Functional description	146

16.4	Registers (Base address : 0x4100_2000)	148

External interrupt enable register (Px_y EXTINT)	148

16.5	Register map	148

17	Pad Controller (PADCON)	149

17.1	Introduction	149

17.2	Features	149

17.3	Functional description	149

17.4	Registers (Base address : 0x4100_3000)	151

PAD Control register (Px_y PCR)(x=A..D, y=0..15)	151

17.5	Register map	151

18	General-purpose I/Os(GPIO)	152

18.1	Introduction	152

18.2	Features	152

18.3	Functional description	152

Masked access	153

18.4	GPIO Registers(Address Base: 0x4200_0000)	155

GPIO Data Register(GPIOx_DATA) (x=A..D)	155

GPIO Output Latch Register(GPIOx_DATAOUT) (x=A..D)	155

GPIO Enable Set Register(GPIOx_OUTENSET) (x=A..D)	155

GPIO Enable Clear Register(GPIOx_OUTENCLR) (x=A..D)	156

GPIO Interrupt Enable Set Register(GPIOx_ INTENSET) (x=A..D)	156

GPIO Interrupt Enable Clear Register(GPIOx_ INTENCLR) (x=A..D)	157

GPIO Interrupt Type Set Register(GPIOx_ INTTYPESET) (x=A..D)	157

GPIO Interrupt Type Clear Register(GPIOx_ INTTYPECLR) (x=A..D)	158

GPIO Interrupt Polarity Set Register(GPIOx_ INTPOLSET) (x=A..D)	158

GPIO Interrupt Polarity Clear Register(GPIOx_ INTPOLCLR) (x=A..D)	159

GPIO Interrupt Status/Clear Register(GPIO_ INTSTATUS/INTCLEAR) (x=A..D)	160

GPIO Lower Byte Masked Access Register(GPIOx_ LB_MASKED) (x=A..D)	160

GPIO Upper Byte Masked Access Register(GPIOx_ UB_MASKED) (x=A..D)	161

18.5	Register map	162

19	Direct memory access controller (DMA)	163

19.1	Introduction	163

19.2	Features	163

19.3	Functional description	163

DMA request mapping	164

DMA arbitration	164

DMA cycle types	164

19.4	Registers (Base address : 0x4100_4000)	167

DMA status register (DMA_STATUS)	167

DMA configuration register (DMA_CFG)	168

DMA control data base pointer register (DMA_CTRL_BASE_PTR)	169

DMA channel alternate control data base pointer register (DMA_ALT_CTRL_BASE_PTR)	169

DMA channel wait on request status register (DMA_WAITONREQ_STATUS)	170

DMA channel software request register (DMA_CHNL_SW_REQUEST)	170

DMA channel useburst set register (DMA_CHNL_USEBURST_SET)	171

DMA channel useburst clear register (DMA_CHNL_USEBURST_CLR)	171

DMA channel request mask set register (DMA_CHNL_REQ_MASK_SET)	172

DMA channel request mask clear register (DMA_CHNL_REQ_MASK_CLR)	173

DMA channel enable set register (DMA_CHNL_ENABLE_SET)	173

DMA channel enable clear register (DMA_CHNL_ENABLE_CLR)	174

DMA channel primary-alternate set register (DMA_CHNL_PRI_ALT_SET)	174

DMA channel primary-alternate clear register (DMA_CHNL_PRI_ALT _CLR)	175

DMA channel priority set register (DMA_CHNL_PRIORITY_SET)	175

DMA channel priority clear register (DMA_CHNL_PRIORITY_CLR)	176

DMA bus error clear register (DMA_ERR_CLR)	176

19.5	Register map	178

20	Analog-to-digital converter (ADC)	179

20.1	Introduction	179

20.2	Features	179

20.3	Functional description	180

Operation ADC with non-interrupt	180

Operation ADC with interrupt	182

20.4	Registers (Base address : 0x4100_0000)	182

ADC control register (ADC_CTR)	182

ADC channel select register (ADC_CHSEL)	183

ADC start register (ADC_START)	184

ADC conversion data register (ADC_DATA)	184

ADC Interrupt register (ADC_INT)	184

ADC Interrupt Clear register (ADC_INTCLR)	185

20.5	Register map	186

21	Pulse-Width Modulation (PWM)	187

21.1	Introduction	187

21.2	Features	187

21.3	Functional description	188

Timer/Counter control	188

Timer/Counter	188

PWM mode	192

Interrupt....	193

Dead zone generation	194

Capture event	195

How to set the PWM	197

21.4	PWM Channel-0 Registers (Base address : 0x4000_5000)	197

Channel-0 interrupt register(PWMCH0IR)	197

Channel-0 interrupt enable register(PWMCH0IER)	198

Channel-0 interrupt clear register(PWMCH0ICR)	199

Channel-0 Timer/Counter Register (PWMCH0TCR)	199

Channel-0 Prescale Counter Register (PWMCH0PCR)	200

Channel-0 Prescale Register (PWMCH0PR)	200

Channel-0 Match Register (PWMCH0MR)	200

Channel-0 Limit Register (PWMCH0LR)	201

Channel-0 Up-Down Mode Register (PWMCH0UDMR)	201

Channel-0 Timer/Counter Mode Register (PWMCH0TCMR)	202

Channel-0 PWM output Enable and External input Enable Register (PWMCH0PEEER)	202

Channel-0 Capture Mode Register (PWMCH0CMR)	203

Channel-0 Capture Register (PWMCH0CR)	203

Channel-0 Periodic Mode Register (PWMCH0PDMR)	203

Channel-0 Dead Zone Enable Register (PWMCH0DZER)	204

Channel-0 Dead Zone Counter Register (PWMCH0DZCR)	204

21.5	Register map	205

21.6	PWM Channel-1 Registers (Base address : 0x4000_5100)	207

Channel-1 interrupt register(PWMCH1IR)	207

Channel-1 interrupt enable register(PWMCH1IER)	207

Channel-1 interrupt clear register(PWMCH1ICR)	208

Channel-1 Timer/Counter Register (PWMCH1TCR)	208

Channel-1 Prescale Counter Register (PWMCH1PCR)	209

Channel-1 Prescale Register (PWMCH1PR)	209

Channel-1 Match Register (PWMCH1MR)	210

Channel-1 Limit Register (PWMCH1LR)	210

Channel-1 Up-Down Mode Register (PWMCH1UDMR)	211

Channel-1 Timer/Counter Mode Register (PWMCH1TCMR)	211

Channel-1 PWM output Enable and External input Enable Register (PWMCH1PEEER)	212

Channel-1 Capture Mode Register (PWMCH1CMR)	212

Channel-1 Capture Register (PWMCH1CR)	212

Channel-1 Periodic Mode Register (PWMCH1PDMR)	213

Channel-1 Dead Zone Enable Register (PWMCH1DZER)	213

Channel-1 Dead Zone Counter Register (PWMCH1DZCR)	214

21.7	Register map	215

21.8	PWM Channel-2 Registers (Base address : 0x4000_5200)	216

Channel-2 interrupt register(PWMCH2IR)	216

Channel-2 interrupt enable register(PWMCH2IER)	216

Channel-2 interrupt clear register(PWMCH2ICR)	217

Channel-2 Timer/Counter Register (PWMCH2TCR)	217

Channel-2 Prescale Counter Register (PWMCH2PCR)	218

Channel-2 Prescale Register (PWMCH2PR)	218

Channel-2 Match Register (PWMCH2MR)	219

Channel-2 Limit Register (PWMCH2LR)	219

Channel-2 Up-Down Mode Register (PWMCH2UDMR)	219

Channel-2 Timer/Counter Mode Register (PWMCH2TCMR)	220

Channel-2 PWM output Enable and External input Enable Register (PWMCH2PEEER)	220

Channel-2 Capture Mode Register (PWMCH2CMR)	221

Channel-2 Capture Register (PWMCH2CR)	221

Channel-2 Periodic Mode Register (PWMCH2PDMR)	222

Channel-2 Dead Zone Enable Register (PWMCH2DZER)	222

Channel-2 Dead Zone Counter Register (PWMCH2DZCR)	223

21.9	Register map	224

21.10	PWM Channel-3 Registers (Base address : 0x4000_5300)	225

Channel-3 interrupt register(PWMCH3IR)	225

Channel-3 interrupt enable register(PWMCH3IER)	225

Channel-3 interrupt clear register(PWMCH3ICR)	226

Channel-3 Timer/Counter Register (PWMCH3TCR)	226

Channel-3 Prescale Counter Register (PWMCH3PCR)	227

Channel-3 Prescale Register (PWMCH3PR)	227

Channel-3 Match Register (PWMCH3MR)	228

Channel-3 Limit Register (PWMCH3LR)	228

Channel-3 Up-Down Mode Register (PWMCH3UDMR)	229

Channel-3 Timer/Counter Mode Register (PWMCH3TCMR)	229

Channel-3 PWM output Enable and External input Enable Register (PWMCH3PEEER)	230

Channel-3 Capture Mode Register (PWMCH3CMR)	230

Channel-3 Capture Register (PWMCH3CR)	230

Channel-3 Periodic Mode Register (PWMCH3PDMR)	231

Channel-3 Dead Zone Enable Register (PWMCH3DZER)	231

Channel-3 Dead Zone Counter Register (PWMCH3DZCR)	232

21.11	Register map	233

21.12	PWM Channel-4 Registers (Base address : 0x4000_5400)	234

Channel-4 interrupt register(PWMCH4IR)	234

Channel-4 interrupt enable register(PWMCH4IER)	234

Channel-4 interrupt clear register(PWMCH4ICR)	235

Channel-4 Timer/Counter Register (PWMCH4TCR)	235

Channel-4 Prescale Counter Register (PWMCH4PCR)	236

Channel-4 Prescale Register (PWMCH4PR)	236

Channel-4 Match Register (PWMCH4MR)	237

Channel-4 Limit Register (PWMCH4LR)	237

Channel-4 Up-Down Mode Register (PWMCH4UDMR)	237

Channel-4 Timer/Counter Mode Register (PWMCH4TCMR)	238

Channel-4 PWM output Enable and External input Enable Register (PWMCH4PEEER)	238

Channel-4 Capture Mode Register (PWMCH4CMR)	239

Channel-4 Capture Register (PWMCH4CR)	239

Channel-4 Periodic Mode Register (PWMCH4PDMR)	240

Channel-4 Dead Zone Enable Register (PWMCH4DZER)	240

Channel-4 Dead Zone Counter Register (PWMCH4DZCR)	241

21.13	Register map	242

21.14	PWM Channel-5 Registers (Base address : 0x4000_5500)	243

Channel-5 interrupt register(PWMCH5IR)	243

Channel-5 interrupt enable register(PWMCH5IER)	243

Channel-5 interrupt clear register(PWMCH5ICR)	244

Channel-5 Timer/Counter Register (PWMCH5TCR)	244

Channel-5 Prescale Counter Register (PWMCH5PCR)	245

Channel-5 Prescale Register (PWMCH5PR)	245

Channel-5 Match Register (PWMCH5MR)	246

Channel-5 Limit Register (PWMCH5LR)	246

Channel-5 Up-Down Mode Register (PWMCH5UDMR)	246

Channel-5 Timer/Counter Mode Register (PWMCH5TCMR)	247

Channel-5 PWM output Enable and External input Enable Register (PWMCH5PEEER)	247

Channel-5 Capture Mode Register (PWMCH5CMR)	248

Channel-5 Capture Register (PWMCH5CR)	248

Channel-5 Periodic Mode Register (PWMCH5PDMR)	249

Channel-5 Dead Zone Enable Register (PWMCH5DZER)	249

Channel-5 Dead Zone Counter Register (PWMCH5DZCR)	250

21.15	Register map	251

21.16	PWM Channel-6 Registers (Base address : 0x4000_5600)	252

Channel-6 interrupt register(PWMCH6IR)	252

Channel-6 interrupt enable register(PWMCH6IER)	252

Channel-6 interrupt clear register(PWMCH6ICR)	253

Channel-6 Timer/Counter Register (PWMCH6TCR)	253

Channel-6 Prescale Counter Register (PWMCH6PCR)	254

Channel-6 Prescale Register (PWMCH6PR)	254

Channel-6 Match Register (PWMCH6MR)	255

Channel-6 Limit Register (PWMCH6LR)	255

Channel-6 Up-Down Mode Register (PWMCH6UDMR)	256

Channel-6 Timer/Counter Mode Register (PWMCH6TCMR)	256

Channel-6 PWM output Enable and External input Enable Register (PWMCH6PEEER)	257

Channel-6 Capture Mode Register (PWMCH6CMR)	257

Channel-6 Capture Register (PWMCH6CR)	257

Channel-6 Periodic Mode Register (PWMCH6PDMR)	258

Channel-6 Dead Zone Enable Register (PWMCH6DZER)	258

Channel-6 Dead Zone Counter Register (PWMCH6DZCR)	259

21.17	Register map	260

21.18	PWM Channel-7 Registers (Base address : 0x4000_5700)	261

Channel-7 interrupt register(PWMCH7IR)	261

Channel-7 interrupt enable register(PWMCH7IER)	261

Channel-7 interrupt clear register(PWMCH7ICR)	262

Channel-7 Timer/Counter Register (PWMCH7TCR)	262

Channel-7 Prescale Counter Register (PWMCH7PCR)	263

Channel-7 Prescale Register (PWMCH7PR)	263

Channel-7 Match Register (PWMCH7MR)	264

Channel-7 Limit Register (PWMCH7LR)	264

Channel-7 Up-Down Mode Register (PWMCH7UDMR)	265

Channel-7 Timer/Counter Mode Register (PWMCH7TCMR)	265

Channel-7 PWM output Enable and External input Enable Register (PWMCH7PEEER)	266

Channel-7 Capture Mode Register (PWMCH7CMR)	266

Channel-7 Capture Register (PWMCH7CR)	266

Channel-7 Periodic Mode Register (PWMCH7PDMR)	267

Channel-7 Dead Zone Enable Register (PWMCH7DZER)	267

Channel-7 Dead Zone Counter Register (PWMCH7DZCR)	268

21.19	Register map	269

21.20	PWM Common Registers (Base address : 0x4000_5800)	270

Interrupt Enable Register (IER)	270

Start/Stop Register (SSR)	271

Pause Register (PSR)	272

21.21	Register map	273

22	Dual timers	274

22.1	Introduction	274

22.2	Features	274

22.3	Functional description	275

Clock and clock enable	275

Timer size....	275

Prescaler....	275

Repetition mode	275

Interrupt....	276

Operation....	276

How to set the dual timers	277

22.4	Dual timer0_0 Registers (Base address : 0x4000_1000)	278

Timer0_0 Load Register(DUALTIMER0_0TimerLoad)	278

Timer0_0 Value Register(DUALTIMER0_0TimerValue)	278

Timer0_0 Control Register(DUALTIMER0_0TimerControl)	278

Timer0_0 Interrupt Clear Register (DUALTIMER0_0TimerIntClr)	279

Timer0_0 Raw Interrupt Status Register (DUALTIMER0_0TimerRIS)	279

Timer0_0 Masked Interrupt Status Register (DUALTIMER0_0TimerMIS)	280

Timer0_0 Background Load Register (DUALTIMER0_0TimerBGLoad)	280

22.5	Register map	282

22.6	Dual timer0_1 Registers (Base address : 0x4000_1020)	283

Timer0_1 Load Register(DUALTIMER0_1TimerLoad)	283

Timer0_1 Value Register(DUALTIMER0_1TimerValue)	283

Timer0_1 Control Register(DUALTIMER0_1TimerControl)	283

Timer0_1 Interrupt Clear Register (DUALTIMER0_1TimerIntClr)	284

Timer0_1 Raw Interrupt Status Register (DUALTIMER0_1TimerRIS)	284

Timer0_1 Masked Interrupt Status Register (DUALTIMER0_1TimerMIS)	285

Timer0_1 Background Load Register (DUALTIMER0_1TimerBGLoad)	285

22.7	Register map	287

22.8	Dual Timer 0 Clock Enable Register (Base address : 0x4000_1080)	288

Timer0_0 Clock Enable Register (TIMCLKEN0_0)	288

Timer0_1 Clock Enable Register (TIMCLKEN0_1)	288

22.9	Register map	289

22.10	Dual timer1_0 Registers (Base address : 0x4000_2000)	290

Timer1_0 Load Register(DUALTIMER1_0TimerLoad)	290

Timer1_0 Value Register(DUALTIMER1_0TimerValue)	290

Timer1_0 Control Register(DUALTIMER1_0TimerControl)	290

Timer1_0 Interrupt Clear Register (DUALTIMER1_0TimerIntClr)	291

Timer1_0 Raw Interrupt Status Register (DUALTIMER1_0TimerRIS)	291

Timer1_0 Masked Interrupt Status Register (DUALTIMER1_0TimerMIS)	292

Timer1_0 Background Load Register (DUALTIMER1_0TimerBGLoad)	292

22.11	Register map	294

22.12	Dual timer1_1 Registers (Base address : 0x4000_2020)	295

Timer1_1 Load Register(DUALTIMER1_1TimerLoad)	295

Timer1_1 Value Register(DUALTIMER1_1TimerValue)	295

Timer1_1 Control Register(DUALTIMER1_1TimerControl)	295

Timer1_1 Interrupt Clear Register (DUALTIMER1_1TimerIntClr)	296

Timer1_1 Raw Interrupt Status Register (DUALTIMER1_1TimerRIS)	296

Timer1_1 Masked Interrupt Status Register (DUALTIMER1_1TimerMIS)	297

Timer1_1 Background Load Register (DUALTIMER1_1TimerBGLoad)	297

22.13	Register map	299

22.14	Dual Timer 1 Clock Enable Register (Base address : 0x4000_2080)	300

Timer1_0 Clock Enable Register (TIMCLKEN1_0)	300

Timer1_1 Clock Enable Register (TIMCLKEN1_1)	300

22.15	Register map	301

23	Watchdog timer	301

23.1	Introduction	301

23.2	Features	301

23.3	Functional description	301

Clock........	301

Interrupt and reset request	302

23.4	Watchdog timer Registers (Base address : 0x4000_0000)	302

Watchdog timer Load Register(WDTLoad)	302

Watchdog timer Value Register(WDTValue)	303

Watchdog timer Control Register(WDTControl)	303

Watchdog timer Interrupt Clear Register (WDTIntClr)	303

Watchdog timer Raw Interrupt Status Register (WDTRIS)	304

Watchdog timer Masked Interrupt Status Register (WDTMIS)	304

Watchdog timer Lock Register(WDTLock)	305

23.5	Register map	306

24	Real-time Clock(RTC)	306

24.1	Introduction	306

24.2	Features	306

24.3	Functional description	306

RTC clock....	307

RTC interrupt	307

RTC Counter and Calendar	307

RTC Setting flow	308

24.4	RTC Registers (Base Address : 0x4000_E000)	309

RTC control register (RTCCON)	309

RTC Interrupt Mask register (RTCINTE)	310

RTC Interrupt Pending register (RTCINTP)	312

RTC Alarm Mask register (RTCAMR)	313

RTC BCD Second register (BCDSEC)	314

RTC BCD Minute register (BCDMIN)	314

RTC BCD Hour register (BCDHOUR)	314

RTC BCD Day register (BCDDAY)	315

RTC BCD Date register (BCDDATE)	315

RTC BCD Month register (BCDMON)	315

RTC BCD Year register (BCDYEAR)	316

RTC Predetermining Second register (PRESEC)	316

RTC Predetermining Minute register (PREMIN)	316

RTC Predetermining Hour register (PREHOUR)	317

RTC Predetermining Day register (PREDAY)	317

RTC Predetermining Date register (PREDATE)	317

RTC Predetermining Month register (PREMON)	318

RTC Predetermining Year register (PREYEAR)	318

RTC Consolidated Time0 register (RTCTIME0)	318

RTC Consolidated Time1 register (RTCTIME1)	319

24.5	Register map	320

25	UART(Universal Asynchronous Receive Transmit)	321

25.1	Introduction	321

25.2	Features	321

25.3	Functional description	322

Baud rate calculation	324

Data transmission	325

Data receive	325

Hardware flow control	326

25.4	UART0 Registers(Base address: 0x4000_C000)	327

UART0DR (UART0 Data Register)	327

UART0RSR/ECR (UART0 Receive Status Register/Error Clear Register)	328

UART0FR (UART0 Flag Register)	329

UART0ILPR (UART0 IrDA Low-Power Counter Register)	330

UART0IBRD (UART0 Integer Baud Rate Register)	331

UART0FBRD (UART0 Fractional Baud Rate Register)	331

UART0LCR_H (UART0 Line Control Register)	332

UART0CR (UART0 Control register)	333

UART0IFLS (UART0 Interrupt FIFO Level Select Register)	335

UART0IMSC (UART0 Interrupt Mask Set/Clear Register)	335

UART0RIS (UART0 Raw Interrupt Status Register)	337

UART0MIS (UART0 Masked Interrupt Status Register)	338

UART0ICR (UART0 Interrupt Clear Register)	339

25.5	Register map	340

25.6	UART1 Registers(Base address: 0x4000_D000)	341

UART1DR (UART1 Data Register)	341

UART1RSR/ECR (UART1 Receive Status Register/Error Clear Register)	341

UART1FR (UART1 Flag Register)	342

UART1ILPR (UART1 IrDA Low-Power Counter Register)	343

UART1IBRD (UART1 Integer Baud Rate Register)	344

UART1FBRD (UART1 Fractional Baud Rate Register)	344

UART1LCR_H (UART1 Line Control Register)	345

UART1CR (UART1 Control register)	346

UART1IFLS (UART1 Interrupt FIFO Level Select Register)	348

UART1IMSC (UART1 Interrupt Mask Set/Clear Register)	348

UART1RIS (UART1 Raw Interrupt Status Register)	350

UART1MIS (UART1 Masked Interrupt Status Register)	351

UART1ICR (UART1 Interrupt Clear Register)	352

25.7	Register map	353

26	Universal Asynchronous Receive Transmit(UART2)	354

26.1	Introduction	354

26.2	Feature	354

26.3	Functional description	354

Baud rate calculation	354

26.4	UART2 Registers(Base address: 0x4000_6000)	356

UART2DR (UART2 Data Register)	356

UART2SR (UART2 Status Register)	356

UART2CR (UART2 Control Register)	357

UART2ISR/ICR (UART2 Interrupt Status/Interrupt Clear Register)	357

UART2BDR (UART2 Baud Rate Divider Register)	358

26.5	Register map	359

27	Synchronous Serial Port (SSP)	360

27.1	Introduction	360

27.2	Features	360

27.3	Functional description	361

Clock prescaler	361

Transmit FIFO	361

Receive FIFO	362

Interrupt generation logic	362

DMA interface	362

Interface reset	364

Configuring the SSP	364

Enable PrimeCell SSP operation	365

Clock ratios	365

Programming the SSPCR0 Control Register	366

Programming the SSPCR1 Control Register	366

Frame format	367

Texas Instruments synchronous serial frame format	368

Motorola SPI frame format	369

National Semiconductor Microwire frame format	375

Master and Slave configurations	377

SSP Flow chart	378

27.4	SSP0 Registers (Base Address : 0x4000_A000)	379

SSP0 Control register 0 (SSP0CR0)	379

SSP0 Control register 1 (SSP0CR1)	381

SSP0 Data register (SSP0DR)	381

SSP0 Status register (SSP0SR)	382

SSP0 Clock prescale register (SSP0CPSR)	383

SSP0 Interrupt mask set or clear register (SSP0IMSC)	383

SSP0 Raw interrupt status register (SSP0RIS)	384

SSP0 Masked interrupt status register, (SSP0MIS)	384

SSP0 Interrupt clear register (SSP0ICR)	385

SSP0 DMA control register, (SSP0DMACR)	385

27.5	Register map	387

27.6	SSP1 Registers (Base Address : 0x4000_B000)	388

SSP1 Control register 0 (SSP1CR0)	388

SSP1 Control register 1 (SSP1CR1)	389

SSP1 Data register (SSP1DR)	391

SSP1 Status register (SSP1SR)	391

SSP1 Clock prescale register (SSP1CPSR)	392

SSP1 Interrupt mask set or clear register (SSP1IMSC)	392

SSP1 Raw interrupt status register (SSP1RIS)	393

SSP1 Masked interrupt status register, (SSP1MIS)	393

SSP1 Interrupt clear register (SSP1ICR)	394

SSP1 DMA control register, (SSP1DMACR)	394

27.7	Register map	396

Document History Information	397

List of table

Table 1 W7500x interrupt vector table	29

Table 2 W7500x sleep mode summary	32

Table 3 operation of mode selection	36

Table 4 description of Flash memory	37

Table 5 CRG register map and reset values	66

Table 6. Offset Address for Common Register	69

Table 7. Offset Address in Socket n Register Block	70

Table 8  RNG register map and reset values	113

Table 9 functional description table	113

Table 10 AFC register map and reset values	143

Table 11 EXTINT register map and reset values	148

Table 12 PAD controller register map and reset values	151

Table 13 GPIO register map and reset values	162

Table 14 Summary of the DMA requests for each channel	164

Table 15 DMA register map and reset values	178

Table 16 ADC register map and reset values	186

Table 17 PWM channel 0 register map and reset values	206

Table 18 PWM channel 1 register map and reset values	215

Table 19 PWM channel 2 register map and reset values	224

Table 20 PWM channel 3 register map and reset values	233

Table 21 PWM channel 4 register map and reset values	242

Table 22 PWM channel 5 register map and reset values	251

Table 23 PWM channel 6 register map and reset values	260

Table 24 PWM channel 7 register map and reset values	269

Table 25 PWM common register map and reset values	273

Table 26 Dual timer 0_0 register map and reset values	282

Table 27 Dual timer 0_1 register map and reset values	287

Table 28 Dual timer 0 clock enable register map and reset values	289

Table 29 Dual timer 1_0 register map and reset values	294

Table 30 Dual timer 1_1 register map and reset values	299

Table 31 Dual timer 1 clock enable register map and reset values	301

Table 32 Watchdog Timer register map and reset values	306

Table 33 RTC register map and reset values	320

Table 34 UART0 register map and reset values	340

Table 35 UART1 register map and reset values	353

Table 36 UART2 register map and reset values	359

Table 37 DMA trigger points for the transmit and receive FIFOs.	363

Table 38 SSP0 register map and reset values	387

Table 39 SSP1 register map and reset values	396

List of figures

Figure 1. W7500x System Architecture	26

Figure 2 W7500x memory map	28

Figure 3. operation of boot code	36

Figure 4 CRG block diagram	41

Figure 5 Typical application with an 8 MHz crystal	41

Figure 6 TOE block diagram	67

Figure 7. Register & Memory Organization	68

Figure 8. Random Number Generator block diagram	108

Figure 9. Flow chart of RNG operation	109

Figure 10. External Interrupt diagram	147

Figure 11. function schematic of digital I/O pad	149

Figure 12. function schematic of digital/analog mux IO pad	149

Figure 13. GPIO block diagram	152

Figure 17. GPIO Flow chart	153

Figure 18. MASK LOWBYTE access	154

Figure 19 MASK HIGHBYTE access	154

Figure 17. DMA Block diagram	163

Figure 18. DMA ping pong cycle	167

Figure 19. ADC block diagram	180

Figure 20. The ADC operation flowchart with non-interrupt	181

Figure 21. The ADC operation flowchart with interrupt	182

Figure 22 PWM block diagram	188

Figure 23 Periodic mode	189

Figure 24 one-shot mode	189

Figure 25 Up-count mode	189

Figure 26 Down-count mode	189

Figure 27 Counter mode with rising edge	190

Figure 28 Counter mode with falling edge	190

Figure 29 Counter mode with rising and falling edge	191

Figure 30 Timer/Counter timing diagram with match interrupt	191

Figure 31 Timer/Counter timing diagram with overflow interrupt	192

Figure 32 The PWM output up to match register	193

Figure 33 The PWM output up to limit register	193

Figure 34 PWM waveform with dead zone time	194

Figure 35 PWM waveform with dead zone counter	195

Figure 36 Capture event with no interrupt clear	195

Figure 37 Capture event with interrupt clear	196

Figure 38 The PWM setting flow	197

Figure 39 Block diagram of Dualtimer	274

Figure 40 The Dual timer setting flow	277

Figure 41 Watchdog timer operation flow diagram	302

Figure 42. RTC block diagram	307

Figure 43. RTC setting flow for Counter Function	308

Figure 44. RTC setting flow for Alarm function	309

Figure 45. UART0,1 Block diagram	322

Figure 46. UART character frame	323

Figure 47. UART divider flow chart	324

Figure 48. UART Initial setting flow chart	324

Figure 49. Transmit and Receive data flow chart	325

Figure 50. Hardware flow control description	326

Figure 51. CTS Functional Timing	326

Figure 52. Algorithm for setting CTS/RTS flowchart	327

Figure 53. UART2 divider flow chart	354

Figure 54. UART2 Initial setting flow chart	355

Figure 55. SSP block diagram	361

Figure 56. DMA transfer waveforms	364

Figure 57. Texas Instruments synchronous serial, single transfer	368

Figure 58. Texas Instruments synchronous serial, continuous transfers	369

Figure 59 Motorola SPI, single transfer, SPO=0 and SPH=0	370

Figure 60 Motorola SPI, continuous transfers, SPO=0 and SPH=0	370

Figure 61 Motorola SPI, single & continuous transfers, SPO=0 and SPH=1	371

Figure 62 Motorola SPI, single transfer, with SPO=1 and SPH=0	372

Figure 63. Motorola SPI, continuous transfers, SPO=1 and SPH=0	373

Figure 64. Motorola SPI, single & continuous transfers, SPO=1 and SPH=1	374

Figure 65. National Semiconductor Microwire, single transfer	375

Figure 66. National Semiconductor Microwire, continuous transfers	377

Figure 67. PrimeCell SSP master coupled to an SPI slave	377

Figure 68. SPI master coupled to a PrimeCell SSP slave	378

Figure 69. how to setting TI or Microwire mode flow chart	378

Figure 70. how to setting SPI mode flow chart	379

- Documentation conventions

  - Glossary

| ARP | Address Resolution Protocol |
| --- | --- |
| AHB | Advanced High-performance Bus |
| AMBA | Advanced Microcontroller Bus Architecture |
| APB | Advanced Peripheral Bus |
| AFC | Alternate Function Controller |
| ADC | Analog-to-Digital Converter |
| BOD | BrownOut Detection |
| CPU | Central Processing Unit |
| CRG | Clock Reset generator |
| DMA | Direct Memory Access |
| EOP | End Of Packet |
| EXTINT | External Interrupt |
| GPIO | General Purpose Input/Output |
| IrDA | Infrared Data Association |
| I/O | Input/Output |
| ICMP | Internet Control Message Protocol |
| IGMP | Internet Group Management Protocol |
| IPv4 | Internet Protocol version 4 |
| IRQ | interrupt request |
| NMI | NonMaskable Interrupt |
| PADCON | Pad Controller |
| PLL | Phase-Locked Loop |
| PHY | Physical Layer |
| PPPoE | Point-to-Point Protocol over Ethernet |
| POR | Power Of Reset |
| PWM | Pulse Width Modulator |
| RAM | Random Access Memory |
| RNG | Random number generator |
| SR | Status Register |
| SSP | Synchronous Serial Port |
| SYSCFG | System configuration controller |
| TOE | TCPIPCore Offload Engine |
| TTL | Transistor-Transistor Logic |
| TCP | Transmission Control Protocol |
| UART | Universal Asynchronous Receiver/Transmitter |
| USB | Universal Serial Bus |
| UDP | User Datagram Protocol |
| WOL | Wake On Lan |
| WDT | Watchdog Timer |

  - Register Bit Conventions

Each register is shown with a key indicating the accessibility of the each individual bit, and the initial condition:

| Key | Bit Accessibility |
| --- | --- |
| rw | Read/Write |
| r | Read Only |
| r0 | Read as 0 |
| r1 | Read as 1 |
| W | Write Only |

System and memory overview

System architecture

Main system consists of :

- Three masters :

- Cortex-M0 core

- TCP/IP Offload Engine

- uDMAC (PL230, 6channel)

- Ten slaves :

- Internal BOOT ROM

- Internal SRAM

- Internal Flash memory

- Two AHB2APB bridge which connects all APB peripherals

- Four AHB dedicated to 16bit GPIOs

- TCPIP Hardware core

- Ethernet :

- IP101G

System architecture and AHB-Lite bus architecture shown in Figure 1.

Figure 1. W7500x System Architecture

AHB-Lite BUS

- This bus connects the Three masters (Cortex-M0 and uDMAC and TCP/IP Offload Engine) and ten AHB slaves.

Two APB BUSs

- These buses connect Seventeen APB peripherals (Watchdog, two Dual timers, PWM, two UARTs, simple UART, two I2Cs, two SSPs, Random Number Generator, Real Time Clock, 12bits Analog Digital Converter, Clock Controller, IO Configuration, PAD MUX controller)

Memory organization

Introduction

Program memory, data memory, registers and I/O ports are organized within the same linear 4-Gbyte address space.

The bytes are coded in memory in Little Endian format. The lowest numbered byte in a word is considered the word’s least significant byte and the highest numbered byte the most significant.

Memory map

Figure 2 W7500x memory map

System configuration controller (SYSCFG)

Main purposes of the system configuration controller are the following

- Control of the memory remap feature

- The ability to enable an automatic reset if the system locks up

- Information about the cause of the last reset

Interrupt and events

Nested vectored interrupt controller (NVIC)

NVIC main features

- 32 maskable interrupt channels (not including the sixteen Cortex®-M0 interrupt lines)

- 4 programmable priority levels (2 bits of interrupt priority are used)

- Low-latency exception and interrupt handling

- Power management control

- Implementation of System Control Registers

The NVIC and the processor core interface are closely coupled, which enables low latency interrupt processing and efficient processing of late arriving interrupts. All interrupts including the core exceptions are managed by the NVIC.

SysTick calibration value register

The SysTick calibration value is set to 6000, which gives a reference time base of 1 ms with the SysTick clock set to 6 MHz (max fHCLK/8).

Interrupt and exception vectors

Table 1 describes the W7500x interrupt vector table.

Table 1 W7500x interrupt vector table

| Priority | Type of priority | Exception type | Device | Description | Address |
| --- | --- | --- | --- | --- | --- |
| -3 | fixed | Reset | CM0 | Reset | 0x0000_0004 |
| -2 | fixed | NMI | Watchdog | Watchdog interrupt | 0x0000_0008 |
| -1 | Fixed | HardFault | CM0 | All class of fault | 0x0000_000C |
| 3 | Settable | SVCall | CM0 | System service call via SWI instruction | 0x0000_002C |
| 5 | settable | PendSV | CM0 | Pendable request for system service | 0x0000_0038 |
| 6 | settable | SysTick | CM0 | System tick timer | 0x0000_003C |
| 7 | settable | IRQ[0] | SSP0 | SSP0 global interrupt | 0x0000_0040 |
| 8 | Settable | IRQ[1] | SSP1 | SSP1 global interrupt | 0x0000_0044 |
| 9 | Settable | IRQ[2] | UART0 | UART0 global interrupt | 0x0000_0048 |
| 10 | Settable | IRQ[3] | UART1 | UART1 global interrupt | 0x0000_004C |
| 11 | Settable | IRQ[4] | UART2 | UART2 global interrupt | 0x0000_0050 |
| 12 | Settable | IRQ[5] | reserved |  | 0x0000_0054 |
| 13 | Settable | IRQ[6] | reserved |  | 0x0000_0058 |
| 14 | Settable | IRQ[7] | GPIO0 | GPIOA global interrupt | 0x0000_005C |
| 15 | Settable | IRQ[8] | GPIO1 | GPIOB global interrupt | 0x0000_0060 |
| 16 | Settable | IRQ[9] | GPIO2 | GPIOC global interrupt | 0x0000_0064 |
| 17 | Settable | IRQ[10] | GPIO3 | GPIOD global interrupt | 0x0000_0068 |
| 18 | Settable | IRQ[11] | DMA | DMA interrupt | 0x0000_006C |
| 19 | Settable | IRQ[12] | Dualtimer0 | Dualtimer0 global interrupt | 0x0000_0070 |
| 20 | Settable | IRQ[13] | Dualtimer1 | Dualtimer1 global interrupt | 0x0000_0074 |
| 21 | Settable | IRQ[14] | PWM0 | PWM0 global interrupt | 0x0000_0078 |
| 22 | Settable | IRQ[15] | PWM1 | PWM1 global interrupt | 0x0000_007C |
| 23 | Settable | IRQ[16] | PWM2 | PWM2 global interrupt | 0x0000_0080 |
| 24 | Settable | IRQ[17] | PWM3 | PWM3 global interrupt | 0x0000_0084 |
| 25 | Settable | IRQ[18] | PWM4 | PWM4 global interrupt | 0x0000_0088 |
| 26 | Settable | IRQ[19] | PWM5 | PWM5 global interrupt | 0x0000_008C |
| 27 | Settable | IRQ[20] | PWM6 | PWM6 global interrupt | 0x0000_0090 |
| 28 | Settable | IRQ[21] | PWM7 | PWM7 global interrupt | 0x0000_0094 |
| 29 | Settable | IRQ[22] | RTC | RTC interrupt | 0x0000_0098 |
| 30 | Settable | IRQ[23] | ADC | ADC acquisition end interrupt | 0x0000_009C |
| 31 | Settable | IRQ[24] | TCPIP | TCPIP global interrupt | 0x0000_00A0 |
| 32 | Settable | IRQ[25] | EXT_INT | External pin interrupt | 0x0000_00A4 |
| 33 | Settable | IRQ[26] | reserved |  | 0x0000_00A8 |
| 34 | Settable | IRQ[27] | reserved |  | 0x0000_00AC |
| 35 | Settable | IRQ[28] | reserved |  | 0x0000_00B0 |
| 36 | Settable | IRQ[29] | reserved |  | 0x0000_00B4 |
| 37 | Settable | IRQ[30] | reserved |  | 0x0000_00B8 |
| 38 | Settable | IRQ[31] | reserved |  | 0x0000_00BC |

Event

W7500x is able to handle internal events in order to wake up the core(WFE). The wakeup event can be generated by

- When after DMA process finished (DMA_DONE)

Power supply

Introduction

W7500x embeds a voltage regulator in order to supply the internal 1.5V digital power domain.

- Require a 2.7V ~ 5.5V operating supply voltage (VDD)

- ADC ref voltage is same as VDD

Voltage regulator

The voltage regulator is always enabled after Reset and works in only one mode.

- In Run mode, the regulator supplies full power to the 1.5V domain.

- There is no power down or sleep mode

Low-power modes

W7500x is in RUN mode after a system or power reset. There are two low power modes to save power when the CPU does not need to be kept running. These modes are useful for instances like when the CPU is waiting for an external interrupt. Please note that there is no power-off mode for W7500x.

The device features two low-power modes:

- Sleep mode

- Deep Sleep mode

Additionally, the power consumption can be reducing by following method:

- User can slow down the system clocks

- User can gate the clocks to the peripherals when they are unused.

Sleep mode

W7500x has two kinds of sleep modes. One is Sleep mode and the other is Deep sleep mode.

Two of them are almost the same except the clock gated peripherals kinds. Table 2 shows the Sleep mode summary.

Table 2 W7500x sleep mode summary

| Mode | Entry | Wakeup | Effect on clocks |
| --- | --- | --- | --- |
| Sleep mode | DEEPSLEEP = 0 Enable WFI | Any interrupt | CPU clock OFF APB Bus Clock ON AHB Bus clock ON Memory clocks ON |
|  | DEEPSLEEP = 0 Enable WFE | Wakeup event |  |
| Deep Sleep mode | DEEPSLEEP = 1 Enable WFI | Any interrupt | CPU clock OFF APB Bus Clock OFF AHB Bus clock OFF Memory clocks OFF |
|  | DEEPSLEEP = 1 Enable WFE | Wakeup event |  |

Peripheral clock gating

In Run mode, individual clocks can be stopped at any time to reduce power.

Peripheral clock gating is controlled by the CRG block.

Below is the list of clocks which can be gating in CRG block.

- ADC clock (ADCCLK)

- SSP0, SSP1 clock (SSPCLK)

- UART0, UART1 clock (UARTCLK)

- Two Timer clocks (TIMCLK0, TIMCLK1)

- 8ea PWM clocks (PWMCLK0 ~ PWMCLK7)

- WDOG clock (WDOGCLK)

- Random number generator clock (RNGCLK)

System tick timer

Introduction

System tick timer(SysTick) is part of the ARM Cortex-M0 core

Features

Simple 24bit timer. Clocked internally by the system clock.

Functional description

The SysTick timer is an integral part of Cortex-M0. The SysTick timer is intended to generated a fixed 10 millisecond interrupt for use by an operating system or other system management software.

Since the SysTick timer is a part of the Cortex-M0, it facilitates porting of software by

providing a standard timer that is available on Cortex-M0 based devices. The SysTick

timer can be used for :

- An RTOS tick timer which fires at a programmable rate (for example 100 Hz) and invokes a SysTick routine.

- A high-speed alarm timer using the core clock.

- A simple counter. Software can use this to measure time to completion and time used.

- An internal clock source control based on missing/meeting durations. The COUNTFLAG bit-field in the control and status register can be used to determine if an action completed within a set duration, as part of a dynamic clock management control loop.

Registers (Base : 0xE000_E000)

System Timer control and status register (SYST_CSR)

Address Offset : 0x010

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  |  |  | 19 |  | 18 |  | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  |  |  | res |  | res |  | res | CNTFLAG |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  |  | 3 |  |  | 2 |  | 1 |  | 0 |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  |  | res |  |  | res |  | TICKINT |  | ENABLE |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  | R/W |

[0] ENABLE – Enables the counter

0 : Counter disabled

1 : Counter enabled

[1] TICKINT – Enables SysTick exception request

O : Counting down to zero does not assert the SysTick exception request

1 : Counting down to zero to asserts the SysTick exception request

[16] COUNTFLAG – Returns 1 if timer counted to 0 since the last read of this register.

SysTick Reload Value Register (SYST_RVR)

Address Offset : 0x014

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  |  | 20 |  | 19 |  | 18 | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | RELOAD[23:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  |  | 4 |  | 3 |  | 2 | 1 |  | 0 |
| RELOAD[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[23:0] RELOAD – Value to load into the SYST_CVR when the counter is enabled and when it reaches 0

- The RELOAD value can be any value in the range 0x0000_0001 – 0x00FFFFFF. You can program a value of 0, but this has no effect because the SysTick exception request and COUNTFLAG are activated when count from 1 to 0.

- To generate a multi-short timer with a period of N processor clock cycles, use a RELOAD value of N-1. For example, if the SysTick interrupt is required every 100 clock pulses, set RELOAD to 99.

SysTick Current Value Register (SYST_CVR)

Address Offset : 0x018

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  |  | 20 |  | 19 |  | 18 | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | CURRENT[23:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  |  | 4 |  | 3 |  | 2 | 1 |  | 0 |
| CURRENT [15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[23:0] CURRENT – Reads return the current value of the SysTick counter.

A write of any value clears the field to 0, and also clears the SYST_CSR.COUNTFLAG bit to 0.

SysTick Calibration Value Register (SYST_CALIB)

Address Offset : 0x01C

Reset value : 0x0000_0000

| 31 |  |  | 30 |  |  | 29 |  |  | 28 |  |  | 27 |  |  | 26 |  |  | 25 |  |  | 24 |  | 23 |  | 22 |  | 21 |  |  | 20 |  | 19 |  | 18 | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NORF |  |  | SKEW |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  | TENMS[23:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  | R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  | 14 |  |  | 13 |  |  | 12 |  |  | 11 |  |  | 10 |  |  | 9 |  |  | 8 |  |  | 7 |  | 6 |  | 5 |  |  | 4 |  | 3 |  | 2 | 1 |  | 0 |
| TENMS [15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[23:0] TENMS - Reads as zero. Indicates calibration value is not known.

[30] SKEW - Reads as one. Calibration value for the 10ms inexact timing is not known because TENMS is not known. This can affect the suitability of SysTick as a software real time clock.

[31] NOREF - Reads as one. Indicates that no separate reference clock is provided.

- Booting Sequence

W7500x has three different boot modes that can be selected through the BOOT pin and TEST pin as shown in Table 3.

Table 3 operation of mode selection

| Mode selection |  | Mode | Aliasing |
| --- | --- | --- | --- |
| TEST | BOOT |  |  |
| 0 | 0 | APP | User code execute in Main Flash memory. |
| 0 | 1 | ISP | In this mode,W7500x can support ISP function in order to control flash using serial interface. |

When W7500x is reset by hardware, it will be operated as below in embedded boot code.

Figure 3. operation of boot code

- Embedded Flash memory

Flash main features

- Up to 128Kbytes of Flash memory

- Memory organization:

  - Main Flash memory block:

Up to 128Kbytes

  - Information block:

Up to 512bytes

Information block is read only

  - Data block:

Up to 512bytes

- Flash memory interface features:

  - Read interface with prefetch buffer( 1 x 32-bit words )

  - Flash Program / Erase operation

- Flash memory lock function:

  - It can be controlled only by ISP tool provided by WIZnet

Flash memory organization

The Flash memory is organized of 32-bit wide memory cells that can be used for storing both code and data constants.

The memory organization is based on a main Flash memory block containing 512 sectors of 256byte or 32 blocks of 4Kbyte.

Table 4 description of Flash memory

| Flash area | Flash memory address | Size (bytes) | Name | Description |
| --- | --- | --- | --- | --- |
| Main Flash memory | 0x0000 0000 ~ 0x0000 00FF | 256 | Sector 0 | Block 0 |
|  | 0x0000 0100 ~ 0x0000 01FF | 256 | Sector 1 |  |
|  | 0x0000 0200 ~ 0x0000 02FF | 256 | Sector 2 |  |
|  | 0x0000 0300 ~ 0x0000 03FF | 256 | Sector 3 |  |
|  | . . . | . . . | . . . | . . . |
|  | 0x0000 7000 ~ 0x0000 70FF | 256 | Sector112 | Block 7 |
|  | 0x0000 7100 ~ 0x0000 71FF | 256 | Sector113 |  |
|  | 0x0000 7200 ~ 0x0000 72FF | 256 | Sector114 |  |
|  | 0x0000 7300 ~ 0x0000 73FF | 256 | Sector115 |  |
|  | . . . | . . . | . . . | . . . |
|  | 0x0001 FC00 ~ 0x0001 FCFF | 256 | Sector509 | Block 32 |
|  | 0x0001 FD00 ~ 0x0001 FDFF | 256 | Sector510 |  |
|  | 0x0001 FE00 ~ 0x0001 FEFF | 256 | Sector511 |  |
|  | 0x0001 FF00 ~ 0x0001 FFFF | 256 | Sector512 |  |
| Information block | 0x0003 FC00 ~ 0x0003 FCFF | 256 |  | Lock info |
|  | 0x0003 FD00 ~ 0x0003 FDFF | Reserved |  |  |
| Data block | 0x0003 FE00 ~ 0x0003 FEFF | 256 |  | Data0 |
|  | 0x0003 FF00 ~ 0x0003 FFFF | 256 |  | Data1 |

The W7500 embedded Flash memory can be programmed using in-application programming. IAP allows the user to re-program the Flash memory while the application is running. The program and erase operations can be performed over the whole product voltage range.

In W7500x_Library_Examples(https://github.com/Wiznet/W7500/tree/master/W7500x_Library_Examples/Projects/Peripheral_Examples/Flash/IAP_Example), there is the IAP Example Project and the below function is supported to use IAP.

void DO_IAP( uint32_t id, uint32_t dst_addr, uint8_t* src_addr, uint32_t size);

This function requests those parameters, id, dst_addr, src_addr and size. 'id' is already defined in 'main.c'. 'dst_addr' is the flash memory address in the upper table. 'src_addr' is the buffer pointer user want to program. 'size' is the flash size user chooses. Please refer flash address and size mentioned in the upper table.

// IAP 'id' paremeter define

#define IAP_ENTRY           0x1FFF1001     // Because Thum code

#define IAP_ERAS            0x010

#define IAP_ERAS_DAT0       (IAP_ERAS + 0) // Erase Data 0 block

#define IAP_ERAS_DAT1       (IAP_ERAS + 1) // Erase Data 1 block

#define IAP_ERAS_SECT       (IAP_ERAS + 2) // Erase a Sector in Main Flash Memory

#define IAP_ERAS_BLCK       (IAP_ERAS + 3) // Erase a Block in Main Flash Memory

#define IAP_ERAS_CHIP       (IAP_ERAS + 4) // Erase all code

#define IAP_ERAS_MASS       (IAP_ERAS + 5) // Erase all code & data

#define IAP_PROG            0x022

This is how to Erase and Program flash memory. Especially, with IAP_ERAS_DAT0 and IAP_ERAS_DAT1, there is no need to put other parameters (there are default values).

// Step 1 DATA0 Erase, Read, Write Test

DO_IAP(IAP_ERAS_DAT0,0,0,0);

DO_IAP(IAP_PROG,DAT0_START_ADDR,buffer,SECT_SIZE);

Operating program can be deleted when user use 'IAP_ERAS_CHIP' or 'IAP_ERAS_MASS'.

// Using IAP_ERAS_CHIP or IAP_ERAS_MASS

DO_IAP(IAP_ERAS_CHIP,0,0,0);

DO_IAP(IAP_ERAS_MASS,0,0,0);

Clock Reset generator (CRG)

Introduction

CRG is clock reset generator block for W7500x System. It provides every clock/reset for all other block include CPU and peripherals. CRG includes PLL and POR.

Features

Reset

- Three types of reset – external reset, Power reset, system reset

- External reset is generated by low level on the RSTn pin (external reset)

- Power reset is generated by Power-on reset (POR)

- Power on reset is generated by POR

- System reset is generated when one of the following events occurs

- Watchdog event

- After remapping

- Software reset (SYSRESETREQ bit in Cortex-M0. Refer to the Cortex-M0 technical reference manual for more detail)

- Power reset sets all registers to their reset values.

- System reset sets all registers to their reset values except the CRG block registers and remap register to protect remap value

Clock

Two clock sources can be used to drive the system clock.

- External oscillator clock (8MHz ~ 24MHz) (OCLK)

- Internal 8MHz RC oscillator clock (RCLK)

One additional clock source

- 32.768KHz low speed external crystal which derives the real time clock.

There is a PLL

One PLL is integrated

- Input clock range is from 8MHz to 24MHz

- Frequency can be generated by M/N/OD registers. (refer register description)

- Bypass option enabled

There are many generated clocks for independent operating with system clock

- System clock (FCLK)

- ADC clock (ADCCLK)

- SSP0, SSP1 clock (SSPCLK)

- UART0, UART1 clock (UARTCLK)

- Two Timer clocks (TIMCLK0, TIMCLK1)

- 8ea PWM clocks (PWMCLK0 - PWMCLK7)

- Real time clock (RTCCLK)

- WDOG clock (WDOGCLK)

- Random number generator clock (RNGCLK)

RNGCLK have only one source (pll output) and no prescaler

Some of the generated clocks turn off automatically when CPU enters sleep mode.

- ADCCLK, RNGCLK

Generate two Hardware TCPIP Clocks (MII_RXC, MII_TXC) are from external PADs.

Hardware TCPIP Clocks can be gated by register control.

All clocks generated from CRG can be monitored.

Functional description

Figure 4 CRG block diagram

External Oscillator Clock

The External oscillator clock (OCLK) can be supplied with a 8 to 24 MHz crystal/ceramic resonator oscillator. In the Typical application, Figure 5, RF must be inserted in External oscillator clock circuit. In W7500x, there is no supported RF for External oscillator clock (see Figure 5).

For CLoad1 and CLoad2, it is recommended to use external ceramic capacitors in the 5 pF to 25 pF range(typ.) and are usually the same size, designed for application, and selected to match the requirements of the crystal or resonator (see Figure 5).

Refer to the crystal resonator manufacturer for more details on the resonator characteristics (frequency, package, accuracy).

REXT value depends on the crystal characteristics

Figure 5 Typical application with an 8 MHz crystal

RC oscillator clock

RC oscillator clock (RCLK) signal is generated from an internal 8MHz RC oscillator.

RC oscillator has the advantage of providing a clock source at low cost (no external components). However the RC oscillator is less accurate than the external crystal or ceramic resonator.

- Accuracy : 1% at TA= 25oC (User don’t need to calibration)

PLL

The internal PLL can be used to multiply the External Oscillator Clock (OCLK) or RC Oscillator Clock (RCLK). PLL input can be selected by register.

PLL output clock can be generated by following the equations below.

- FOUT = FIN x M / N x 1 / OD

- Where:

- M = M[5] x 25 + M[4] x 24 + M[3] x 23 + M[2] x 22 + M[1] x 2 + M[0] x 1

- N = N[5] x 25 + N[4] x 24 + N[3] x 23 + N[2] x 22 + N[1] x 2 + N[0] x 1

- OD = 2 (2 x OD[1]) x 2 (1 x OD[0])

Generated clock

Each generated clock source can be selected among 3 clock source as independent by each clock source select register.

- PLL output clock (MCLK)

- Internal 8MHz RC oscillator clock (RCLK)

- External oscillator clock (8MHz ~ 24MHz) (OCLK)

Each generated clock has own prescaler which can be selected individually by each prescale value register.

- FCLK, ADCCLK, SSPCLK, UARTCLK : 1/1, 1/2, 1/4, 1/8

- TIMCLK0, TIMCLK1, PWMCLK0 – PWMCLK7, RTCCLK, WDOGCLK : 1/1, 1/2, 1/4, 1/8, 1/16, 1/32, 1/64, 1/128

Registers (Base address : 0x4100_1000)

OSC power down register (OSC_PDR)

Address offset : 0x000

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | OSCPD |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[0] OSCPD – Internal 8MHz RC oscillator power down register

This bit written by S/W to RCOSC enter sleep mode or not

0 : normal operation

1 : power down (enter sleep mode)

PLL power down register (PLL_PDR)

Address offset : 0x010

Reset value : 0x0000_0001

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | PLLPD |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[0] PLLPD – PLL power down register

This bit written by S/W to PLL power down or not

0 : power down

1 : normal operation

PLL frequency calculating register (PLL_FCR)

Address offset : 0x014

Reset value : 0x0005_0200

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | M |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | N |  |  |  |  |  | res | res | res | res | res | res | OD |  |
|  |  | R/W |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] OD

[13:8] N

[21:16] M

These bits are written by S/W to set frequency of PLL output.

PLL output frequency FOUT is calculated by the following equations:

FOUT = FIN x M / N x 1 / OD

Where:

M = M[5] x 32 + M[4] x 16 + M[3] x 8 + M[2] x 4 + M[1] x 2 + M[0] x 1 (2 ~ 63)

N = N[5] x 32 + N[4] x 16 + N[3] x 8 + N[2] x 4 + N[1] x 2 + N[0] x 1 (1 ~ 63)

OD = 2 ^ (2 x OD[1]) x 2 ^ (1 x OD[0])

PLL output enable register (PLL_OER)

Address offset : 0x018

Reset value : 0x0000_0001

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | PLLOEN |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[0] PLLOEN – output enable register of PLL

This bit written by S/W to control output enable of PLL

0 : Clock out is disable. VCO is working but FOUT is low only.

1 : Clock out is enable.

PLL bypass register (PLL_BPR)

Address offset : 0x01c

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | PLLBP |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[0] PLLBP – bypass register of PLL

This bit written by S/W to control bypass or not of PLL

0 : bypass disable. Normal operation

1 : bypass enable. Clock out is clock input

PLL input clock source select register (PLL_IFSR)

Address offset : 0x020

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  |  | 19 |  | 18 |  | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  |  | res |  | res |  | res | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 | 2 |  | 1 |  | 0 |  |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res | res |  | res |  | PLLIS |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |

[0] PLLIS – select register of PLL input clock source

This bit written by S/W to select

0 : Internal 8MHz RC oscillator clock (RCLK)

1 : External oscillator clock (OCLK, 8MHz ~ 24MHz)

FCLK source select register (FCLK_SSR)

Address offset : 0x030

Reset value : 0x0000_0001

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | FCKSRC |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] FCKSRC – select register of FCLK clock source

These bits are written by S/W to select

00, 01 : output clock of PLL (MCLK)

10 : Internal 8MHz RC oscillator clock (RCLK)

11 : External oscillator clock (OCLK, 8MHz ~ 24MHz)

FCLK prescale value select register (FCLK_PVSR)

Address offset : 0x034

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | FCKPRE |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] FCKPRE – select prescale value of FCLK clock

These bits are written by S/W to select

00 : 1/1 (bypass)

01 : 1/2

10 : 1/4

11 : 1/8

SSPCLK source select register (SSPCLK_SSR)

Address offset : 0x040

Reset value : 0x0000_0001

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | SSPCSS |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] SSPCSS – SSPCLK clock source select register.

These bits are written by S/W to select clock source

00 : disable clock

01 : PLL output clock (MCLK)

10 : Internal 8MHz RC oscillator clock (RCLK)

11 : External oscillator clock (OCLK, 8MHz ~ 24MHz)

SSPCLK prescale value select register (SSPCLK_PVSR)

Address offset : 0x044

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | SSPCP |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] SSPCP – select prescale value of SSPCLK clock

These bits are written by S/W to select

00 : 1/1 (bypass)

01 : 1/2

10 : 1/4

11 : 1/8

ADCCLK source select register (ADCCLK_SSR)

Address offset : 0x060

Reset value : 0x0000_0001

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | ADCSS |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] ADCSS – ADCCLK clock source select register.

These bits are written by S/W to select clock source

00 : disable clock

01 : PLL output clock (MCLK)

10 : Internal 8MHz RC oscillator clock (RCLK)

11 : External oscillator clock (OCLK, 8MHz ~ 24MHz)

ADCCLK prescale value select register (ADCCLK_PVSR)

Address offset : 0x064

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | ADCCP |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] ADCCP – select prescale value of ADCCLK clock

These bits are written by S/W to select

00 : 1/1 (bypass)

01 : 1/2

10 : 1/4

11 : 1/8

TIMER0CLK source select register (TIMER0CLK_SSR)

Address offset : 0x070

Reset value : 0x0000_0001

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | T0CSS |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] T0CSS – TIMCLK0 clock source select register.

These bits are written by S/W to select clock source

00 : disable clock

01 : PLL output clock (MCLK)

10 : Internal 8MHz RC oscillator clock (RCLK)

11 : External oscillator clock (OCLK, 8MHz ~ 24MHz)

TIMER0CLK prescale value select register (TIMER0CLK_PVSR)

Address offset : 0x074

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | T0CPS |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |

[2:0] T0CPS – select prescale value of TIM0CLK clock

These bits are written by S/W to select

000 : 1/1 (bypass)

001 : 1/2

010 : 1/4

011 : 1/8

100 : 1/16

101 : 1/32

110 : 1/64

111 : 1/128

TIMER1CLK source select register (TIMER1CLK_SSR)

Address offset : 0x080

Reset value : 0x0000_0001

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | T1CSS |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] T1CSS – TIMCLK1 clock source select register.

These bits are written by S/W to select clock source

00 : disable clock

01 : PLL output clock (MCLK)

10 : Internal 8MHz RC oscillator clock (RCLK)

11 : External oscillator clock (OCLK, 8MHz ~ 24MHz)

TIMER1CLK prescale value select register (TIMER1CLK_PVSR)

Address offset : 0x084

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | T1CPS |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |

[2:0] T1CPS – select prescale value of TIM1CLK clock

These bits are written by S/W to select

000 : 1/1 (bypass)

001 : 1/2

010 : 1/4

011 : 1/8

100 : 1/16

101 : 1/32

110 : 1/64

111 : 1/128

PWM0CLK source select register (PWM0CLK_SSR)

Address offset : 0x0b0

Reset value : 0x0000_0001

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | P0CSS |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |

[1:0] P0CPS – PWMCLK0 clock source select register.

These bits are written by S/W to select clock source

00 : disable clock

01 : PLL output clock (MCLK)

10 : Internal 8MHz RC oscillator clock (RCLK)

11 : External oscillator clock (OCLK, 8MHz ~ 24MHz)

PWM0CLK prescale value select register (PWM0CLK_PVSR)

Address offset : 0x0b4

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | P0CPS |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |

[2:0] P0CPS – select prescale value of PWM0CLK clock

These bits are written by S/W to select

000 : 1/1 (bypass)

001 : 1/2

010 : 1/4

011 : 1/8

100 : 1/16

101 : 1/32

110 : 1/64

111 : 1/128

PWM1CLK source select register (PWM1CLK_SSR)

Address offset : 0x0c0

Reset value : 0x0000_0001

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | P1CSS |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |

[1:0] P1CSS – PWMCLK1 clock source select register.

These bits are written by S/W to select clock source

00 : disable clock

01 : PLL output clock (MCLK)

10 : Internal 8MHz RC oscillator clock (RCLK)

11 : External oscillator clock (OCLK, 8MHz ~ 24MHz)

PWM1CLK prescale value select register (PWM1CLK_PVSR)

Address offset : 0x0c4

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | P1CPS |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |

[2:0] P1CPS – select prescale value of PWM1CLK clock

These bits are written by S/W to select

000 : 1/1 (bypass)

001 : 1/2

010 : 1/4

011 : 1/8

100 : 1/16

101 : 1/32

110 : 1/64

111 : 1/128

PWM2CLK source select register (PWM2CLK_SSR)

Address offset : 0x0d0

Reset value : 0x0000_0001

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | P2CSS |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |

[1:0] P2CSS – PWMCLK2 clock source select register.

These bits are written by S/W to select clock source

00 : disable clock

01 : PLL output clock (MCLK)

10 : Internal 8MHz RC oscillator clock (RCLK)

11 : External oscillator clock (OCLK, 8MHz ~ 24MHz)

PWM2CLK prescale value select register (PWM2CLK_PVSR)

Address offset : 0x0d4

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | P2CPS |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |

[2:0] PWM2CLK_PRE – select prescale value of PWM2CLK clock

These bits are written by S/W to select

000 : 1/1 (bypass)

001 : 1/2

010 : 1/4

011 : 1/8

100 : 1/16

101 : 1/32

110 : 1/64

111 : 1/128

PWM3CLK source select register (PWM3CLK_SSR)

Address offset : 0x0e0

Reset value : 0x0000_0001

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | P3CSS |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |

[1:0] P3CSS – PWMCLK3 clock source select register.

These bits are written by S/W to select clock source

00 : disable clock

01 : PLL output clock (MCLK)

10 : Internal 8MHz RC oscillator clock (RCLK)

11 : External oscillator clock (OCLK, 8MHz ~ 24MHz)

PWM3CLK prescale value select register (PWM3CLK_PVSR)

Address offset : 0x0e4

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | P3CPS |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |

[2:0] P3CPS – select prescale value of PWM3CLK clock

These bits are written by S/W to select

000 : 1/1 (bypass)

001 : 1/2

010 : 1/4

011 : 1/8

100 : 1/16

101 : 1/32

110 : 1/64

111 : 1/128

PWM4CLK source select register (PWM4CLK_SSR)

Address offset : 0x0f0

Reset value : 0x0000_0001

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | P4CSS |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |

[1:0] P4CSS – PWMCLK4 clock source select register.

These bits are written by S/W to select clock source

00 : disable clock

01 : PLL output clock (MCLK)

10 : Internal 8MHz RC oscillator clock (RCLK)

11 : External oscillator clock (OCLK, 8MHz ~ 24MHz)

PWM4CLK prescale value select register (PWM4CLK_PVSR)

Address offset : 0x0f4

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | P4CPS |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |

[2:0] P4CPS – select prescale value of PWM4CLK clock

These bits are written by S/W to select

000 : 1/1 (bypass)

001 : 1/2

010 : 1/4

011 : 1/8

100 : 1/16

101 : 1/32

110 : 1/64

111 : 1/128

PWM5CLK source select register (PWM5CLK_SSR)

Address offset : 0x100

Reset value : 0x0000_0001

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | P5CSS |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |

[1:0] P5CSS – PWMCLK5 clock source select register.

These bits are written by S/W to select clock source

00 : disable clock

01 : PLL output clock (MCLK)

10 : Internal 8MHz RC oscillator clock (RCLK)

11 : External oscillator clock (OCLK, 8MHz ~ 24MHz)

PWM5CLK prescale value select register (PWM5CLK_PVSR)

Address offset : 0x104

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | P5CPS |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |

[2:0] P5CPS – select prescale value of PWM5CLK clock

These bits are written by S/W to select

000 : 1/1 (bypass)

001 : 1/2

010 : 1/4

011 : 1/8

100 : 1/16

101 : 1/32

110 : 1/64

111 : 1/128

PWM6CLK source select register (PWM6CLK_SSR)

Address offset : 0x110

Reset value : 0x0000_0001

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | P6CSS |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |

[1:0] P6CSS – PWMCLK6 clock source select register.

These bits are written by S/W to select clock source

00 : disable clock

01 : PLL output clock (MCLK)

10 : Internal 8MHz RC oscillator clock (RCLK)

11 : External oscillator clock (OCLK, 8MHz ~ 24MHz)

PWM6CLK prescale value select register (PWM6CLK_PVSR)

Address offset : 0x114

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | P6CPS |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |

[2:0] P6CPS – select prescale value of PWM6CLK clock

These bits are written by S/W to select

000 : 1/1 (bypass)

001 : 1/2

010 : 1/4

011 : 1/8

100 : 1/16

101 : 1/32

110 : 1/64

111 : 1/128

PWM7CLK source select register (PWM7CLK_SSR)

Address offset : 0x120

Reset value : 0x0000_0001

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | P7CSS |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |

[1:0] P7CSS – PWMCLK7 clock source select register.

These bits are written by S/W to select clock source

00 : disable clock

01 : PLL output clock (MCLK)

10 : Internal 8MHz RC oscillator clock (RCLK)

11 : External oscillator clock (OCLK, 8MHz ~ 24MHz)

PWM7CLK prescale value select register (PWM7CLK_PVSR)

Address offset : 0x124

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | P7CPS |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |

[2:0] P7CPS – select prescale value of PWM7CLK clock

These bits are written by S/W to select

000 : 1/1 (bypass)

001 : 1/2

010 : 1/4

011 : 1/8

100 : 1/16

101 : 1/32

110 : 1/64

111 : 1/128

RTC High Speed source select register (RTC_HS_SSR)

Address offset : 0x130

Reset value : 0x0000_0001

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | RTCHS |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |

[1:0] RTCHS – RTCCLK_hs clock source select register.

These bits are written by S/W to select clock source

00 : disable clock

01 : PLL output clock (MCLK)

10 : Internal 8MHz RC oscillator clock (RCLK)

11 : External oscillator clock (OCLK, 8MHz ~ 24MHz)

RTC High Speed prescale value select register (RTC_HS_PVSR)

Address offset : 0x134

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | RTCPRE |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |

[2:0] RTCPRE – select prescale value of RTCCLK_hs clock

These bits are written by S/W to select

000 : 1/1 (bypass)

001 : 1/2

010 : 1/4

011 : 1/8

100 : 1/16

101 : 1/32

110 : 1/64

111 : 1/128

RTC source select register (RTC_SSR)

Address offset : 0x13c

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  |  | 19 |  | 18 |  | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  |  | res |  | res |  | res | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 | 2 |  | 1 |  | 0 |  |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res | res |  | res |  | RTCSEL |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |

[0] RTCSEL – RTCCLK clock source select register.

These bits are written by S/W to select clock source

0 : RTCCLK_hs

1 : 32K_OSC_CLK (Low speed external oscillator clock)

WDOGCLK High Speed source select register (WDOGCLK_HS_SSR)

Address offset : 0x140

Reset value : 0x0000_0001

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | WDHS |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |

[1:0] WDHS – WDOGCLK_hs clock source select register.

These bits are written by S/W to select clock source

00 : disable clock

01 : PLL output clock (MCLK)

10 : Internal 8MHz RC oscillator clock (RCLK)

11 : External oscillator clock (OCLK, 8MHz ~ 24MHz)

WDOGCLK High Speed prescale value select register (WDOGCLK_HS_PVSR)

Address offset : 0x144

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | WDPRE |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |

[2:0] WDPRE – select prescale value of WDOGCLK_hs clock

These bits are written by S/W to select

000 : 1/1 (bypass)

001 : 1/2

010 : 1/4

011 : 1/8

100 : 1/16

101 : 1/32

110 : 1/64

111 : 1/128

WDOGCLK clock source select register (WDOGCLK_SSR)

Address offset : 0x14c

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  |  | res |  | res |  | res |  | res |  | res | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res | res |  | res |  | res |  | res |  | WDSEL |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |

[0] WDSEL – WDOGCLK clock source select register.

These bits are written by S/W to select clock source

0 : WDOGCLK_hs

1 : 32K_OSC_CLK (Low speed external oscillator clock)

UARTCLK source select register (UARTCLK_SSR)

Address offset : 0x150

Reset value : 0x0000_0001

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | UCSS |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |

[1:0] UCSS – UARTCLK clock source select register.

These bits are written by S/W to select clock source

00 : disable clock

01 : PLL output clock (MCLK)

10 : Internal 8MHz RC oscillator clock (RCLK)

11 : External oscillator clock (OCLK, 8MHz ~ 24MHz)

UARTCLK prescale value select register (UARTCLK_PVSR)

Address offset : 0x154

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | UCP |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |

[1:0] UCP – select prescale value of UARTCLK clock

These bits are written by S/W to select

00 : 1/1 (bypass)

01 : 1/2

10 : 1/4

11 : 1/8

MIICLK enable control register (MIICLK_ECR)

Address offset : 0x160

Reset value : 0x0000_0003

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  |  | res |  | res |  | res |  | res |  | res |  | res |  | res | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res | res |  | res |  | res |  | res |  | res |  | res |  | MIITEN |  |  | MIIREN |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  | R/W |  |

[0] MIIREN – MII RX Clock source enable register

This bit is written by S/W to set enable or disable

0 : Disable MII_RCK and MII_RCK_N

1 : Enable MII_RCK and MII_RCK_N

[1] MIITEN – MII TX Clock source enable register

This bit is written by S/W to set enable or disable

0 : Disable MII_TCK and MII_TCK_N

1 : Enable MII_TCK and MII_TCK_N

Monitoring Clock source select register (MONCLK_SSR)

Address offset : 0x170

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | CLKMON_SEL |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |  |

[4:0] CLKMON_SEL – Select clock source for monitoring (monitoring pin : PA_02)

This bit is written by S/W to set enable or disable

00000 : PLL output clock (MCLK)

00001 : FCLK

00010 : Internal 8MHz RC oscillator clock (RCLK)

00011 : External oscillator clock (OCLK, 8MHz ~ 24MHz)

00100 : ADCCLK

00101 : SSPCLK

00110 : TIMCLK0

00111 : TIMCLK1

01000 : PWMCLK0

01001 : PWMCLK1

01010 : PWMCLK2

01011 : PWMCLK3

01100 : PWMCLK4

01101 : PWMCLK5

01110 : PWMCLK6

01111 : PWMCLK7

10000 : UARTCLK

10001 : MII_RCK

10010 : MII_TCK

10011 : RTCCLK

Register map

The following table summarizes the CRG registers.

Table 5 CRG register map and reset values

Tcp/ip core Offload Engine (TOE)

Introduction

The TCP/IP Core Offload Engine (TOE) is a Hardwired TCP/IP embedded Ethernet controller that provides easier Internet connection to embedded systems. TOE enables users to have Internet connectivity in their applications by using the TCP/IP stack.

WIZnet’s Hardwired TCP/IP is the market-proven technology that supports TCP, UDP, IPv4, ICMP, ARP, IGMP, and PPPoE protocols. TOE embeds the 32Kbyte internal memory buffer for the Ethernet packet processing. Using TOE allows users to implement the Ethernet application by adding the simple socket program. It’s faster and easier than using any other Embedded Ethernet solutions. 8 independent hardware sockets can be used simultaneously.

TOE also provides WOL (Wake on LAN) to reduce power consumption of the system.

Features

- Supports Hardwired TCP/IP Protocols : TCP, UDP, ICMP, IPv4, ARP, IGMP, PPPoE

- Supports 8 independent sockets simultaneously

- Supports Power down mode

- Supports Wake on LAN over UDP

- Internal 32Kbytes Memory for TX/RX Buffers

- Not supports IP Fragmentation

Functional description

Figure 6 shows the TOE block diagram.

Figure 6 TOE block diagram

TOE Memory map

TOE has one Common Register Block, eight Socket Register Blocks, and TX/RX Buffer Blocks allocated to each Socket. Figure 7 shows the selected block by the base address and the available offset address range of Socket TX/RX Buffer Blocks. Each Socket’s TX Buffer Block physically exists in one 16KB TX memory and is initially allocated with 2KB. Also, Each Socket’s RX Buffer Block physically exists in one 16KB RX Memory and is initially allocated with 2KB.

Regardless of the allocated size of each Socket TX/RX Buffer, it can be accessible within the 16 bits offset address range (From 0x0000 to 0xFFFF).

Figure 7. Register & Memory Organization

Common register map

Common Register Block configures the general information of TOE such as IP and MAC address. <Table 6> defines the offset address of registers in this block.

Table 6. Offset Address for Common Register

| Address | Register |
| --- | --- |
| 0x0000 | TOE Version (VERSIONR) |
| 0x2000 | TICKCLOK (TCLKR) |
| 0x2100 | Interrupt (IR) |
| 0x2104 | Interrupt Mask (IMR) |
| 0x2108 | Interrupt Clear (IRCR) |
| 0x2110 | Socket Interrupt (SIR) |
| 0x2114 | Socket Mask (SIMR) |
| 0x2300 | Mode (MR) |
| 0x2400 | PPP Timer (PTIMER) |
| 0x2404 | PPP Magic (PMAGIC) |
| 0x2408 | PPP Destination MAC Address (PHAR1) |
| 0x240C | PPP Destination MAC address (PHAR0) |
| 0x2410 | PPP Session Identification (PSIDR) |
| 0x2414 | PPP Maximum Segment Size (PMSS) |
| 0x6000 | Source Hardware Address (SHAR1) |
| 0x6004 | Source Hardware Address (SHAR0) |
| 0x6008 | Gateway Address (GA) |
| 0x600C | Subnet Mask (SUB) |
| 0x6010 | Source IP Address (SIP) |
| 0x6020 | Network Configuration Lock (NCONFL) |
| 0x6040 | Retry Time (RTR) |
| 0x6044 | Retry Counter (RCR) |
| 0x6050 | Unreachable IP Address (UIP) |
| 0x6054 | Unreachable Port Address (UPORT) |

Socket register map

TOE supports 8 Sockets for communication channel. Each Socket is controlled by Socket n Register (n = 0,…,7  ,where n is socket number). <Table 2> defines the 16bits Offset Address of registers in Socket n Register Block.

Table 7. Offset Address in Socket n Register Block (n = 0,…,7, where n is Socket number)

| Offset | Register |
| --- | --- |
| 0x0000 | Socket Mode (Sn_MR) |
| 0x0010 | Socket Command (Sn_CR) |
| 0x0020 | Socket Interrupt (Sn_IR) |
| 0x0024 | Socket Interrupt Mask (Sn_IMR) |
| 0x0028 | Socket Interrupt Clear (Sn_ICR) |
| 0x0030 | Socket Status (Sn_SR) |
| 0x0100 | Socket Protocol Number (Sn_PNR) |
| 0x0104 | Socket IP Type of Service (Sn_TOS) |
| 0x0108 | Socket TTL (Sn_TTLR) |
| 0x010C | Socket Fragment Offset (Sn_FRAG) |
| 0x0110 | Socket Maximum Segment (Sn_MSSR) |
| 0x0114 | Socket Port Number (Sn_PORTR) |
| 0x0118 | Socket Destination Hardware address0 (Sn_DHAR0) |
| 0x011C | Socket Destination Hardware address1 (Sn_DHAR1) |
| 0x0120 | Socket Destination Port Number (Sn_DPORTR) |
| 0x0124 | Socket Destination IP Address (Sn_DIPR) |
| 0x0180 | Socket Keep Alive Timer (Sn_KATMR) |
| 0x0184 | Socket Retry Time (Sn_RTR) |
| 0x0188 | Socket Retry Counter (Sn_RCR) |
| 0x0200 | Socket TX Memory Size (Sn_TXBUF_SIZE) |
| 0x0204 | Socket TX Free Size (Sn_TX_FSR) |
| 0x0208 | Socket TX Read Pointer (Sn_TX_RD) |
| 0x020C | Socket TX Write Pointer (Sn_TX_WR) |
| 0x0220 | Socket RX Memory Size (Sn_RXBUF_SIZE) |
| 0x0224 | Socket RX Received Size (Sn_RX_RSR) |
| 0x0228 | Socket RX Read Pointer (Sn_RX_RD) |
| 0x022C | Socket RX Write Pointer (Sn_RX_WR) |

Memory

TOE has one 16KB TX memory for Socket n TX Buffer Blocks and one 16KB RX memory for Socket n RX buffer Blocks.

16KB TX memory is initially allocated in 2KB size for each Socket TX Buffer Block (2KB X 8 = 16KB). The initial allocated 2KB size of Socket n TX Buffer can be re-allocated by using ‘Socket n TX Buffer Size Register (Sn_TXBUF_SIZE)’.

Once all Sn_TXBUF_SIZE registers have been configured, Socket TX Buffer is allocated with the configured size of 16KB TX Memory and is assigned sequentially from Socket 0 to Socket 7. Its physical memory address is automatically determined in 16KB TX memory. Therefore, the total sum of Sn_TXBUF_SIZE should not exceed 16 in case of error in data transmission.

The 16KB RX memory allocation method is the same as the 16KB TX memory allocation method. 16KB RX memory is initially allocated into 2KB size for each Socket RX Buffer Block (2KB X 8 = 16KB). The initial allocated 2KB size of Socket n RX Buffer can be re-allocated by using ‘Socket n RX Buffer Size Register (Sn_RXBUF_SIZE)’.

When all Sn_RXBUF_SIZE registers have been configured, the Socket RX Buffer is allocated with the configured size in 16KB RX Memory and is assigned sequentially from Socket 0 to Socket 7. The physical memory address of the Socket RX Buffer is automatically determined in 16KB RX memory. Therefore, the total sum of Sn_RXBUF_SIZE should not exceed 16 or data reception error will occur.

For 16KB TX/RX memory allocation, refer to Sn_TXBUF_SIZE & Sn_RXBUF_SIZE in ‘Chapter 0’. The Socket n TX Buffer Block allocated in 16KB TX memory is buffer for saving data to be transmitted by host. The 16bits Offset Address of Socket n TX Buffer Block has 64KB address space ranged from 0x0000 to 0xFFFF, and is configured with reference to ‘Socket n TX Write Pointer Register (Sn_TX_WR)’ & ‘Socket n TX Read Pointer Register(Sn_RX_RD)’. However, the 16bits Offset Address automatically converts into the physical address to be accessible in 16KB TX memory such as Figure 7.

The Socket n RX Buffer Block allocated in 16KB RX memory is buffer for saving the received data through the Ethernet. The 16bits Offset Address of Socket n RX Buffer Block has 64KB address space ranged from 0x0000 to 0xFFFF, and is configured with reference to ‘Socket n RX RD Pointer Register (Sn_RX_RD)’ & ‘Socket n RX Write Pointer Register (Sn_RX_WR)’. However, the 16bits Offset Address automatically converts into the physical address to be accessible in 16KB RX memory such as Figure 7.

Common register (Base : 0x4600_0000)

VERSIONR (TOE Version Register)

Address Offset : 0x0000

Reset value : 0x0000_0005

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  | 0 |
| res | res |  | Res |  | res |  | res |  | res |  | res |  | res |  | VERSION[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  |  | R/W |  |

[7:0] VERSION - indicates the TOE version as 0x05.

TCKCNTR (Ticker Counter Register)

Address Offset : 0x2000

Reset value : 0x0000_07D0

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | Res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| TCKCNT[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[15:0] TCKCNT – Ticker counter register is used Tick counter of 100usec. for internal timer of TOE. The unit of tick is HCLK. RTR, Sn_RTR, Sn_KATMR operates on the value of this register.

1f ×TCKCNT=Reference time of TOE

Ex) When the reference time of TOE is set to 100us, and when f = 20MHz,

1f ×TCKCNT=100us

TCKCNT=100us × f

TCKCNT = 2000 = 0x07D0

IR (Interrupt Register)

Address Offset : 0x2100

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | Res | res | res | res | res | res | res | res | res | res | es | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | IR[7:4] |  |  |  | res | res | res | res |
|  |  |  |  |  |  |  |  | R | R | R | R |  |  |  |  |

IR indicates the interrupt status. If IR is not equal to ‘0x00’, INTn PIN is asserted low until it is ‘0x00’.

[4] WOL – Magic Packet

When WOL mode is enabled and receives the magic packet over UDP, this bit is set.

[5] PPPoE – PPPoE Close

When PPPoE is disconnected during PPPoE mode, this bit is set.

[6] UNREACH – Destination unreachable

When receiving the ICMP (Destination port unreachable) packet, this bit is set as ‘1’. When this bit is ‘1’, Destination Information such as IP address and Port number may be checked with the corresponding UIPR & UPORTR.

[7] Conflict – IP Conflict

Bit is set as ‘1’ when own source IP address is same with the sender IP address in the received ARP request.

IMR (Interrupt Mask Register)

Address Offset : 0x2104

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | IMR[7:4] |  |  |  | res | res | res | res |
|  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |

IMR is used to mask interrupts. Each bit of IMR corresponds to each bit of IR. When a bit of IMR is ‘1’ and the corresponding bit of IR is ‘1’, an interrupt will be issued. In other words, if a bit of IMR is ‘0’, an interrupt will not be issued even if the corresponding bit of IR is ‘1’.

[4] Magic Packet

0: Disable Magic Packet Interrupt

1: Enable Magic Packet Interrupt

[5] PPPoE Close Interrupt Mask

0: Disable PPPoE Close Interrupt

1: Enable PPPoE Close Interrupt

[6] Destination unreachable Interrupt Mask

0: Disable Destination unreachable Interrupt

1: Enable Destination unreachable Interrupt

[7] IP Conflict Interrupt Mask

0: Disable IP Conflict Interrupt

1: Enable IP Conflict Interrupt

IRCR (Interrupt Clear Register)

Address Offset : 0x2108

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | IRC[8:4] |  |  |  | res | res | res | res |
|  |  |  |  |  |  |  |  | *R/C_W1 |  |  |  |  |  |  |  |

IRCR is used to clear interrupts. Each bit of IR can be cleared when the host writes ‘1’ value to each bit of IRCR corresponding to each bit of IR.

* ReadClearWrite1 (R/C_W1) : Software can read as well as clear this bit by writing ‘1’. Writing ‘0’ has no effect on the bit value.

[4] Magic Packet Interrupt Clear

[5] PPPoE Close Interrupt Clear

[6] Destination unreachable Interrupt Clear

[7] IP Conflict Interrupt Clear

SIR (Socket Interrupt Register)

Address Offset : 0x2110

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | S7 | S6 | S5 | S4 | S3 | S2 | S1 | S0 |
|  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

SIR indicates the interrupt status of Socket. Each bit of SIR be still ‘1’ until Sn_IR is cleared by the host. If Sn_IR is not equal to ‘0x00’, the n-th bit of SIR is ‘1’ and INTn PIN is asserted until SIR is ‘0x00’

[7:0] SIR - When the interrupt of Socket n occurs, the n-th bit of SIR becomes ‘1’.

SIMR (Socket Interrupt Mask Register)

Address Offset : 0x2114

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | S7 | S6 | S5 | S4 | S3 | S2 | S1 | S0 |
|  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

Each bit of SIMR corresponds to each bit of SIR. When a bit of SIMR is ‘1’ and the corresponding bit of SIR is ‘1’, Interrupt will be issued. In other words, if a bit of SIMR is ‘0’, an interrupt will be not issued even if the corresponding bit of SIR is ‘1’.

[7:0] SIR - Socket n Interrupt Mast

0: Disable Socket n Interrupt

1: Enable Socket n Interrupt

MR (Mode Register)

Address Offset : 0x2300

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | NTR | res | RST | res | WOL | PB | res | NSC | FA | res |
|  |  |  |  |  |  | R/W |  | R/W |  | R/W | R/W |  | R/W | R/W |  |

MR is used for S/W reset, ping block mode and PPPoE mode

[2] NSC – MACRAW No Size Check

If this bit is ‘1’, it does not check packet size In MACRAW mode.

0 : Enable Check Packet Size

1 : Disable Check Packet Size

[4] PB – Ping Block

If the bit is ‘1’, it blocks the response to a ping request.

0: Disable Ping block

1: Enable Ping block

[5] WOL – Wake on Lan

If WOL mode is enabled and the received magic packet over UDP has been normally processed, the Interrupt PIN (INTn) asserts to low. When using WOL mode, the UDP Socket should be opened with any source port number. (Refer to Socket n Mode Register (Sn_MR) for opening Socket.)

Notice: The magic packet over UDP supported by TOE consists of 6 bytes synchronization stream (‘0xFFFFFFFFFFFF’) and 16 times Target MAC address stream in UDP payload. The options such like password are ignored. You can use any UDP source port number for WOL mode.

0: Disable WOL mode

1: Enable WOL mode

[7] RST – Software Reset

If this bit is ‘1’, All internal registers will be initialized. It will be automatically cleared as ‘0’ after S/W reset.

[10] NTR – No Send TCP Reset Packet

If this bit is ‘1’, in TCP mode, Socket does not send Reset Packet.

0 : Send Reset Pakcet

1 : No Send Reset Packet

PTIMER (PPP Link Control Protocol Request Timer Register)

Address Offset : 0x2400

Reset value : 0x0000_0028

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | Res | res | res | res | res | PTIME[7:0] |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

[7:0] PTIME configures the time for sending LCP echo request. The unit of time is 25ms

Ex) in case that PTIMER is 200,

200 * 25(ms) = 5000(ms) = 5 seconds

PMAGICR (PPP Link Control Protocol Magic number Register)

Address Offset : 0x2404

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | Res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | Res | res | res | res | res | PMAGIC[7:0] |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

PMAGICR configures the 4bytes magic number to be used in LCP echo request.

[7:0] PMAGIC

Ex) PMAGIC = 0x01

0x001D

0x01

LCP Magic number = 0x01010101

PHAR (Destination Hardware Address Register in PPPoE)

Address Offset : 0x2408

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PHAR[31:24] |  |  |  |  |  |  |  | PHAR1[23:16] |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| PHAR[15:8] |  |  |  |  |  |  |  | PHAR[7:0] |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

Address Offset : 0x240C

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PHAR[31:24] |  |  |  |  |  |  |  | PHAR[23:16] |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res |  |  |  |  |  |  |  | res |  |  |  |  |  |  |  |

PHAR should be written to the PPPoE server hardware address acquired in PPPoE connection process.

PHAR0 and PHAR1 – configures Destination hardware address

Ex) In case that destination hardware address is 00:08:DC:12:34:56

| PHAR0[32:24] | PHAR0[23:16] | PHAR0[15:8] | PHAR0[32:24] | PHAR1[32:24] | PHAR1[23:16] |
| --- | --- | --- | --- | --- | --- |
| 0x00 | 0x08 | 0xDC | 0x12 | 0x34 | 0x56 |

PSIDR (Session ID Register in PPPoE)

Address Offset : 0x2410

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | Res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| PSID[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

[15:0] PSID - should be written to the PPPoE sever session ID acquired in PPPoE connection process.

PMRUR (Maximum Receive Unit Register in PPPoE)

Address Offset : 0x2414

Reset value : 0x0000_FFFF

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | Res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| PMSS[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

[15:0] PMRUR configures the maximum receive unit of PPPoE.

Ex) in case that maximum receive unit in PPPoE is 0x1234

| PMSS[15:8] | PMSS[7:0] |
| --- | --- |
| 0x12 | 0x34 |

SHAR (Source Hardware Address Register)

Address Offset : 0x6000

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SHAR0[31:24] |  |  |  |  |  |  |  | SHAR0[23:16] |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| SHAR0[15:8] |  |  |  |  |  |  |  | SPHAR0[7:0] |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

Address Offset : 0x6004

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SHA1[31:24] |  |  |  |  |  |  |  | SHAR1[23:16] |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res |  |  |  |  |  |  |  | res |  |  |  |  |  |  |  |

SHAR configures the source hardware address.

Ex) In case of “00.08.DC.12.34.56”

| SHAR0[32:24] | SHAR0[23:16] | SHAR0 [15:8] | SHAR0 [32:24] | SHAR1 [32:24] | SHAR2 [23:16] |
| --- | --- | --- | --- | --- | --- |
| 0x00 | 0x08 | 0xDC | 0x12 | 0x34 | 0x56 |

GAR (Gateway Address)

Address Offset : 0x6008

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GA[31:24] |  |  |  |  |  |  |  | GA[23:16] |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| GA[15:8] |  |  |  |  |  |  |  | GA[7:0] |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

GAR[31:0] – configures the default gateway address

Ex) In case of “192.168.0.1”

| GA[31:24] | GA[23:16] | GA[15:8] | GA[7:0] |
| --- | --- | --- | --- |
| 192 (0xC0) | 168 (0xA8) | 0 (0x00) | 1 (0x01) |

SUBR ( Subnet Mask Register)

Address Offset : 0x600C

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SUB[31:24] |  |  |  |  |  |  |  | SUB[23:16] |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| SUBR[15:8] |  |  |  |  |  |  |  | SUBR[7:0] |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

SUBR configures the subnet mask address.

Ex) In case of “255.255.255.0”

| SUB[31:24] | SUB[23:16] | SUB[15:8] | SUB[7:0] |
| --- | --- | --- | --- |
| 255 (0xFF) | 255 (0xFF) | 255 (0xFF) | 0 (0x00) |

SIPR (Source IP address Register)

Address Offset : 0x6010

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SIPR[31:24] |  |  |  |  |  |  |  | SIPR[23:16] |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| SIPR[15:8] |  |  |  |  |  |  |  | SIPR[7:0] |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

SIPR configures the source IP address.

Ex) In case of “192.168.0.2”

| SIPR[31:24] | SIPR[23:16] | SIPR[15:8] | SPIR[7:0] |
| --- | --- | --- | --- |
| 192 (0xC0) | 168 (0xA8) | 0 (0x00) | 2 (0x02) |

NCONFLR ( Network Configuration Lock Register)

Address Offset : 0x6020

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NCONFL[31:24] |  |  |  |  |  |  |  | NCONFL [23:16] |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| NCONFL [15:8] |  |  |  |  |  |  |  | NCONFL [7:0] |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

NCONFLR is used to unlock and lock the network configuration registers which are SIR, SUBR, GAR and SHAR. When LOCK is ‘ON’, the protected registers are not able to access. In this case a value of 0x01ACCE55 is written to NCONFLR. When LOCK is ‘OFF’, the protected registers are allowed to access. In this case any value except 0x01ACCE55 is written.

Ex) In case of ‘LOOK is ON’

| NCONFL[31:24] | NCONFL [23:16] | NCONFL [15:8] | NCONFL [7:0] |
| --- | --- | --- | --- |
| 1 (0x01) | 172 (0xAC) | 206 (0xCE) | 85 (0x55) |

RTR (Retry Time Register)

Address Offset : 0x6040

Reset value : 0x0000_07D0

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| RTR[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

RTR configures the retransmission timeout period. The unit of timeout period is 100us(Refer to TCKCNTR) and the default of RTR is ‘0x07D0’ or ‘2000’. And so the default timeout period is 200ms(100us X 2000).

During the time configured by RTR, WZTOE waits for the peer response to the packet that is transmitted by Sn_CR(CONNECT, DISCON, CLOSE, SEND, SEND_MAC, SEND_KEEP command). If the peer does not respond within the RTR time, WZTOE retransmits the packet or issues timeout.

When RTR is not ‘0x0000_0000, RTR is used to configure the timeout period of all socket. When RTR is ‘0x0000_0000’, the timeout period of each socket could set by using Socket n Retry Time Register (Sn_RTR).

Ex) When timeout-period is set as 400ms, RTR = (400ms / 1ms) X 10 = 4000(0x0FA0)

RTR[15:8]

0x0F

RCR (Retry Counter Register)

Address Offset : 0x6044

Reset value : 0x0000_0008

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  |  |  |  |  |  |  | res |  |  |  |  |  |  |  |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res |  |  |  |  |  |  |  | RC[7:0] |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

RCR configures the number of time of retransmission. When retransmission occurs as many as ‘RCR+1’, Timeout interrupt is issued (Sn_IR[TIMEOUT] = ‘1’).

When RCR is not ‘0x0000_0000, RCR is used to configure the timeout period of all socket. When RCR is ‘0x0000_0000’, the timeout period of each socket could set by using Socket n Retry Counter Register (Sn_RCR).

Ex) RCR = 0x0007

RC[7:0]

0x07

The timeout of WZTOE can be configurable with RTR and RCR. WZTOE has two kind timeout such as Address Resolution Protocol (ARP) and TCP retransmission.

At the ARP (Refer to RFC 826, http://www.ietf.org/rfc.html) retransmission timeout, WZTOE automatically sends ARP-request to the peer’s IP address in order to acquire MAC address information (used for communication of IP, UDP, or TCP). While waiting for ARP-response from the peer, if there is no response during the configured RTR time, a temporary timeout is occurred and ARP-request is retransmitted. It is repeated as many as ‘RCR + 1’ times. Even after the ARP-request retransmissions are repeated as ‘RCR+1’ and there is no response to the ARP-request, the final timeout is occurred and Sn_IR(TIMEOUT) becomes ‘1’. The time of final timeout (ARPTO) of ARP-request is as below.

ARPTO= RTR×0.1ms× RCR+1

At the TCP packet retransmission timeout, WZTOE transmits TCP packets (SYN, FIN, RST, DATA packets) and waits for the acknowledgement (ACK) during the configured RTR time and RCR. If there is no ACK from the peer, a temporary timeout occurs and the TCP packet is retransmitted. The retransmission is repeated as many as ‘RCR+1’. Even after TCP retransmission is repeated as ‘RCR+1’ and there is no response to the TCP retransmission, the final timeout is occurred and Sn_IR(TIMEOUT) becomes ‘1’. The time of final timeout (TCPTO) of TCP retransmission is as below.

TCPTO= N=0MRTR×2N + RCR-M× RTRMAX×0.1ms N : Retransmission count, 0≤N ≤ M M : Minimum value when RTR x 2(M+1) > 65535 and 0 ≤ M ≤ RCR RTRMAX : RTR x 2M

Ex) When RTR = 2000(0x07D0), RCR = 8(0x0008),

ARPTO = 2000 X 0.1ms X 9 = 1800ms = 1.8s

TCPTO = (0x07D0+0x0FA0+0x1F40+0x3E80+0x7D00+0xFA00+0xFA00+0xFA00+0xFA00) X 0.1ms

= (2000 + 4000 + 8000 + 16000 + 32000 + ((8 - 4) X 64000)) X 0.1ms

= 318000 X 0.1ms = 31.8s

UIPR (Unreachable IP address Register)

Address Offset : 0x6050

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UIP[31:24] |  |  |  |  |  |  |  | UIPR[23:16] |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| UIPR[15:8] |  |  |  |  |  |  |  | UIPR[7:0] |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

TOE receives an ICMP packet(Destination port unreachable) when data is sent to a port number which socket is not open and UNREACH bit of IR becomes ‘1’ and UIPR indicates the destination IP address.

Ex) In case of “192.168.0.11”

| UIP[31:24] | UIP[23:16] | UIP[15:8] | UIP[7:0] |
| --- | --- | --- | --- |
| 192 (0xC0) | 168 (0xA8) | 0 (0x00) | 11 (0x0E) |

UPORTR (Unreachable Port Register)

Address Offset : 0x6054

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| UPORT[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

TOE receives an ICMP packet(Destination port unreachable) when data is sent to a port number which socket is not open and UNREACH bit of IR becomes ‘1’ and UPORTR indicates the destination port number.

[15 :0] UPORT – Destination port number bits

Ex) In case of “0x1234”

| UPORT[15:8] | UPORT[7:0] |
| --- | --- |
| 18 (0x12) | 52(0x34) |

Socket register (Base : 0x4601_0000 + 0x0004_000 x n)[n=0,…7, where n is socket number]

Sn_MR (Socket n Mode Register)

Address Offset : 0x0000

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | Sn_MR[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |

Sn_MR configures the option or protocol type of Socket n.

[3:0] These bits configures the protocol mode of Socket n as follows

| Sn_MR[3:0] |  |  |  | Meaning |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | Closed |
| 0 | 0 | 0 | 1 | TCP |
| 0 | 0 | 1 | 0 | UDP |
| 0 | 1 | 0 | 0 | MACRAW |

- MACRAW mode should be only used in Socket 0.

[4] UNICAST Blocking and IPv6 packet Blocking

UNICAST Blocking in UDP mode

0 : disable Unicast Blocking

1 : enable Unicast Blocking

This bit blocks receiving the unicast packet during UDP mode(P[3:0] = ‘0010’) and MULTI = ‘1’.

IPv6 packet Blocking in MACRAW mode

0 : disable IPv6 Blocking

1 : enable IPv6 Blocking

This bit is applied only during MACRAW mode (P[3:0] = ‘0100’). It blocks to receiving the IPv6 packet.

[5] Use No Delayed ACK, Multicast and Multicast Blocking mode

Use No Delayed ACK

0 : Disable No Delayed ACK option

1 : Enable No Delayed ACK option

This bit is applied only during TCP mode (P[3:0] = ‘0001’).When this bit is ‘1’, It sends the ACK packet without delay as soon as a

Multicast

0 : using IGMP version 2

1 : using IGMP version 1

This bit is applied only during UDP mode(P[3:0] = ‘0010’) and MULTI = ‘1’. It configures the version for IGMP messages (Join/Leave/Report).

Multicast Blocking in MACRAW mode

0 : disable Multicast Blocking

1 : enable Multicast Blocking

This bit is applied only when MACRAW mode(P[3:0] = ‘0100’). It blocks to receive the packet with multicast MAC address.

[6] Broadcast Blocking in MACRAW and UDP mode

0 : disable Broadcast Blocking

1 : enable Broadcast Blocking

This bit blocks to receive broadcasting packet during UDP mode(P[3:0] = ‘0010’). In addition, This bit does when MACRAW mode(P[3:0] = ‘0100’) Data packet is received from a peer. When this bit is ‘0’, It sends the ACK packet after waiting for the timeout time configured by RTR.

[7] Multicasting and MAC Filter Enable mode

Multicasting in UDP mode

0 : disable Multicasting

1 : enable Multicasting

This bit is applied only during UDP mode(P[3:0] = ‘0010’).To use multicasting, Sn_DIPR & Sn_DPORT should be respectively configured with the multicast group IP address & port number before Socket n is opened by OPEN command of Sn_CR

MAC Filter Enable in MACRAW mode

0 : disable MAC Filtering

1 : enable MAC Filtering

This bit is applied only during MACRAW mode(P[3:0] = ‘0100’).When set as ‘1’, WZTOE can only receive broadcasting packet or packet sent to itself. When this bit is ‘0’, WZTOE can receive all packets on Ethernet. If user wants to implement Hybrid TCP/IP stack, it is recommended that this bit is set as ‘1’ for reducing host overhead to process the all received packets.

Sn_CR (Socket n Command Register)

Address Offset : 0x0010

Reset value : 0x0000_0000

| 31 |  | 30 |  |  | 29 |  |  | 28 |  |  | 27 |  |  | 26 |  |  | 25 |  |  | 24 |  |  | 23 |  |  | 22 |  |  |  | 21 |  |  | 20 |  |  | 19 |  |  | 18 |  |  | 17 |  |  | 16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  |  | Res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |
| 15 |  | 14 |  |  | 13 |  |  | 12 |  |  | 11 |  |  | 10 |  |  | 9 |  |  | 8 |  |  | 7 |  |  | 6 |  |  |  | 5 |  |  | 4 |  |  | 3 |  |  | 2 |  |  | 1 |  |  | 0 |  |
| res |  |  | Res |  |  | Res |  |  | res |  |  | res |  |  | res |  |  |  | res |  |  | res |  |  | Sn_CR[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  | R/W |  |  | R/W |  |  | R/W |  |  | R/W |  |  | R/W |  |  | R/W |  |  | R/W |

This is used to set the command for Socket n such as OPEN, CLOSE, CONNECT, LISTEN, SEND, and RECEIVE. After WZTOE accepts the command, the Sn_CR register is automatically cleared to 0x00. Even though Sn_CR is cleared to 0x00, the command is still being processed. To check whether the command is completed or not, please check the Sn_IR or Sn_SR.

| Value | Symbol | Description |
| --- | --- | --- |
| 0x01 | OPEN | Socket n is initialized and opened according to the protocol selected in Sn_MR (P3:P0). The table below shows the value of Sn_SR corresponding to Sn_MR. Sn_MR (P[3:0]); Sn_SR. Sn_MR_CLOSE (‘0000’); -. Sn_MR_TCP (‘0001’); SOCK_INIT (0x13). Sn_MR_UDP (‘0010’); SOCK_UDP (0x22). S0_MR_MACRAW (‘0100’); SOCK_MACRAW (0x02). . |
| 0x02 | LISTEN | This is valid only in TCP mode (Sn_MR(P3:P0) = Sn_MR_TCP). In this mode, Socket n operates as a ‘TCP server’ and waits for connection-request (SYN packet) from any ‘TCP client’. The Sn_SR changes the state from SOCK_INIT to SOCKET_LISTEN. When a ‘TCP client’ connection request is successfully established, the Sn_SR changes from SOCK_LISTEN to SOCK_ESTABLISHED and the Sn_IR(0) becomes ‘1’. But when a ‘TCP client’ connection request is failed, Sn_IR(3) becomes ‘1’ and the status of Sn_SR changes to SOCK_CLOSED. |
| 0x04 | CONNECT | This is valid only in TCP mode and operates when Socket n acts as ‘TCP client’. To connect, a connect-request (SYN packet) is sent to ‘TCP server’ configured by Sn_DIPR & Sn_DPORT(destination address & port). If the connect-request is successful, the Sn_SR is changed to SOCK_ESTABLISHED and the Sn_IR(0) becomes ‘1’. The connect-request fails in the following three cases. 1. When a ARPTO occurs (Sn_IR(3)=‘1’) because the destination hardware address is not acquired through the ARP-process. 2. When a SYN/ACK packet is not received and TCPTO (Sn_IR(3) = ‘1 ) 3. When a RST packet is received instead of a SYN/ACK packet. In these cases, Sn_SR is changed to SOCK_CLOSED. |
| 0x08 | DISCON | Valid only in TCP mode. Regardless of ‘TCP server’ or ‘TCP client’, the DISCON command processes the disconnect-process (‘Active close’ or ‘Passive close’). Active close: it transmits disconnect-request(FIN packet) to the connected peer Passive close: When FIN packet is received from peer, a FIN packet is replied back to the peer. When the disconnect-process is successful (that is, FIN/ACK packet is received successfully), Sn_SR is changed to SOCK_CLOSED. Otherwise, TCPTO occurs (Sn_IR(3)=‘1)= and then Sn_SR is changed to SOCK_CLOSED. cf> If CLOSE is used instead of DISCON, only Sn_SR is changed to SOCK_CLOSED without disconnect-process. If a RST packet is received from a peer during communication, Sn_SR is unconditionally changed to SOCK_CLOSED. |
| 0x10 | CLOSE | Close Socket n. Sn_SR is changed to SOCK_CLOSED. |
| 0x20 | SEND | SEND transmits all the data in the Socket n TX buffer. For more details, please refer to Socket n TX Free Size Register (Sn_TX_FSR), Socket n, TX Write Pointer Register (Sn_TX_WR), and Socket n TX Read Pointer Register(Sn_TX_RD). |
| 0x21 | SEND_MAC | Valid only in UDP mode. The basic operation is same as SEND. Normally SEND transmits data after destination hardware address is acquired by the automatic ARP-process(Address Resolution Protocol). But SEND_MAC transmits data without the automatic ARP-process. In this case, the destination hardware address is acquired from Sn_DHAR configured by host, instead of APR-process. |
| 0x22 | SEND_KEEP | Valid only in TCP mode. It checks the connection status by sending 1byte keep-alive packet. If the peer cannot respond to the keep-alive packet during timeout time, the connection is terminated and the timeout interrupt will occur. |
| 0x40 | RECV | RECV completes the processing of the received data in Socket n RX Buffer by using a RX read pointer register (Sn_RX_RD). For more details, refer to Socket n RX Received Size Register (Sn_RX_RSR), Socket n RX Write Pointer Register (Sn_RX_WR), and Socket n RX Read Pointer Register (Sn_RX_RD). |

Sn_IR (Socket n Interrupt Register)

Address Offset : 0x0020

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res |  | Res |  | Res |  | res |  | res |  | Res |  | res |  | res |  | res |  | res |  | res |  | Sn_IR[4:0] |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  | R |  | R |  | R |  | R |

Sn_IR indicates the status of Socket Interrupt such as establishment, termination, receiving data, timeout). When an interrupt occurs and the corresponding bit of Sn_IMR is ‘1’, the corresponding bit of Sn_IR becomes ‘1’.

[0] CONNECT Interrupt - This is issued one time when the connection with peer is successful and then Sn_SR is changed to SOCK_ESTABLISHED

[1] DISCONNECT Interrupt - This is issued when FIN or FIN/ACK packet is received.

[2] RECV Interrupt - This is issued whenever data is received from a peer.

[3] TIMEOUT Interrupt - This is issued when ARPTO or TCPTO occurs.

[4] SENDOK Interrupt - This is issued when SEND command is completed

Sn_IMR (Socket n Interrupt Mask Register)

Address Offset : 0x0024

Reset value: 0x0000_00FF

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | Sn_IMR[4:0] |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |

Sn_IMR is used to mask interrupts. Each bit of Sn_IMR corresponds to each bit of Sn_IR. When a bit of Sn_IMR is ‘1’ and the corresponding bit of Sn_IR is ‘1’, an interrupt will be issued. In other words, if a bit of Sn_IMR is ‘0’, an interrupt will not be issued even if the corresponding bit of Sn_IR is ‘1’.

[0] CONNECT Interrupt Mask

0: Disable CONNECT Interrupt

1: Enable CONNECT Interrupt

[1] DISCONNECT Interrupt Mask

0: Disable DISCONNECT Interrupt

1: Enable DISCONNECT Interrupt

[2] RECV Interrupt Mask

0: Disable RECV Interrupt

1: Enable RECV Interrupt

[3] TIMEOUT Interrupt Mask

0: Disable TIMEOUT Interrupt

1: Enable TIMEOUT Interrupt

[4] SENDOK Interrupt Mask

0: Disable SENDOK Interrupt

1: Enable SENDOK Interrupt

Sn_ICR (Socket n Interrupt Clear Register)

Address Offset : 0x0028

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  |  |  |  |  | Sn_ICR[4:0] |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |

Sn_ICR is used to clear interrupts. Each bit of Sn_IR can be cleared when the host writes ‘1’ value to each bit of Sn_ICR corresponding to each bit of Sn_IR.

[0] CONNECT Interrupt Clear

[1] DISCONNECT Interrupt Clear

[2] RECV Interrupt Mask

[3] TIMEOUT Interrupt Mask

[4] SENDOK Interrupt Mask

* ReadClearWrite1 (R/C_W1) : Software can read as well as clear this bit by writing 1. Writing ‘0’ has no effect on the bit value.

Sn_SR (Socket n Status Register)

Address Offset : 0x0030

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | Sn_SR[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |

Sn_SR indicates the status of Socket n. The status of Socket n is changed by Sn_CR or some special control packet as SYN, FIN packet in TCP.

| Value | Symbol | Description |
| --- | --- | --- |
| 0x00 | SOCK_CLOSED | This indicates that Socket n is released. When DISCON, CLOSE command is ordered, or when a timeout occurs, it is changed to SOCK_CLOSED regardless of previous status. |
| 0x13 | SOCK_INIT | This indicates Socket n is opened with TCP mode. It is changed to SOCK_INIT when Sn_MR (P[3:0]) = ‘0001’ and OPEN command is ordered. After SOCK_INIT, user can use LISTEN /CONNECT command. |
| 0x14 | SOCK_LISTEN | This indicates Socket n is operating as ‘TCP server’ mode and waiting for connection-request (SYN packet) from a peer (‘TCP client’). It will change to SOCK_ESTALBLISHED when the connection-request is successfully accepted. Otherwise it will change to SOCK_CLOSED after TCPTO occurred (Sn_IR(TIMEOUT) = ‘1’). |
| 0x17 | SOCK_ESTABLISHED | This indicates the status of the connection of Socket n. It changes to SOCK_ESTABLISHED when the ‘TCP SERVER’ processed the SYN packet from the ‘TCP CLIENT’ during SOCK_LISTEN, or when the CONNECT command is successful. During SOCK_ESTABLISHED, DATA packet can be transferred using SEND or RECV command. |
| 0x1C | SOCK_CLOSE_WAIT | This indicates Socket n received the disconnect-request (FIN packet) from the connected peer. This is half-closing status, and data can be transferred. For full-closing, DISCON command is used. But For just-closing, CLOSE command is used. |
| 0x22 | SOCK_UDP | This indicates Socket n is opened in UDP mode(Sn_MR(P[3:0]) = ‘0010’). It changes to SOCK_UDP when Sn_MR(P[3:0]) = ‘0010’) and OPEN command is ordered. Unlike TCP mode, data can be transfered without the connection-process. |
| 0x42 | SOCK_MACRAW | This indicates Socket 0 is opened in MACRAW mode (S0_MR(P[3:0]) = ‘0100’)and is valid only in Socket 0. It changes to SOCK_MACRAW when S0_MR(P[3:0] = ‘0100’ and OPEN command is ordered. Like UDP mode socket, MACRAW mode Socket 0 can transfer a MAC packet (Ethernet frame) without the connection-process. |

The following table shows a temporary status indicated during changing the status of Socket n.

| Value | Symbol | Description |
| --- | --- | --- |
| 0x15 | SOCK_SYNSENT | This indicates Socket n sent the connect-request packet (SYN packet) to a peer. It is temporarily shown when Sn_SR is changed from SOCK_INIT to SOCK_ESTABLISHED by CONNECT command. If connect-accept(SYN/ACK packet) is received from the peer at SOCK_SYNSENT, it changes to SOCK_ESTABLISHED. Otherwise, it changes to SOCK_CLOSED after TCPTO (Sn_IR[TIMEOUT] = ‘1’) is occurred. |
| 0x16 | SOCK_SYNRECV | It indicates Socket n successfully received the connect-request packet (SYN packet) from a peer. If socket n sends the response (SYN/ACK packet) to the peer successfully, it changes to SOCK_ESTABLISHED. If not, it changes to SOCK_CLOSED after timeout occurs (Sn_IR[TIMEOUT] = ‘1’). |
| 0x18 | SOCK_FIN_WAIT | These indicate Socket n is closing. These are shown in disconnect-process such as active-close and passive-close. When Disconnect-process is successfully completed, or when timeout occurs, these change to SOCK_CLOSED. |
| 0x1A | SOCK_CLOSING |  |
| 0X1B | SOCK_TIME_WAIT |  |
| 0X1D | SOCK_LAST_ACK | This indicates Socket n is waiting for the response (FIN/ACK packet) to the disconnect-request (FIN packet) by passive-close. It changes to SOCK_CLOSED when Socket n received the response successfully, or when timeout occurs (Sn_IR[TIMEOUT] = ‘1’). |

Sn_PNR (Socket n Protocol Number Register)

Address Offset : 0x0100

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | Sn_PNR[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |

This IP Protocol Register is user to set up the Protocol Field of IP Header at the IP layer RAW mode. There are several protocol number defined in advance by registering to IANA. For the overall list of upper level protocol identification number that IP is using, refer to online documents of IANA (http://www.iana.org/assignments/protocol-numbers).

Ex) Internet Control Message Protocol (ICMP) = 0x01, Internet Group Management Protocol = 0x02

Sn_TOSR (Socket n IP Type of Service Register)

Address Offset : 0x0104

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | Sn_ROS[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |

Sn_TOSR configures the TOS(Type Of Service field in IP Header) of Socket n. It is set before OPEN command.

For more the details, refer to http://www.iana.org/assignments/ip-parameters.

Sn_TTLR (Socket n TTL Register)

Address Offset : 0x0108

Reset value : 0x0000_0080

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | Sn_TTL[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |

Sn_TTL configures the TTL(Time To Live field in IP Header) of Socket n. It is set before OPEN command.

For more the details, refer to http://www.iana.org/assignments/ip-parameters.

Sn_FRAGR (Socket n Fragment offset Register)

Address Offset : 0x010C

Reset value : 0x0000_4000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| Sn_FRAG[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |

[15:0] Sn_FRAG configures the FRAG(Fragment field in IP header)

Ex) Sn_FRAGR = 0x0000 (Don’t Fragment)

| Sn_FRAG[15:8] | Sn_FRAG[7:0] |
| --- | --- |
| 0x00 | 0x00 |

Sn_MSSR (Socket n Maximum Segment Register)

Address Offset : 0x0110

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| Sn_MSS[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |

This register is used for MSS (Maximum Segment Size) of TCP, and the register displays MSS set by the other party when TCP is activated in Passive Mode.

Ex) In case of Socket 0 MSS = 1460 (0x05B4), configure as below,

0x4101_0110

0x05B4

Sn_PORTR (Socket n Source Port Register)

Address Offset : 0x0114

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| Sn_PORT[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |

Sn_PORTR configures the source port number of Socket n. It is valid when Socket n is used in TCP/UDP mode. It should be set before OPEN command is ordered.

Ex) In case of Socket 0 Port = 5000(0x1388), configure as below,

0x4101_0114

0x1388

Sn_DHAR (Socket n Destination Hardware address Register)

Address Offset : 0x0118

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sn_DHAR0[31:24] |  |  |  |  |  |  |  | Sn_DHAR0 [23:16] |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Sn_DHAR0 [15:8] |  |  |  |  |  |  |  | Sn_DHAR0 [7:0] |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

Address Offset : 0x011E

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 |  | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Sn_DHAR1 [31:24] |  |  |  |  |  |  |  | Sn_DHAR1 [23:16] |  |  |  |  |  |  |  |
| R | R | R | R | R |  | R | R | R | R | R | R | R | R | R | R | R |

Address Offset : 0x011C

Reset value : 0x0000_0000

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sn_DHAR1 [15:8] |  |  |  |  |  |  |  | Sn_DHAR1 [7:0] |  |  |  |  |  |  |  |
| W | W | W | W | W | W | W | W | W | W | W | W | W | W | W | W |

Sn_DHAR configures the destination hardware address of Socket n when using SEND_MAC command in UDP mode or it indicates that it is acquired in ARP-process by CONNECT/SEND command.

Ex) In case of writing MAC address “00.08.DC.12.34.56”

| Sn_DHAR0 [31:24] | Sn_DHAR0 [23:16] | Sn_DHAR0 [15:8] | Sn_DHAR0 [7:0] | Sn_DHAR1 [15:8] | Sn_DHAR1 [7:0] |
| --- | --- | --- | --- | --- | --- |
| 0x00 | 0x08 | 0xDC | 0x12 | 0x34 | 0x56 |

Ex) In case of reading MAC address “00.08.DC.12.34.56”

| Sn_DHAR0 [31:24] | Sn_DHAR0 [23:16] | Sn_DHAR0 [15:8] | Sn_DHAR0 [7:0] | Sn_DHAR1 [31:24] | Sn_DHAR1 [23:16] |
| --- | --- | --- | --- | --- | --- |
| 0x00 | 0x08 | 0xDC | 0x12 | 0x34 | 0x56 |

Sn_DPORTR (Socket n Destination Port Number Register)

Address Offset : 0x0120

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| Sn_DPROT[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |

Sn_DPORT configures or indicates the destination port number of Socket n. It is valid when Socket n is used in TCP/UDP mode.

In TCP client mode, it configures the listen port number of ‘TCP server’ before CONNECT command.

In TCP server mode, it indicates the port number of ‘TCP client’ after successfully establishing connection.

In UDP mode, it configures the port number of peer to be transmitted the UDP packet by SEND/SEND_MAC command.

Ex) In case of Socket 0 Destination Port = 5000(0x1388), configure as below,

0x4101_0120

0x1388

Sn_DIPR (Socket n Destination IP address Register)

Address Offset : 0x0124

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sn_DIPR[31:24] |  |  |  |  |  |  |  | Sn_DIPR [23:16] |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Sn_DIPR [15:8] |  |  |  |  |  |  |  | Sn_DIPR [7:0] |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

Sn_DIPR configures or indicates the destination IP address of Socket n. It is valid when Socket n is used in TCP/UDP mode.

In TCP client mode, it configures an IP address of ‘TCP server’ before CONNECT command.

In TCP server mode, it indicates an IP address of ‘TCP client’ after successfully establishing connection.

In UDP mode, it configures an IP address of peer to be received the UDP packet by SEND or SEND_MAC command.

Ex) In case of “192.168.0.2”

| Sn_DIPR[31:24] | Sn_DIPR [23:16] | Sn_DIPR [15:8] | Sn_DIPR [7:0] |
| --- | --- | --- | --- |
| 192 (0xC0) | 168 (0xA8) | 0 (0x00) | 2 (0x02) |

Sn_KATMR (Socket n Keep Alive Timer Register)

Address Offset : 0x0180

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | Sn_KATM[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |

Sn_KPALVTR configures the transmitting timer of ‘KEEP ALIVE(KA)’ packet of SOCKETn. It is valid only in TCP mode, and ignored in other modes. The time unit is 5s(Refer to TCKCNTR).

KA packet is transmittable after Sn_SR is changed to SOCK_ESTABLISHED and after the data is transmitted to a peer at least once. In case of 'Sn_KPALVTR > 0', WZTOE automatically transmits KA packet after time-period for checking the TCP connection (Auto-keepalive-process). In case of 'Sn_KPALVTR = 0', Auto-keep-alive-process will not operate, and KA packet can be transmitted by SEND_KEEP command by the host (Manual-keep-alive-process). Manual-keep-alive-process is ignored in case of 'Sn_KPALVTR > 0'.

Ex) Sn_KPALVTR = 10 (Keep Alive packet will be transmitted every 50 seconds.)

Sn_KATM[7:0]

0x0A

Sn_RTR (Socket n  Retry Time Register)

Address Offset : 0x0184

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Sn_RTR[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

Sn_RTR configures the retransmission timeout period of Socket n. When Retry Counter Register(RTR) is zero, Sn_RTR is valid. The unit of timeout period is 100us(Refer to TCKCNTR).

During the time configured by Sn_RTR, WZTOE waits for the peer response to the packet that is transmitted by Sn_CR(CONNECT, DISCON, CLOSE, SEND, SEND_MAC, SEND_KEEP command). If the peer does not respond within the RTR time, WZTOE retransmits the packet or issues timeout.

Ex) When timeout-period is set as 400ms, RTR = (400ms / 1ms) X 10 = 4000(0x0FA0)

RTR[15:0]

0x0FA0

Sn_RCR (Socket n Retry Counter Register)

Address Offset : 0x0188

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  |  |  |  |  |  |  | Res |  |  |  |  |  |  |  |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res |  |  |  |  |  |  |  | Sn_RC[7:0] |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

Sn_RCR configures the number of time of retransmission of Socket n. When Retry Counter Register(RCR) is zero, Sn_RCR is valid. When retransmission occurs as many as ‘Sn_RCR+1’, Timeout interrupt is issued (Sn_IR[TIMEOUT] = ‘1’).

Ex) Sn_RCR = 0x0007

Sn_RC[7:0]

0x07

Sn_TXBUF_SIZE (Socket n TX Buffer Size Register)

Address Offset : 0x0200

Reset value : 0x0000_0000

| 31 | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | Sn_TMS[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |

Sn_TXBUF_SIZE configures the TX Buffer size of Socket n. Socket n TX Buffer size can be configured with 0,1,2,4,8 and 16 Kbytes. If a different size is configured, the data can’t be normally transmitted to a peer.

Although Socket n TX Buffer size is initially configured to 2Kbytes, user can be re-configure its size using Sn_TXBUF_SIZE. The total sum of Sn_TXBUF_SIZE cannot be exceed 16Kbytes. When exceeded, the data transmission error is occurred.

When all Sn_TXBUF_SIZE have been configured, Socket n TX Buffer is allocated with the configured size in 16KB TX Buffer and is assigned sequentially from Socket 0 to Socket 7.

Socket n TX Buffer can be accessible with 16bits Offset Address ranged from 0x0000 to 0xFFFF regardless of the configured size. (Refer to Sn_TX_WR & Sn_TX_RD).

| Value (dec) | 0 | 1 | 2 | 4 | 8 | 16 |
| --- | --- | --- | --- | --- | --- | --- |
| Buffer size | 0KB | 1KB | 2KB | 4KB | 8KB | 16KB |

Ex) Socket 0 TX Buffer Size = 4KB

0x4600_0200

0x04

Sn_TX_FSR (Socket n TX Free Size Register)

Address Offset : 0x0204

Reset value : 0x0000_0800

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |

Sn_TX_FSR indicates the free size of Socket n TX Buffer. It is initialized to the configured size by Sn_TX_SIZE. Data bigger than Sn_TX_SIZE should not be saved in the Socket n TX Buffer because the bigger data overwrites the previous saved data not yet sent. Therefore, check before saving the data to the Socket n TX Buffer, and if data is equal or smaller than its checked size, transmit the data with SEND/SEND_MAC command after saving the data in Socket n TX Buffer. But, if data is bigger than its checked size, transmit the data after dividing into the checked size and saving in the Socket n TX Buffer.

If Sn_MR(P[3:0]) is not TCP mode(‘0001’), it is automatically calculated as the difference between ‘Socket n TX Write Pointer (Sn_TX_WR)’ and ‘Socket n TX Read Pointer (Sn_TX_RD)’.

If Sn_MR(P[3:0]) is TCP mode(‘0001’), it is automatically calculated  as the difference between Sn_TX_WR and the internal ACK pointer which indicates the point of data is received already by the connected peer.

Ex) In case of 2048(0x0800) in S0_TX_FSR,

0x4600_0204

0x0800

Note) Because this register for representing the size information is 16 bits, it is impossible to read all bytes at the same time. Before 16 bit-read operation is not completed, the value may be changed.

Therefore, it is recommended that you read all 16-bits twice or more until getting the same value.

Sn_TX_RD (Socket n TX Read Pointer Register)

Address Offset : 0x0208

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| Sn_TXRP[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |

Sn_TX_RD is initialized by OPEN command. However, if Sn_MR(P[3:0]) is TCP mode(‘0001’), it is re-initialized while connecting with TCP.

After its initialization, it is auto-increased by SEND command. SEND command transmits the saved data from the current Sn_TX_RD to the Sn_TX_WR in the Socket n TX Memory. After transmitting the saved data, the SEND command increases the Sn_TX_RD as same as the Sn_TX_WR. If its increment value exceeds the maximum value 0xFFFF, (greater than 0x10000 and the carry bit occurs), then the carry bit is ignored and will automatically update with the lower 16bits value.

Sn_TX_WR (Socket n TX Write Pointer Register)

Address Offset : 0x020C

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| Sn_TXWP[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |

Sn_TX_WR is initialized by OPEN command. However, if Sn_MR(P[3:0]) is TCP mode(‘0001’), it is re-initialized while connecting with TCP.

It should be read or to be updated like as follows.

- Read the starting address for saving the transmitting data.

- Save the transmitting data from the starting address of Socket n TX buffer.

- After saving the transmitting data, update Sn_TX_WR to the increased value as many as transmitting data size. If the increment value exceeds the maximum value 0xFFFF(greater than 0x10000 and the carry bit occurs), then the carry bit is ignored and will automatically update with the lower 16bits value.

- Transmit the saved data in Socket n TX Buffer by using SEND/SEND command

Sn_RXBUF_SIZE (Socket n RX Buffer Size Register)

Address Offset : 0x0220

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| Res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | Sn_RXMS[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |

Sn_RXBUF_SIZE configures the RX Buffer size of Socket n. Socket n RX Buffer size can be configured with 1,2,4,8, and 16 Kbytes. If a different size is configured, the data cannot be normally received from a peer.

Although Socket n RX Buffer size is initially configured to 2Kbytes, user can re-configure its size using Sn_RXBUF_SIZE. The total sum of Sn_RX_SIZE cannot be exceed 16Kbytes. When exceeded, the data reception error is occurred.

When all Sn_RXBUF_SIZE have been configured, Socket n RX Memory is allocated with the configured size in 16KB RX Memory and is assigned sequentially from Socket 0 to Socket 7.

Socket n RX Buffer Block can be accessible with the 16bits Offset Address ranged from 0x0000 to 0xFFFF regardless of the configured size. (Refer to Sn_RX_RD & Sn_RX_WR).

| Value (dec) | 0 | 1 | 2 | 4 | 8 | 16 |
| --- | --- | --- | --- | --- | --- | --- |
| Buffer size | 0KB | 1KB | 2KB | 4KB | 8KB | 16KB |

Ex) Socket 0 RX Buffer Size = 8KB

0x4101_0220

0x08

Sn_RX_RSR (Socket n RX Received Size Register)

Address Offset : 0x0224

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| Sn_RX_RSR[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |

Sn_RX_RSR indicates the data size received and saved in Socket n RX Buffer. Sn_RX_RSR does not exceed the Sn_RXBUF_SIZE and is calculated as the difference between ‘Socket n RX Write Pointer (Sn_RX_WR)’ and ‘Socket n RX Read Pointer (Sn_RX_PD)’.

Ex) In case of 2048(0x0800) in S0_RXSR,

0x4101_0224

0x0800

Note) Because this register for representing the size information is 16 bits, it is impossible to read all bytes at the same time. Before 16 bit-read operation is not completed, the value may be changed.

Therefore, it is recommended that you read all 16-bits twice or more until getting the same value.

Sn_RX_RD (Socket n RX Read Pointer Register)

Address Offset : 0x0228

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| Sn_RXRP[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |

Sn_RX_RD is initialized by OPEN command. Make sure to be read or updated as follows.

1. Read the starting save address of the received data

2. Read data from the starting address of Socket n RX Buffer.

3. After reading the received data, Update Sn_RX_RD to the increased value as many as the reading size. If the increment value exceeds the maximum value 0xFFFF, that is, is greater than 0x10000 and the carry bit occurs, update with the lower 16bits value ignored the carry bit.

4. Order RECV command is for notifying the updated Sn_RX_RD to TOE.

Ex) In case of 2048(0x0800) in S0_RX_RD,

0x4101_0228

0x08

Sn_RX_WR (Socket n RX Write Pointer Register)

Address Offset : 0x022C

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Res |  | Res |  | Res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| Sn_RX_WR[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |

Sn_RX_WR is initialized by OPEN command and it is auto-increased by the data reception.

If the increased value exceeds the maximum value 0xFFFF, (greater than 0x10000 and the carry bit occurs), then the carry bit is ignored and will automatically update with the lower 16bits value.

Ex) In case of 2048(0x0800) in S0_RX_WR,

0x4101_022C

0x0800

- Random number generator (RNG)

Introduction

RNG is a 32bit random number generator. RNG generates power on random number when power on reset. RNG can run/stop by software. RNG seed value and polynomial of RNG can be modified by software.

Features

- 32bit pseudo random number generator

- Formula of pseudo random number generator (polynomial) can be modified.

- Seed value of random generator can be modified.

- Support power on reset random value.

- Random value can be obtains by control start/stop by software.

Functional description

Figure 8 shows the RNG block diagram.

Figure 8. Random Number Generator block diagram

Operation RNG

Figure 9 shows the flowchart of RNG operation.

A random number is automatically generated after powering on reset,

Follow the procedure below to manually generate a random number.

- Change MODE to start/stop by register.

- Change clock source / seed value / polynomial value if need.

- Run and Stop the RNG.

- Read Random value.

Figure 9. Flow chart of RNG operation

Registers (Base address : 0x4000_7000)

RNG run register (RNG_RUN)

Address offset : 0x000

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | RUN |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] RUN – run RNG shift register

This bit written by S/W to run or stop RNG

0 : stop random number generator shift register

1 : run random number generator shift register

RNG SEED register (RNG_SEED)

Address offset : 0x004

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEED[31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| SEED[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] SEED – seed value of random number generator shift register

These bits written by S/W to set seed value of RNG before start(run) RNG shift register

RNG clock select register (RNG_CLKSEL)

Address offset : 0x008

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | CLKSEL |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[0] CLKSEL – select clock source register of RNG shift register

This bit written by S/W to select clock source of RNG shift register

0 : RNG clock (refer to clock generator block)

1 : PCLK

RNG manual mode select register (RNG_MODE)

Address offset : 0x00c

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | MODE |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[0] MODE – RNG run mode select register

This bit written by S/W to select which mode

0 : run/stop by PLL_LOCK signal (which is for power on random number)

1 : run/stop by RNG_RUN register (refer 1.4.1)R

RNG random number value register (RNG_RN)

Address offset : 0x010

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RN[31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| RN[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] RN – random number of RNG shift register

These bits are read only registers.

RNG polynomial register (RNG_POLY)

Address offset : 0x014

Reset value : 0xE000_0202

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| POLY[31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| POLY[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] POLY – 32bit polynomial of random number generator

These bits are written by S/W to modify the formula of random number generator

Default polynomial: F(x) = x31 + x30 + x29 + x9 + x

Register map

The following Table 8 summarizes the RNG registers.

Table 8  RNG register map and reset values

Alternate Function Controller (AFC)

Introduction

Each functional PADs have several functions.

Users can select a function in Alternate Function Controller block.

Features

Each functional pad has 2 ~ 4 functions.

Pads can be selected by each registers individually.

Each pad can be used as an external interrupt source.

Functional description

Table 9 shows the function table of each functional pad.

Table 9 functional description table

|  |  |  | function selection register value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 00 (reset value) | 01 | 10 | 11 |
| PAD NAME | Chip | PIN NUM | Normal Function | 2nd Function | 3rd Function | 4th Function |
| PA_00 | W7500 | 29 | GPIOA_0 |  | PWM6/CAP6 |  |
|  | W7500P | 27 |  |  |  |  |
| PA_01 | W7500 | 30 | GPIOA_1 |  | PWM7/CAP7 |  |
|  | W7500P | 28 |  |  |  |  |
| PA_02 | W7500 | 31 | GPIOA_2 |  | CLKOUT |  |
|  | W7500P | 29 |  |  |  |  |
| PA_03 | W7500 | 49 | SWCLK |  |  | PWM0/CAP0 |
|  | W7500P | 30 |  |  |  |  |
| PA_04 | W7500 | 50 | SWDIO |  |  | PWM1/CAP1 |
|  | W7500P | 31 |  |  |  |  |
| PA_05 | W7500 | 33 | SSEL0 |  | SCL1 | PWM2/CAP2 |
|  | W7500P | 19 |  |  |  |  |
| PA_06 | W7500 | 34 | SCLK0 |  | SDA1 | PWM3/CAP3 |
|  | W7500P | 20 |  |  |  |  |
| PA_07 | W7500 | 35 | MISO0 |  | U_CTS1 | PWM4/CAP4 |
|  | W7500P | 21 |  |  |  |  |
| PA_08 | W7500 | 36 | MOSI0 |  | U_RTS1 | PWM5/CAP5 |
|  | W7500P | 22 |  |  |  |  |
| PA_09 | W7500 | 37 | SCL0 |  | U_TXD1 | PWM6/CAP6 |
|  | W7500P | 23 |  |  |  |  |
| PA_10 | W7500 | 38 | SDA0 |  | U_RXD1 | PWM7/CAP7 |
|  | W7500P | 24 |  |  |  |  |
| PA_11 | W7500 | 40 | U_CTS0 |  | SSEL1 |  |
|  | W7500P | 41 |  |  |  |  |
| PA_12 | W7500 | 41 | U_RTS0 |  | SCLK1 |  |
|  | W7500P | 42 |  |  |  |  |
| PA_13 | W7500 | 42 | U_TXD0 |  | MISO1 |  |
|  | W7500P | 43 |  |  |  |  |
| PA_14 | W7500 | 43 | U_RXD0 |  | MOSI1 |  |
|  | W7500P | 44 |  |  |  |  |
| PA_15 | W7500 | 44 | GPIOA_15 | GPIOA_15 |  |  |
| PB_00 | W7500 | 45 | SSEL1 | GPIOB_0 | U_CTS0 |  |
|  | W7500P | 45 |  |  |  |  |
| PB_01 | W7500 | 46 | SCLK1 | GPIOB_1 | U_RTS0 |  |
|  | W7500P | 46 |  |  |  |  |
| PB_02 | W7500 | 47 | MISO1 | GPIOB_2 | U_TXD0 |  |
|  | W7500P | 47 |  |  |  |  |
| PB_03 | W7500 | 48 | MOSI1 | GPIOB_3 | U_RXD0 |  |
|  | W7500P | 48 |  |  |  |  |
| PB_04 | W7500 | 24 | TXEN | GPIOB_4 |  |  |
| PB_05 | W7500 | 25 | COL | GPIOB_5 |  |  |
| PB_06 | W7500 | 16 | RXD3 | GPIOB_6 |  |  |
|  | W7500P | 15 | DUP |  |  |  |
| PB_07 | W7500 | 17 | RXCLK | GPIOB_7 |  |  |
| PB_08 | W7500 | 18 | DUP | GPIOB_8 |  |  |
| PB_09 | W7500 | 19 | TXCLK | GPIOB_9 |  |  |
| PB_10 | W7500 | 20 | TXD0 | GPIOB_10 |  |  |
| PB_11 | W7500 | 21 | TXD1 | GPIOB_11 |  |  |
| PB_12 | W7500 | 22 | TXD2 | GPIOB_12 |  |  |
| PB_13 | W7500 | 23 | TXD3 | GPIOB_13 |  |  |
| PB_14 | W7500 | 26 | GPIOB_14 | GPIOB_14 |  |  |
| PB_15 | W7500 | 27 | GPIOB_15 | GPIOB_15 |  |  |
| PC_00 | W7500 | 53 | U_CTS1 | GPIOC_0 | PWM0/CAP0 |  |
|  | W7500P |  |  |  |  |  |
| PC_01 | W7500 | 54 | U_RTS1 | GPIOC_1 | PWM1/CAP1 |  |
|  | W7500P |  |  |  |  |  |
| PC_02 | W7500 | 55 | U_TXD1 | GPIOC_2 | PWM2/CAP2 |  |
|  | W7500P |  |  |  |  |  |
| PC_03 | W7500 | 56 | U_RXD1 | GPIOC_3 | PWM3/CAP3 |  |
|  | W7500P |  |  |  |  |  |
| PC_04 | W7500 | 57 | SCL1 | GPIOC_4 | PWM4/CAP4 |  |
|  | W7500P |  |  |  |  |  |
| PC_05 | W7500 | 58 | SDA1 | GPIOC_5 | PWM5/CAP5 |  |
|  | W7500P |  |  |  |  |  |
| PC_06 | W7500 | 51 | GPIOC_6 | GPIOC_6 | U_TXD2 |  |
|  | W7500P | 11 |  |  |  |  |
| PC_07 | W7500 | 52 | GPIOC_7 | GPIOC_7 | U_RXD2 |  |
| PC_08 | W7500 | 1 | PWM0/CAP0 | GPIOC_8 | SCL0 | AIN7 |
|  | W7500P |  |  |  |  |  |
| PC_09 | W7500 | 2 | PWM1/CAP1 | GPIOC_9 | SDA0 | AIN6 |
|  | W7500P |  |  |  |  |  |
| PC_10 | W7500 | 3 | U_TXD2 | GPIOC_10 | PWM2/CAP2 | AIN5 |
|  | W7500P |  |  |  |  |  |
| PC_11 | W7500 | 4 | U_RXD2 | GPIOC_11 | PWM3/CAP3 | AIN4 |
|  | W7500P |  |  |  |  |  |
| PC_12 | W7500 | 5 | AIN3 | GPIOC_12 | SSEL0 | AIN3 |
|  | W7500P |  |  |  |  |  |
| PC_13 | W7500 | 6 | AIN2 | GPIOC_13 | SCLK0 | AIN2 |
|  | W7500P |  |  |  |  |  |
| PC_14 | W7500 | 7 | AIN1 | GPIOC_14 | MISO0 | AIN1 |
|  | W7500P |  |  |  |  |  |
| PC_15 | W7500 | 8 | AIN0 | GPIOC_15 | MOSI0 | AIN0 |
|  | W7500P |  |  |  |  |  |
| PD_00 | W7500 | 11 | CRS | GPIOD_0 |  |  |
| PD_01 | W7500 | 12 | RXDV | GPIOD_1 |  |  |
| PD_02 | W7500 | 13 | RXD0 | GPIOD_2 |  |  |
| PD_03 | W7500 | 14 | RXD1 | GPIOD_3 |  |  |
| PD_04 | W7500 | 15 | RXD2 | GPIOD_4 |  |  |

Registers (Base address : 0x4100_2000)

PA_00 pad alternate function select register (PA_00_AFR)

Address offset : 0x000

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PA00AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PA00AF – PA_00 Pad alternate function selection register.

These bits are written by S/W.

00 : GPIOA_0

01 : GPIOA_0

10 : PWM6/CAP6

11 : None

PA_01 pad alternate function select register (PA_01_AFR)

Address offset : 0x004

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PA01AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PA01AF – PA_01 Pad function selection register.

These bits are written by S/W.

00 : GPIOA_1

01 : GPIOA_1

10 : PWM7/CAP7

11 : None

PA_02 pad alternate function select register (PA_02_AFR)

Address offset : 0x008

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PA02AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PA02AF – PA_02 Pad function selection register.

These bits are written by S/W.

00 : GPIOA_2

01 : GPIOA_2

10 : CLKOUT

11 : None

PA_03 pad alternate function select register (PA_03_AFR)

Address offset : 0x00c

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PA03AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PA03AF – PA_03 Pad function selection register.

These bits are written by S/W.

00 : SWCLK

01 : GPIOA_3

10 : None

11 : PWM0/CAP0

PA_04 pad alternate function select register (PA_04_AFR)

Address offset : 0x010

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PA04AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PA04AF – PA_04 Pad function selection register.

These bits are written by S/W.

00 : SWDIO

01 : GPIOA_4

10 : None

11 : PWM1/CAP1

PA_05 pad alternate function select register (PA_05_AFR)

Address offset : 0x014

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PA05AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PA05AF – PA_05 Pad function selection register.

These bits are written by S/W.

00 : SSEL0

01 : GPIOA_5

10 : SCL1

11 : PWM2/CAP2

PA_06 pad alternate function select register (PA_06_AFR)

Address offset : 0x018

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PA06AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PA06AF – PA_06 Pad function selection register.

These bits are written by S/W.

00 : SCLK0

01 : GPIOA_6

10 : SDA1

11 : PWM3/CAP3

PA_07 pad alternate function select register (PA_07_AFR)

Address offset : 0x01c

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PA07AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PA07AF – PA_07 Pad function selection register.

These bits are written by S/W.

00 : MISO0

01 : GPIOA_7

10 : CTS1

11 : PWM4/CAP4

PA_08 pad alternate function select register (PA_08_AFR)

Address offset : 0x020

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PA08AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PA08AF – PA_08 Pad function selection register.

These bits are written by S/W.

00 : MOSI0

01 : GPIOA_8

10 : RTS1

11 : PWM5/CAP5

PA_09 pad alternate function select register (PA_09_AFR)

Address offset : 0x024

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PA09AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PA09AF – PA_09 Pad function selection register.

These bits are written by S/W.

00 : SCL0

01 : GPIOA_9

10 : TXD1

11 : PWM6/CAP6

PA_10 pad alternate function select register (PA_10_AFR)

Address offset : 0x028

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PA10AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PA10AF – PA_10 Pad function selection register.

These bits are written by S/W.

00 : SDA0

01 : GPIOA_10

10 : RXD1

11 : PWM7/CAP7

PA_11 pad alternate function select register (PA_11_AFR)

Address offset : 0x02c

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PA11AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PA11AF – PA_11 Pad function selection register.

These bits are written by S/W.

00 : CTS0

01 : GPIOA_11

10 : SSEL1

11 : None

PA_12 pad alternate function select register (PA_12_AFR)

Address offset : 0x030

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PA12AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PA12AF – PA_12 Pad function selection register.

These bits are written by S/W.

00 : RTS0

01 : GPIOA_12

10 : SCLK1

11 : None

PA_13 pad alternate function select register (PA_13_AFR)

Address offset : 0x034

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PA13AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PA13AF – PA_13 Pad function selection register.

These bits are written by S/W.

00 : TXD0

01 : GPIOA_13

10 : MISO1

11 : None

PA_14 pad alternate function select register (PA_14_AFR)

Address offset : 0x038

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PA14AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PA14AF – PA_14 Pad function selection register.

These bits are written by S/W.

00 : RXD0

01 : GPIOA_14

10 : MOSI1

11 : None

PA_15 pad alternate function select register (PA_15_AFR)

Address offset : 0x03c

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PA15AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PA15AF – PA_15 Pad function selection register.

These bits are written by S/W.

00 : GPIOA_15

01 : GPIOA_15

10 : None

11 : None

PB_00 pad alternate function select register (PB_00_AFR)

Address offset : 0x040

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PB00AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PB00AF – PB_00 Pad function selection register.

These bits are written by S/W.

00 : SSEL1

01 : GPIOB_0

10 : CTS0

11 : None

PB_01 pad alternate function select register (PB_01_AFR)

Address offset : 0x044

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PB01AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PB01AF – PB_01 Pad function selection register.

These bits are written by S/W.

00 : SCLK1

01 : GPIOB_1

10 : RTS0

11 : None

PB_02 pad alternate function select register (PB_02_AFR)

Address offset : 0x048

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PB02AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PB02AF – PB_02 Pad function selection register.

These bits are written by S/W.

00 : MISO1

01 : GPIOB_2

10 : TXD0

11 : None

PB_03 pad alternate function select register (PB_03_AFR)

Address offset : 0x04c

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PB03AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PB03AF – PB_03 Pad function selection register.

These bits are written by S/W.

00 : MOSI1

01 : GPIOB_3

10 : RXD0

11 : None

PB_04 pad alternate function select register (PB_04_AFR)

Address offset : 0x050

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PB04AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PB04AF – PB_04 Pad function selection register.

These bits are written by S/W.

00 : TXEN

01 : GPIOB_4

10 : None

11 : None

PB_05 pad alternate function select register (PB_05_AFR)

Address offset : 0x054

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PB05AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PB05AF – PB_05 Pad function selection register.

These bits are written by S/W.

00 : COL

01 : GPIOB_5

10 : None

11 : None

PB_06 pad alternate function select register (PB_06_AFR)

Address offset : 0x058

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PB06AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PB06AF – PB_06 Pad function selection register.

These bits are written by S/W.

00 : RXD3

01 : GPIOB_6

10 : None

11 : None

PB_07 pad alternate function select register (PB_07_AFR)

Address offset : 0x05c

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PB07AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PB07AF – PB_07 Pad function selection register.

These bits are written by S/W.

00 : RXCLK

01 : GPIOB_7

10 : None

11 : None

PB_08 pad alternate function select register (PB_08_AFR)

Address offset : 0x060

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PB08AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PB08AF – PB_08 Pad function selection register.

These bits are written by S/W.

00 : DUP

01 : GPIOB_8

10 : None

11 : None

PB_09 pad alternate function select register (PB_09_AFR)

Address offset : 0x064

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PB09AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PB09AF – PB_09 Pad function selection register.

These bits are written by S/W.

00 : TXCLK

01 : GPIOB_9

10 : None

11 : None

PB_10 pad alternate function select register (PB_10_AFR)

Address offset : 0x068

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PB10AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PB10AF – PB_10 Pad function selection register.

These bits are written by S/W.

00 : TXD0

01 : GPIOB_10

10 : None

11 : None

PB_11 pad alternate function select register (PB_11_AFR)

Address offset : 0x06c

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PB11AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PB11AF – PB_11 Pad function selection register.

These bits are written by S/W.

00 : TXD1

01 : GPIOB_11

10 : None

11 : None

PB_12 pad alternate function select register (PB_12_AFR)

Address offset : 0x070

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PB12AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PB12AF – PB_12 Pad function selection register.

These bits are written by S/W.

00 : TXD2

01 : GPIOB_12

10 : None

11 : None

PB_13 pad alternate function select register (PB_13_AFR)

Address offset : 0x074

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PB13AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PB13AF – PB_13 Pad function selection register.

These bits are written by S/W.

00 : TXD3

01 : GPIOB_13

10 : None

11 : None

PB_14 pad alternate function select register (PB_14_AFR)

Address offset : 0x078

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PB14AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PB14AF – PB_14 Pad function selection register.

These bits are written by S/W.

00 : MDIO

01 : GPIOB_14

10 : None

11 : None

PB_15 pad alternate function select register (PB_15_AFR)

Address offset : 0x07c

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PB15AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PB15AF – PB_15 Pad function selection register.

These bits are written by S/W.

00 : MDC

01 : GPIOB_15

10 : None

11 : None

PC_00 pad alternate function select register (PC_00_AFR)

Address offset : 0x080

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PC00AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PC00AF – PC_00 Pad function selection register.

These bits are written by S/W.

00 : CTS1

01 : GPIOC_0

10 : PWM0/CAP0

11 : None

PC_01 pad alternate function select register (PC_01_AFR)

Address offset : 0x084

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PC01AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PC01AF – PC_01 Pad function selection register.

These bits are written by S/W.

00 : RTS1

01 : GPIOC_1

10 : PWM1/CAP1

11 : None

PC_02 pad alternate function select register (PC_02_AFR)

Address offset : 0x088

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PC02AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PC02AF – PC_02 Pad function selection register.

These bits are written by S/W.

00 : TXD1

01 : GPIOC_2

10 : PWM2/CAP2

11 : None

PC_03 pad alternate function select register (PC_03_AFR)

Address offset : 0x08c

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PC03AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PC03AF – PC_03 Pad function selection register.

These bits are written by S/W.

00 : RXD1

01 : GPIOC_3

10 : PWM3/CAP3

11 : None

PC_04 pad alternate function select register (PC_04_AFR)

Address offset : 0x090

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PC04AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PC04AF – PC_04 Pad function selection register.

These bits are written by S/W.

00 : SCL1

01 : GPIOC_4

10 : PWM4/CAP4

11 : None

PC_05 pad alternate function select register (PC_05_AFR)

Address offset : 0x094

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PC05AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PC05AF – PC_05 Pad function selection register.

These bits are written by S/W.

00 : SDA1

01 : GPIOC_5

10 : PWM5/CAP5

11 : None

PC_06 pad alternate function select register (PC_06_AFR)

Address offset : 0x098

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PC06AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PC06AF – PC_06 Pad function selection register.

These bits are written by S/W.

00 : GPIOC_6

01 : GPIOC_6

10 : TXD2

11 : None

PC_07 pad alternate function select register (PC_07_AFR)

Address offset : 0x09c

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PC07AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PC07AF – PC_07 Pad function selection register.

These bits are written by S/W.

00 : GPIOC_7

01 : GPIOC_7

10 : RXD2

11 : None

PC_08 pad alternate function select register (PC_08_AFR)

Address offset : 0x0a0

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PC08AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PC08AF – PC_08 Pad function selection register.

These bits are written by S/W.

00 : PWM0/CAP0

01 : GPIOC_8

10 : SCL0

11 : ADC_IN7

PC_09 pad alternate function select register (PC_09_AFR)

Address offset : 0x0a4

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PC09AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PC09AF – PC_09 Pad function selection register.

These bits are written by S/W.

00 : PWM1/CAP1

01 : GPIOC_9

10 : SDA0

11 : ADC_IN6

PC_10 pad alternate function select register (PC_10_AFR)

Address offset : 0x0a8

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PC10AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PC10AF – PC_10 Pad function selection register.

These bits are written by S/W.

00 : TXD2

01 : GPIOC_10

10 : PWM2/CAP2

11 : ADC_IN5

PC_11 pad alternate function select register (PC_11_AFR)

Address offset : 0x0ac

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PC11AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PC11AF – PC_11 Pad function selection register.

These bits are written by S/W.

00 : RXD2

01 : GPIOC_11

10 : PWM3/CAP3

11 : ADC_IN4

PC_12 pad alternate function select register (PC_12_AFR)

Address offset : 0x0b0

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PC12AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PC12AF – PC_12 Pad function selection register.

These bits are written by S/W.

00 : ADC_IN3

01 : GPIOC_12

10 : SSEL0

11 : ADC_IN3

PC_13 pad alternate function select register (PC_13_AFR)

Address offset : 0x0b4

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PC13AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PC13AF – PC_13 Pad function selection register.

These bits are written by S/W.

00 : ADC_IN2

01 : GPIOC_13

10 : SCLK0

11 : ADC_IN2

PC_14 pad alternate function select register (PC_14_AFR)

Address offset : 0x0b8

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PC14AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PC14AF – PC_14 Pad function selection register.

These bits are written by S/W.

00 : ADC_IN1

01 : GPIOC_14

10 : MISO0

11 : ADC_IN1

PC_15 pad alternate function select register (PC_15_AFR)

Address offset : 0x0bc

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PC15AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PC15AF – PC_15 Pad function selection register.

These bits are written by S/W.

00 : ADC_IN0

01 : GPIOC_15

10 : MOSI0

11 : ADC_IN0

PD_00 pad alternate function select register (PD_00_AFR)

Address offset : 0x0c0

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PD00AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PD00AF – PD_00 Pad function selection register.

These bits are written by S/W.

00 : CRS

01 : GPIOD_0

10 : None

11 : None

PD_01 pad alternate function select register (PD_01_AFR)

Address offset : 0x0c4

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PD01AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PD01AF – PD_01 Pad function selection register.

These bits are written by S/W.

00 : RXDV

01 : GPIOD_1

10 : None

11 : None

PD_02 pad alternate function select register (PD_02_AFR)

Address offset : 0x0c8

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PD02AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PD02AF – PD_02 Pad function selection register.

These bits are written by S/W.

00 : RXD0

01 : GPIOD_2

10 : None

11 : None

PD_03 pad alternate function select register (PD_03_AFR)

Address offset : 0x0cc

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PD03AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PD03AF – PD_03 Pad function selection register.

These bits are written by S/W.

00 : RXD1

01 : GPIOD_3

10 : None

11 : None

PD_04 pad alternate function select register (PD_04_AFR)

Address offset : 0x0d0

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PD04AF |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PD04AF – PD_04 Pad function selection register.

These bits are written by S/W.

00 : RXD2

01 : GPIOD_4

10 : None

11 : None

Register map

The following Table 10 summarizes the AFC registers.

Table 10 AFC register map and reset values

- External Interrupt (EXTI)

Introduction

Each functional pads are connected to the external interrupt(EXTINT) source.

Features

- All functional pads can be used as an external interrupt source regardless of any set of pad function.

- External Interrupt controller has the following functions and can be controlled by registers.

- Interrupt mask (enable or disable, default : disable)

- Interrupt polarity (rising or falling, default : rising)

Functional description

All pads are connected to the control register individually. (External interrupt mask register and External Interrupt polarity register)

External interrupt working as following expression:

- Each pad interrupt = Interrupt mask & (Interrupt polarity ^ Pad input)

- EXTINT = any Each pad interrupt

Figure 10 shows the External Interrupt diagram.

Figure 10. External Interrupt diagram

Registers (Base address : 0x4100_2000)

External interrupt enable register (Px_y EXTINT)

Address offset : 0x200

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  |  |  | 24 |  |  | 23 |  |  |  | 22 |  |  | 21 |  | 20 |  | 19 |  | 18 | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | Res |  | res |  | res |  |  |  | res |  |  | res |  |  |  | res |  |  | res |  | res |  | res |  | res | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  |  |  | 7 |  |  | 6 |  |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  |  |  | res |  |  | res |  |  | res |  | res |  | res |  | res |  | IEN |  |  | POL |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  | R/W |  |

[0] POL – External interrupt polarity selection register

These bits are written by S/W.

0 : interrupt occurs when pad detect HIGH level signal

1 : interrupt occurs when pad detect LOW level signal

[1] PA00IEN – External interrupt enable register

These bits are written by S/W.

0 : external interrupt disable

1 : external interrupt enable

Register map

The following Table 11 summarizes the EXTINT registers.

Table 11 EXTINT register map and reset values

- Pad Controller (PADCON)

Introduction

Pads of W7500x are controllable. User can control pad’s characteristic.

Features

- W7500x has digital I/O pads and digital/analog mux I/O pads

- Controllable characteristics of pads are pull-up, pull-down, driving strength, input enable, and CMOS/Schmitt trigger input buffer

- Each pad can be controlled individually by register.

Functional description

Figure 11 shows the function schematic of digital I/O pad of W7500x.

Figure 11. function schematic of digital I/O pad

Figure 12 shows the function schematic of digital/analog mux IO pad of W7500x

Figure 12. function schematic of digital/analog mux IO pad

Initials of Pad diagram is same as below.

P - PAD

YA – Analog Input (connect to ADC input)

Y – Digital Input

IE – Input buffer enable

| Condition |  | A | Y | P |
| --- | --- | --- | --- | --- |
| Input buffer enable (IE = 1) | Output mode | OUT | OUT | OUT |
|  | Input mode | No use | IN | IN |
| Input buffer disable (IE = 0) | Output mode | OUT | Low (0) | OUT |
|  | Input mode | No use | IN | IN |

CS – CMOS/Schmitt trigger input buffer select

PU – Pull-up enable

A – Digital Output

OD – Open Drain

DS – Driving strength select

| Condition |  | Rise/Fall Time (nSec) |  | Propagation Delay (nSec) |  |
| --- | --- | --- | --- | --- | --- |
| Driving Strength | Capacitance loading | Min | Max | Min | Max |
| Low (DS = 1) | 25pF | 4 | 18 | 7 | 27 |
|  | 100pF | 11 | 53 | 11 | 44 |
| High (DS = 0) | 25pF | 1 | 8 | 4 | 16 |
|  | 100pF | 4 | 23 | 7 | 24 |

PD – Pull-down enable

User can set pad condition with IE, CS, PU/PD, OD, DS by register.

And pads are can be controlled individually.

Registers (Base address : 0x4100_3000)

PAD Control register (Px_y PCR)(x=A..D, y=0..15)

Address offset : 0x000

Reset value : 0x0000_0060

| 31 | 30 | 29 | 28 | 27 | 26 |  |  | 25 |  | 24 |  | 23 | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 | 16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res |  |  | res |  | res |  | res | res |  | res |  | res |  | res |  | res |  | res | res |  |
| 15 | 14 | 13 | 12 | 11 | 10 |  |  | 9 |  | 8 |  | 7 | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 | 0 |  |
| res | res | res | res | res | res |  |  | Res |  | res |  | res | CS |  | IE |  | res |  | OD |  | DS |  | PUPD |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  | R/W |  |  |  | R/W |  | R/W |  | R/W |  |  |

[1:0] PUPD – Pull-up, Pull-down selection register

These bits are written by S/W.

00 : Neither

01 : pull-down

10 : pull-up

11 : Neither

[2] DS – Driving strength selection register

0 : High driving strength

1 : Low driving strength

[3] OD – Open Drain output mode register

0 : Open Drain output mode disable

1 : Open Drain output mode enable

[5] IE : Input buffer enable selection register

0 : Input buffer disable

1 : Input buffer enable

[6] CS – CMOS input or Summit trigger input selection register

0 : Summit trigger input buffer

1 : CMOS input buffer

Register map

The following Table 12 summarizes the PAD controller registers.

Table 12 PAD controller register map and reset values

- General-purpose I/Os(GPIO)

Introduction

The GPIO(General-Purpose I/O Port) is composed of three physical GPIO blocks, each corresponding to an individual GPIO port(PORT A, PORT B and PORT C). The GPIO supports up to 34 programmable input/output pins, depending on the peripherals being used.

Features

- The GPIO peripheral consists the following features.

- GPIO_DATAOUT can SET/CLEAR by the SET register and CLEAR register. (1 for set and 0 for clear)

- Mask registers allow treating sets of port bits as a group leaving other bits unchanged.

- Up to 34 GPIOs depending on configuration

- Programmable control for GPIO interrupts

- Interrupt generation masking

- Edge-triggered on rising, falling, or both

Functional description

Figure 13 shows the GPIO block diagram.

Figure 13. GPIO block diagram

Figure 14 shows the operation sequences available for the GPIO.

The pad alternate function is using the pad alternate function select register.

Refer to ‘Alternate Function Controller(AFC)’ for more details about each register.

The pad control supports pull-down, pull-up, input buffer, and summit trigger input buffer.

Refer to ‘Pad Controller (PADCON)’ for more details about each register.

Figure 14. GPIO Flow chart

Masked access

The masked access feature permits individual bits or multiple bits to be read from or written to in a single transfer. This avoids software-based read-modify-write operations that are not thread safe. With the masked access operations, the 16-bit I/O is divided into two halves, lower byte and upper byte. The bit mask address spaces are defined as two arrays each containing 256 words.

For example, to set bits[1:0] to 1 and clear bits[7:6] in a single operation, users can carry out the write to the lower byte mask access address space. The required bit mask is 0xC3, and users can write the operation as MASKLOWBYTE[0xC3] = 0x03. Refer to Figure 15 below.

Figure 15. MASK LOWBYTE access

To update some of the bits in the upper eight bits of the GPIO port, users can use the MASKHIGHBYTE array as Figure 16 below.

Figure 16 MASK HIGHBYTE access

GPIO Registers(Address Base: 0x4200_0000)

GPIO Data Register(GPIOx_DATA) (x=A..D)

Address offset: 0x0000

Reset value: 0x----

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| DAT15 | DAT14 |  | DAT13 |  | DAT12 |  | DAT11 |  | DAT10 |  | DAT9 |  | DAT8 |  | DAT7 |  | DAT6 |  | DAT5 |  | DAT4 |  | DAT3 |  | DAT2 |  | DAT1 |  | DAT0 |  |
| R | R |  | R |  | R |  | R |  | R |  | R |  | R |  | R |  | R |  | R |  | R |  | R |  | R |  | R |  | R |  |

[15:0] DATy(y = 0..15)

READ as :  Port input data bit

GPIO Output Latch Register(GPIOx_DATAOUT) (x=A..D)

Address offset: 0x0004

Reset value: 0x----

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| DAO15 | DAO14 | DAO13 | DAO12 | DAO11 | DAO10 | DAO9 | DAO8 | DAO7 | DAO6 | DAO5 | DAO4 | DAO3 | DAO2 | DAO1 | DAO0 |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

[15:0] DAOy(y = 0..15)

READ as :  Port out data bit

WRITE as :  WRITE to GPIOA_DATAOUT register

GPIO Enable Set Register(GPIOx_OUTENSET) (x=A..D)

Address offset: 0x0010

Reset value: 0x0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| ES15 | ES14 | ES13 | ES12 | ES11 | ES10 | ES9 | ES8 | ES7 | ES6 | ES5 | ES4 | ES3 | ES2 | ES1 | ES0 |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

[15:0] ESy(y = 0..15)

WRITE as :

‘0’ is no effect

‘1’ is sets the corresponding output enable bit

READ as :

‘0’ is indicates the signal direction as input

‘1’ is indicates the signal direction as output

GPIO Enable Clear Register(GPIOx_OUTENCLR) (x=A..D)

Address offset: 0x0014

Reset value: 0x0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| EC15 | EC14 | EC13 | EC12 | EC11 | EC10 | EC9 | EC8 | EC7 | EC6 | EC5 | EC4 | EC3 | EC2 | EC1 | EC0 |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

[15:0] ECy(y = 0..15)

WRITE as :

‘0’ is no effect

‘1’ is clears the output enable bit

READ as :

‘0’ is indicates the signal direction as input

‘1’ is indicates the signal direction as output

GPIO Interrupt Enable Set Register(GPIOx_ INTENSET) (x=A..D)

Address offset: 0x0020

Reset value: 0x0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| IES15 | IES14 | IES13 | IES12 | IES11 | IES10 | IES9 | IES8 | IES7 | IES6 | IES5 | IES4 | IES3 | IES2 | IES1 | IES0 |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

[15:0] IESy(y = 0..15)

WRITE as :

‘0’ is no effect

‘1’ is sets the interrupt enable bit

READ as :

‘0’ is indicates the interrupt disable

‘1’ is indicates the interrupt enable

GPIO Interrupt Enable Clear Register(GPIOx_ INTENCLR) (x=A..D)

Address offset: 0x0024

Reset value: 0x----

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| IEC15 | IEC14 | IEC13 | IEC12 | IEC11 | IEC10 | IEC9 | IEC8 | IEC7 | IEC6 | IEC5 | IEC4 | IEC3 | IEC2 | IEC1 | IEC0 |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

[15:0] IECy(y = 0..15)

WRITE as :

‘0’ is no effect

‘1’ is clears the interrupt enable bit

READ as :

‘0’ is indicates the interrupt disable

‘1’ is indicates the interrupt enable

GPIO Interrupt Type Set Register(GPIOx_ INTTYPESET) (x=A..D)

Address offset: 0x0028

Reset value: 0x----

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| ITS15 | ITS14 | ITS13 | ITS12 | ITS11 | ITS10 | ITS9 | ITS8 | ITS7 | ITS6 | ITS5 | ITS4 | ITS3 | ITS2 | ITS1 | ITS0 |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

[15:0] ITSy(y = 0..15)

WRITE as :

‘0’ is no effect

‘1’ is sets the interrupt type bit

READ as :

‘0’ is indicates for LOW or HIGH level

‘1’ is indicates for falling edge or rising edge

GPIO Interrupt Type Clear Register(GPIOx_ INTTYPECLR) (x=A..D)

Address offset: 0x002c

Reset value: 0x----

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| ITC15 | ITC14 | ITC13 | ITC12 | ITC11 | ITC10 | ITC9 | ITC8 | ITC7 | ITC6 | ITC5 | ITC4 | ITC3 | ITC2 | ITC1 | ITC0 |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

[15:0] ITCy(y = 0..15)

WRITE as :

‘0’ is no effect

‘1’ is clears the interrupt type bit

READ as :

‘0’ is indicates for LOW or HIGH level

‘1’ is indicates for falling edge or rising edge

GPIO Interrupt Polarity Set Register(GPIOx_ INTPOLSET) (x=A..D)

Address offset: 0x0030

Reset value: 0x----

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| IPS15 | IPS14 | IPS13 | IPS12 | IPS11 | IPS10 | IPS9 | IPS8 | IPS7 | IPS6 | IPS5 | IPS4 | IPS3 | IPS2 | IPS1 | IPS0 |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

[15:0] IPSy(y = 0..15)

WRITE as :

‘0’ is no effect

‘1’ is sets the interrupt polarity bit

READ as :

‘0’ is indicates for LOW level or falling edge

‘1’ is indicates for HIGH level or rising edge

GPIO Interrupt Polarity Clear Register(GPIOx_ INTPOLCLR) (x=A..D)

Address offset: 0x0034

Reset value: 0x----

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| IPC15 | IPC14 | IPC13 | IPC12 | IPC11 | IPC10 | IPC9 | IPC8 | IPC7 | IPC6 | IPC5 | IPC4 | IPC3 | IPC2 | IPC1 | IPC0 |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

[15:0] IPCy(y = 0..15)

WRITE as :

‘0’ is no effect

‘1’ is clears the interrupt polarity bit

READ as :

‘0’ is indicates for LOW level or falling edge

‘1’ is indicates for HIGH level or rising edge

GPIO Interrupt Status/Interrupt Clear Register(GPIO_ INTSTATUS/ INTCLEAR) (x=A..D)

Address offset: 0x0038

Reset value: 0x----

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| ISC15 | ISC14 | ISC13 | ISC12 | ISC11 | ISC10 | ISC9 | ISC8 | ISC7 | ISC6 | ISC5 | ISC4 | ISC3 | ISC2 | ISC1 | ISC0 |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

[15:0] ISCy(y = 0..15)

WRITE as :

‘0’ is no effect

‘1’ is request to clear the interrupt

READ as :  IRQ status Register

GPIO Lower Byte Masked Access Register(GPIOx_ LB_MASKED) (x=A..D)

Address offset: 0x0400 – 0x07FC

Reset value: 0x----

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | LBM |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

Lower eight bits masked access

[15:8] Not used

[7:0] Data for lower byte access, with bits[9:2] of address value used as enable bit mask for each bit

GPIO Upper Byte Masked Access Register(GPIOx_ UB_MASKED) (x=A..D)

Address offset: 0x0800-0x0FC

Reset value: 0x----

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| HBM |  |  |  |  |  |  |  | res | res | res | res | res | res | res | res |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

Higher eight bits masked access

[15:8] Data for higher byte access, with bits[9:2] of address value used as enable bit mask for each bit

[7:0] Not used

Register map

The following Table 13 summarizes the GPIO registers.

Table 13 GPIO register map and reset values

- Direct memory access controller (DMA)

Introduction

Direct memory access (DMA) is used in order to provide high-speed data transfer between peripherals and memory as well as memory to memory. Data can be quickly moved by DMA without any CPU actions. This keeps CPU resources free for other operations.

The DMA controller has up to 6 channels in total, each dedicated to managing memory access requests from one or more peripherals. It has an arbiter for handling the priority between DMA requests. For more details, refer to “PrimeCell® μDMA Controller (PL230)” from the Technical Reference Manual

Features

•   6 channels

•   Each channel is connected to dedicated hardware DMA requests and software trigger is also supported on each channel.

•   Priorities between requests from the DMA channels are software programmable (2 levels consisting of high, default)

•   Memory-to-memory transfer (software request only)

•   TCP/IP-to-memory transfer (software request only)

•   SPI/UART-to-memory transfer (hardware request and software request)

•   Access to Flash, SRAM, APB and AHB peripherals as source and destination

Functional description

Figure 17 shows the DMA block diagram.

Figure 17. DMA Block diagram

DMA request mapping

The hardware requests from the peripherals (UART0, UART1, SSP0, SSP1) are simply connected to the DMA. Refer to Table 17 which lists the DMA requests for each channel.

Table 14 Summary of the DMA requests for each channel

|  | Channel 1 | Channel 2 | Channel 3 | Channel 4 | Channel 5 | Channel 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Hardware Request | SSP0_TX SSP0_RX | SSP1_TX SSP1_RX | UART0_TX UART0_RX | UART1_TX UART1_RX | NONE | NONE |
| Software Request(1) | Support | Support | Support | Support | Support | Support |

- Software request is the only way to use DMA for memory-to-memory or TCP/IP-to-memory.

DMA arbitration

The controller can be configured to perform arbitration during a DMA cycle before and after a programmable number of transfers. This reduces the latency for servicing a higher priority channel.

The controller uses four bits in the channel control data structure that configures how many AHB bus transfers occur before the controller re-arbitrates. These bits are known as the R_power bits because the value R is raised to the power of two and this determines the arbitration rate. For example, if R = 4, then the arbitration rate is 24, which means the controller arbitrates every 16 DMA transfers.

Remark: Do not assign a low-priority channel with a large R_power value because this prevents the controller from servicing high-priority requests until it re-arbitrates.

When N > 2R and is not an integer multiple of 2R, then the controller always performs sequences of 2Rtransfers until N < 2R remain to be transferred. The controller performs the remaining N transfers at the end of the DMA cycle.

DMA cycle types

The cycle_ctrl bits in the channel control data structure controls how the DMA controller performs a cycle.

The controller uses four cycle types described in this manual:

- Invalid

- Basic

- Auto-request

- Ping-pong

See ARM micro DMA (PL230) documentation for additional cycle types.

For all cycle types, the controller arbitrates after 2R DMA transfers. If a low-priority channel is set to a large 2R value then it prevents all other channels from performing a DMA transfer until the low-priority DMA transfer completes. Therefore, the user must take care when setting the R_power bit in the channel_cfg data structure, that the latency for high-priority channels is not significantly increased.

Invalid cycle

After the controller completes a DMA cycle, it sets the cycle type to invalid to prevent it from repeating the same DMA cycle.

Basic cycle

In this mode, the controller can be configured to use either the primary or the alternate channel control data structure. After the channel is enabled and the controller receives a request for this channel, the flow for basic cycle is as below:

- The controller performs 2R transfers.

If the number of transfers remaining is zero the flow continues at step 3.

- The controller arbitrates:

- If a higher-priority channel is requesting service, then the controller services that channel.

- If the peripheral or software signals a request to the controller, then it continues at step 1.

- The controller sets dma_done[c] signal for this channel HIGH for one system clock cycle. This indicates to the host processor that the DMA cycle is complete.

Auto-request cycle

When the controller operates in this mode, it is only necessary to receive a single request to enable the controller to complete the entire DMA cycle. This enables a large data transfer to occur, without significantly increasing the latency for servicing higher priority requests or requiring multiple requests from the processor or peripheral.

The auto-request cycle is typically used for memory-to-memory requests. In this case, software generates the starting request for the 2R transfers after setting up the DMA control data structure.

In this mode, the controller can be configured to use either the primary or the alternate channel control data structure. After the channel is enabled and the controller receives a request for this channel, the flow for the auto-request cycle is as below:

- The controller performs 2R transfers. If the number of transfers remaining is zero the flow continues at step 3.

- The controller arbitrates if there are any transfers remaining after 2R transfers. If the current channel c has the highest priority, the cycle continues at step 1.

The controller sets dma_done[c] signal for this channel HIGH for one system clock cycle. This indicates to the host processor that the DMA cycle is complete.

Ping-pong cycle

In this mode, the controller performs a DMA cycle using one of the data structures and then performs a DMA cycle using the other data structure. The controller continues to switch between primary and alternate structures until it reads a data structure that is invalid, until the user reprograms the cycle_type to basic, or until the host processor disables the channel.

In ping-pong mode, the user can program or reprogram one of the two channel data structures (primary or alternate) while using the other channel data structure for the active transfer. When a transfer is done, the next transfer can be started immediately using the prepared channel data structure – provided that a higher priority channel does not require servicing. If the user does not reprogram the channel control data structure not in use for a transfer, the cycle type remains invalid (which is the value at the end of the last transfer using that structure), and the ping-pong cycle completes.

The ping-pong cycle can be used for transfers to or from peripherals or for memory- to-memory transfers.

Figure 18. DMA ping pong cycle

Registers (Base address : 0x4100_4000)

DMA status register (DMA_STATUS)

Address offset : 0x000

Reset value : 0x0005_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 |  | 1 |  | 0 |  |
| res | res | res | res | res | res | res | res | STATE |  |  |  | res | res |  | res |  | ENABLE |  |
|  |  |  |  |  |  |  |  | RO |  |  |  |  |  |  |  |  | RO |  |

[0] ENABLE – Enable status of the controller

This bit is read only register to check enable status of DMA controller

0 : controller is disabled

1 : controller is enabled

[7:4] STATE – Current state of the control state machine.

These bits are read only register to check current state of controller. State can be one of the following

0000 : idle

0001 : reading channel controller data

0010 : reading source data end pointer

0011 : reading destination data end pointer

0100 : reading source data

0101 : writing destination data

0110 : waiting channel controller data

1000 : stalled

1001 : done

1010 : peripheral scatter-gather transition

1011 – 1111 : undefined.

DMA configuration register (DMA_CFG)

Address offset : 0x004

Reset value : -

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | PROT_CTRL |  |  |  |  |  | res |  | res |  | res |  | res |  | ENABLE |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | WO |  |  |  |  |  |  |  |  |  |  |  |  |  | WO |  |

[0] ENABLE – Enable for the controller

This bit is write only register to enable of DMA controller

0 : disable the controller

1 : enable the controller

[7:5] PROT_CTRL – Set the AHB-Lite protection by controlling the HPROT[3:1] signal levels

These bits are write only register to set HPROT[3:1] signal as follows

[7] : controls HPROT[3] to indicate if a cacheable access is occurring.

[6] : controls HPROT[2] to indicate if a bufferable access is occurring.

[5] : controls HPROT[1] to indicate if a privileged access is occurring.

DMA control data base pointer register (DMA_CTRL_BASE_PTR)

Address offset : 0x008

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CTRL_BASE_PTR[31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  | 0 |
| CTRL_BASE_PTR[15:8] |  |  |  |  |  |  |  | res | res | res | res | res | res | res |  | res |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:8] CTRL_BASE_PTR – Pointer to the base address of the primary data structure

These bits are read/write register. User must configure this register so that the base pointer points to a location in system memory.

DMA channel alternate control data base pointer register (DMA_ALT_CTRL_BASE_PTR)

Address offset : 0x00c

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ALT_CTRL_BASE_PTR[31:16] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  | 0 |
| ALT_CTRL_BASE_PTR[15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] ALT_CTRL_BASE_PTR : Base address of the alternate data structure

This read only register returns the base address of the alternate data structure.

DMA channel wait on request status register (DMA_WAITONREQ_STATUS)

Address offset : 0x010

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | DMA_WAITONREQ[5:0] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | RO |  |  |  |  |  |  |

[Channel-1] DMA_WAITONREQ – Channel wait on request status

This read-only register returns the status of dma_waitonreq[Channel-1].

0 : dma_waitonreq is low

1 : dma_waitonreq is high

DMA channel software request register (DMA_CHNL_SW_REQUEST)

Address offset : 0x014

Reset value : -

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | CHNL_SW_REQUEST[5:0] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | WO |  |  |  |  |  |  |

[Channel-1] CHNL_SW_REQUEST – Set the appropriate bit to generate a software DMA request on the corresponding DMA channel

This read-only register enables to generate a software DMA request.

0 : dose not create a DMA request for [Channel-1]

1 : creates a DMA request for [Channel -1]

DMA channel useburst set register (DMA_CHNL_USEBURST_SET)

Address offset : 0x018

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | CHNL_USEBURST_SET[5:0] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | RW |  |  |  |  |  |  |

[Channel-1] CHNL_USEBURST_SET – Returns the useburst status, or disable dma_sreq[Channel-1] form generating DMA requests.

This read/write register disables the single request dma_sreq[Channel-1] input from generating requests, and therefore only the request, dma_req[Channel-1],generates requests. Reading the register returns the useburst status.

Read as :

0 : DMA [Channel-1] responds to requests that it receives on

dma_req[Channel-1] or dma_sreq[Channel-1]

1 : DMA [Channel-1] does not responds to requests that it receives on

dma_sreq[Channel-1]. The controller only responds to dma_req[Channel-1] requests

Write as :

0 : No effect. Use the CHNL_USEBURST_CLR register to set bit [Channel-1]

to 0

1 : Disables dma_sreq[Channel-1] form generating DMA requests.

DMA channel useburst clear register (DMA_CHNL_USEBURST_CLR)

Address offset : 0x01c

Reset value : -

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | CHNL_USEBURST_CLR[5:0] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | WO |  |  |  |  |  |  |

[Channel-1] CHNL_USEBURST_CLR - Set the appropriate bit to enable dma_sreq[Channel-1] to generate requests.

This Write register enables dma_sreq[Channel-1] to generate requests.

0 – No effect. User the CHNL_USEBURST_SET register to disable dma_sreq[Channel-1] from generating requests.

1 – Enable dma_sreq[Channel-1] to generate DMA requests.

DMA channel request mask set register (DMA_CHNL_REQ_MASK_SET)

Address offset : 0x020

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | CHNL_REQ_MASK_SET[5:0] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |  |

[Channel-1] CHNL_REQ_MASK_SET - Returns the request mask status of dma_req[Channel-1] and dma_sreq[Channel-1], or disables the corresponding channel from generating DMA requests.

This read/write register disables a HIGH on dma_req[Channel-1], for dma_sreq[Channel-1], from generating a request. Reading the register returns the request mask status for dma_req[Channel-1] and dma_sreq[Channel-1].

Read as :

0 – External requests are enabled for channel [Channel-1]

1 – External requests are disabled for channel [Channel-1]

Write as :

0 – No effect. Use the CHNL_REQ_MASK_CLR register to enable DMA requests.

1 – Disables dma_req[Channel-1] and dma_sreq[Channel-1] from generating  DMA requests.

DMA channel request mask clear register (DMA_CHNL_REQ_MASK_CLR)

Address offset : 0x024

Reset value : -

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | CHNL_REQ_MASK_CLR[5:0] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | WO |  |  |  |  |  |  |

[Channel-1] CHNL_REQ_MASK_CLR – Set the appropriate bit to enable DMA requests for the channel corresponding to dma_req[Channel-1] and dma_sreq[Channel-1]

This write only register enables a HIGH on dma_req[Channel-1], or dma_sreq[Channel-1].

0 : No effect. Use the CHNL_REQ_MASK_SET register to disable dma_req[Channel-1] and dma_sreq[Channel-1] from generating requests.

1 : Enables dma_req[Channel-1] or dma_sreq[Channel] to generate DMA requests.

DMA channel enable set register (DMA_CHNL_ENABLE_SET)

Address offset : 0x028

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | CHNL_ENABLE_SET[5:0] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |  |

[Channel-1] CHNL_ENABLE_SET - Returns the enable status of channels or enables the corresponding channels.

This read/write register enables a DMA channel. Reading the register returns the enable status of the channels.

Read as :

0 – Channel [Channel-1] is disabled

1 – Channel [Channel-1] is enabled

Write as :

0 – No effect. Use the CHNL_ENABLE_CLR register to disable a channel.

1 – Enables channels [Channel-1]

DMA channel enable clear register (DMA_CHNL_ENABLE_CLR)

Address offset : 0x02c

Reset value : -

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | CHNL_ENABLE_CLR[5:0] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | WO |  |  |  |  |  |  |

[Channel-1] CHNL_ENABLE_CLR – Set the appropriate bit to disable the corresponding DMA channel.

This write only register disable a DMA channel.

0 : No effect. Use the CHNL_ENABLE_SET register to enable DMA channel.

1 : Disable channel [Channel-1]

DMA channel primary-alternate set register (DMA_CHNL_PRI_ALT_SET)

Address offset : 0x030

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | CHNL_PRI_ALT_SET[5:0] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |  |

[Channel-1] CHNL_PRI_ALT_SET - Returns the channel control data structure status, or selects the alternate data structure for the corresponding DMA channels.

This read/write register configure a DMA channel to use the alternate data structure. Reading the register returns the status of which data structure is in use for the corresponding DMA channel.

Read as :

0 – DMA Channel [Channel-1] is using the primary data structure.

1 – DMA Channel [Channel-1] is using the alternate data structure.

Write as :

0 – No effect. Use the CHNL_PRI_ALT _CLR register to set bit [Channel-1] to 0

1 – Selects the alternate data structure for channel [Channel-1]

DMA channel primary-alternate clear register (DMA_CHNL_PRI_ALT _CLR)

Address offset : 0x034

Reset value : -

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | CHNL_PRI_ALT_CLR[5:0] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | WO |  |  |  |  |  |  |

[Channel-1] CHNL_PRI_ALT_CLR – Set the appropriate bit to select the primary data structure for the corresponding DMA channel.

This write only register configures a DMA channels to use the primary data structure.

0 : No effect. Use the CHNL_PRI_ALT _SET register to select the alternate data structure.

1 : Selects the primary data structure for channel [Channel-1]

DMA channel priority set register (DMA_CHNL_PRIORITY_SET)

Address offset : 0x038

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | CHNL_PRIORITY_SET [5:0] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |  |

[Channel-1] CHNL_PRIORITY_SET - Returns the channel priority mask status, or set the channel priority to high.

This read/write register configure a DMA channel to use the high priority level. Reading the register returns the status of channel priority mask

Read as :

0 – DMA Channel [Channel-1] is using the default priority level.

1 – DMA Channel [Channel-1] is using a high priority level.

Write as :

0 – No effect. Use the CHNL_PRIORITY_CLR register to set bit [Channel-1] to default priority level

1 – Channel [Channel-1] uses the high priority level.

DMA channel priority clear register (DMA_CHNL_PRIORITY_CLR)

Address offset : 0x03C

Reset value : -

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | CHNL_PRIORITY_CLR[5:0] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | WO |  |  |  |  |  |  |

[Channel-1] CHNL_PRIORITY_CLR – Set the appropriate bit to select the default priority level for the specified DMA channel.

This write only register configures a DMA channels to use the default priority level.

0 : No effect. Use the CHNL_PRIORITY_SET register to set channel [Channel-1] to the high priority level.

1 : channel [Channel-1] uses the default priority level.

DMA bus error clear register (DMA_ERR_CLR)

Address offset : 0x04C

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | ERR_CLR |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |

[0] ERR_CLR – Returns the status of DMA_ERR, or set the signal LOW.

This read/write register returns the status of DMA_ERR, and enables set DMA_ERR LOW.

Read as :

0 : DMA_ERR is LOW

1 : DMA_ERR is HIGH

Write as :

0 : No effect, status of DMA_ERR is unchanged.

1 : Sets DMA_ERR LOW.

Register map

The following Table 15 summarizes the DMA registers.

Table 15 DMA register map and reset values

- Analog-to-digital converter (ADC)

Introduction

ADC is a 12bit analog-to-digital converter. It has up to 9 multiplexed channels allowing it to measure signals from 8 externals and 1 internal source.

ADC of various channels can be performed in single mode. The result of the ADC is stored in 12 bit register.

Features

- 12bit configuration resolution

- Conversion time : Max 10MHz (Sampling time can be programmable)

8 channel for external analog inputs

  - CH0 : PC_15

  - CH1 : PC_14

  - CH2 : PC_13

  - CH3 : PC_12

  - CH4 : PC_11

  - CH5 : PC_10

  - CH6 : PC_09

  - CH7 : PC_08

1 channel for internal LDO(1.5v) voltage.

  - CH15 : Internal voltage

- Start of conversion can be initiated by software.

- Convert selected inputs once per trigger.

- Interrupt generation at the end of conversion.

Functional description

Figure 19 shows the ADC block diagram.

Figure 19. ADC block diagram

Operation ADC with non-interrupt

Figure 20 shows the flowchart of ADC operation with non-interrupt.

ADC can be used as below:

- ADC needs to be initialized before operation.

To initialize the ADC, clear the PWD bit first.

- Select the ADC channel from 0 to 7 and 15 (initial core voltage).

- Run start ADC conversion by set ADC_SRT bit.

- Check INT bit to know finish of conversion.

- If INT bit is high (1), read ADC conversion data.

- Finally, ADC operation is finished by setting the PWD bit.

Figure 20. The ADC operation flowchart with non-interrupt

Operation ADC with interrupt

Figure 21 shows the flowchart of ADC operation with interrupt.

Operation is almost the same as the non-interrupt mode except checking INT register bit to know when enabling interrupt mask bit and conversion is completed.

Figure 21. The ADC operation flowchart with interrupt

Registers (Base address : 0x4100_0000)

ADC control register (ADC_CTR)

Address offset : 0x000

Reset value : 0x0000_0003

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | PWD |  | SMPSEL |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  | R/W |  |

[0] SMPSEL – Sampling mode select

This bit written by S/W to select sampling mode

0 : Abnormal mode

1 : Normal mode

[1] PWD – Power down

This bit set and cleared by S/W to enable/disable power down mode

O : Active

1 : Power down

ADC channel select register (ADC_CHSEL)

Address offset : 0x004

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | CHSEL |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |

[3:0] CHSEL – ADC Channel Select

These bits are written by S/W to define which channel is selected to be converted.

0000 : Channel 0 select

0001 : Channel 1 select

0010 : Channel 2 select

0011 : Channel 3 select

0100 : Channel 4 select

0101 : Channel 5 select

0110 : Channel 6 select

0111 : Channel 7 select

1000 ~ 1000 : no select

1111 : LDO output(1.5V) select

ADC start register (ADC_START)

Address offset : 0x008

Reset value : 0x0000_0000

| 31 |  |  | 30 |  |  | 29 |  |  | 28 |  |  | 27 |  |  | 26 |  |  | 25 |  |  | 24 |  |  | 23 |  |  | 22 |  |  | 21 |  |  | 20 |  |  | 19 |  |  | 18 |  |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  | res |
| 15 | 14 |  |  | 13 |  |  | 12 |  |  | 11 |  |  | 10 |  |  | 9 |  |  | 8 |  |  | 7 |  |  | 6 |  |  | 5 |  |  | 4 |  |  | 3 |  |  | 2 |  |  | 1 |  |  | 0 |  |
| res | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | res |  |  | ADC_SRT |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | W |  |

[0] ADC_SRT – ADC Start bit

This bit set by S/W to start ADC for conversion. This bit is write-only.

0 : ready to start

1 : start ADC for conversion (This bit clear automatically after conversion)

ADC conversion data register (ADC_DATA)

Address offset : 0x00C

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | ADC_DATA |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | R |  |  |  |  |  |  |  |  |  |  |  |

[11:0] ADC_DATA – ADC conversion result data

It contains ADC conversion result data of last converted channel. These bits are read-only.

ADC Interrupt register (ADC_INT)

Address offset : 0x010

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | MASK |  | INT |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  | R |  |

[0] DONE – Interrupt bit

This bit indicates that conversion is done or not. This bit is set after conversion is done and this bit is cleared by set of Interrupt clear bit. This bit is read-only.

[1] MASK – Interrupt mask signal.

This bit is interrupt mask bit of ADC. This bit can be set and cleared by S/W to enable/disable interrupt mask.

0 : Interrupt disable

1 : Interrupt enable

ADC Interrupt Clear register (ADC_INTCLR)

Address offset : 0x01c

Reset value : 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | INTCLR |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | W |  |

[0] INTCLR – Interrupt Clear bit.

This bit set by S/W to clear interrupt signal to CM0. This bit is write-only.

0 – nothing

1 – Clear interrupt signal (This bit clear automatically after clear interrupt)

Register map

The following Table 16 summarizes the ADC registers.

Table 16 ADC register map and reset values

- Pulse-Width Modulation (PWM)

Introduction

The PWM consists a 8-channel 32-bit Timer/Counter driven by a programmable prescaler. The function of the PWM is based on the basic Timer. Each timer and counter runs independently.

The PWM can be used to control the width of the pulse, formally the pulse duration, to generate output waveform or to count the counter triggered by external input.

Features

- Counter or Timer operation can use the peripheral clock, external clock source, or one of the capture inputs as the clock source.

- Eight independent 32-bit Timer/Counter driven by a programmable 6 bits prescaler runs as the PWM or standard timer if the PWM mode is not enabled.

- Eight PWM output waveforms.

- Each of Timer/Counter can have different or same clock source.

- Counter or timer operation.

- Eight capture registers that can take the timer value when an external input signal. A capture event can generate an interrupt signal optionally.

- 32-bit match register and limit register.

Figure 22 PWM block diagram

Functional description

Timer/Counter control

The PWM has Start/Stop register. It controls start or stop of the Timer/Counter. If you set this register, the Timer/Counter starts to run. If you reset this register, the Timer/Counter stops immediately. Also there is a pause register. The pause register is used to stop temporarily after one period. Although you set this register while the Timer/Counter is running, the Timer/Counter will stop when the period ends.

The registers of PWM can be updated when it stops or pauses. Users cannot update the registers while PWM is running.

Timer/Counter

The PWM has 8 Timer/Counter clocks, which can be divided by a prescaler. Each Timer/Counter runs independently. The Timer/Counter is designed to count cycles of the clocks or external input signal and generate interrupts when specified timer values are occurred based on match register and limit register. The Timer/Counter can count up or down.

The PWM has match registers and limit registers. The match registers control the duty cycle of PWM output waveform. The limit registers control the period of the PWM output waveform. The Timer/Counter becomes 0 when it reaches value of the limit register. If PDMR(Periodic Mode Register) is set, the Timer/Counter counts stops counting and if PDMR is reset, the Timer/Counter counts repeatedly.

Match register should be smaller than limit register(LR). If not, match interrupt is not occurred and PWM output waveform is always 1.

Repetition mode

The Timer/Counter has two repetition mode: periodic and one-shot mode. In periodic mode, the Timer/Counter recycles and then restarts when the Timer/Counter reaches the value of limit register. Figure 23 shows periodic mode timing diagram.

Figure 23 Periodic mode

In one-shot mode, the Timer /Counter reset to the initial value and then stops when the Timer/Counter reaches the value of limit register. Figure 24 shows one-shot mode timing diagram.

Figure 24 one-shot mode

Counting mode

The Timer/Counter has two counting mode: Up-count and Down-count mode. In up-count mode, the Timer/Counter counts up from 0 to the limit register value, then recycles. If repetition mode is periodic, the Timer/Counter restarts, if repetition mode is one-shot mode, the Timer/Counter stops. Figure 25 shows up-count mode timing diagram.

Figure 25 Up-count mode

In Down-count mode, the Timer/Counter counts from 0xFFFF_FFFF, then recycles. If repetition mode is periodic, the Timer/Counter restarts, if repetition mode is one-shot mode, the Timer/Counter stops. Figure 26 shows down-count mode timing diagram.

Figure 26 Down-count mode

Timer and Counter mode

The Timer/Counter can run in timer mode or counter mode. In timer mode, the Timer/Counter is counted by PWMCLK after Prescale counter is overflowed. If prescale is set by 0, the Timer/Counter counts every PWMCLK period. In counter mode, the Timer/Counter is counted by external input signal. There are three counting method: rising edge, falling edge, and both edge. The counter mode has up-count or down-count mode and also has periodic or one-shot mode. The external input pin and PWM output pin are the same, so PWM output is disabled in counter mode.

Figure 27 is counter mode example with rising edge mode,

Figure 28 is with falling edge mode and

Figure 29 is with both rising and falling edge mode.

Figure 27 Counter mode with rising edge

Figure 28 Counter mode with falling edge

Figure 29 Counter mode with rising and falling edge

Prescaler description

The PWM has 6-bit prescale counter(PC) and the prescaler can divide the Timer/Counter clock frequency. Users can control it by Prescale Register(PR).

Figure 30 and Figure 31 shows some examples of the Timer/Counter timing with prescale register is 2, match register is 2, limit register is 12, timer mode, periodic mode, up-count mode, and no interrupt clear.

Figure 30 Timer/Counter timing diagram with match interrupt

Figure 31 Timer/Counter timing diagram with overflow interrupt

PWM mode

Pulse Width Modulation mode generates a waveform with a period determined by the value of limit register and a duty cycle determined by the value of the match register.

The PWM output becomes always 1 when the Timer/Counter starts to count. Then the PWM output becomes 0 when the Timer/Counter reaches the value of match register. If the Timer/Counter is in periodic mode, the PWM output becomes 1 again when the Timer/Counter reaches the value of limit register. In one-shot mode, the PWM output does not change to 1 but stays 0 and the Timer/Counter stops.

The PWM mode can be selected independently on each channel(0~7) by PWM output enable and external input enable register. The external input pin and PWM output pin are the same, so external input is disabled in PWM mode.

Figure 32 is an example of the PWM output waveform when the Timer/Counter is reached to the value of match register.

Figure 33 is example of the PWM output waveform when to the Timer/Counter is reached to the value of limit register.

Figure 32 The PWM output up to match register

Figure 33 The PWM output up to limit register

If match register is set as 0, the PWM output will be 1 while the Timer/Counter is 0.

If the match register is bigger than the limit register, the PWM output is always 1.

Interrupt

The PWM has 8-bit interrupt enable register(IER) and each bit of IER corresponds to each interrupt of channel. Each PWM channel has Channel-n Interrupt Enable register(CHn_IER). The CHn_IER includes three types of interrupt: match, overflow, and capture. The match interrupt occurs when the Timer/Counter is reached to value of match register. The overflow interrupt occurs when the Timer/Counter is reached to value of limit register. The capture interrupt occurs when external input is entered for capture.

If interrupt occurs, corresponded bit of Channel-x interrupt register(CHn_IR) bit is set and PWM channel-n interrupt signal is generated. All CHn_IR is cleared by channel-n interrupt clear register(CHn_ICR) and then PWM channel-n interrupt signal is cleared.

Dead zone generation

Each PWM channel can output two complementary signals with dead zone time and it can be enabled by Channel-n Dead Zone Enable Register(CHn_DZER). Only 4 channels can be enabled because there are 8 PWM output pins. Channel 0 and 1 are a pair, channel 2 and 3 are a pair, channel 4 and 5 are a pair, and channel 6 and 7 are a pair. If users want to use channel-0 dead zone generation, channel-1 should be disabled. If channel 0 and 1 dead zone generation are enabled both, all outputs are 0. In that case, users should choose 1 channel.

Dead zone time are generated by the value of Channel-n Dead Zone Counter Register(DZCR). The dead zone counter counts up to value of DZCR. During the dead zone time, both complementary signals are both 0. Users have to adjust the signal depending on the devices that are connected to the outputs and their characteristics. If DZCR is bigger than the limit register, main output signal is toggled 0 to1 and then 1 to 0 while 1 PWMCLK and inverted output signal is always 0.

Figure 34 shows two complementary PWM outputs with dead zone time. During dead zone time, both outputs are 0. Figure 35 shows a more detailed timing with dead zone counter. The dead zone counter and the Timer/Counter starts to count together and PWM output is 0 until dead zone counter is reached to value of dead zone counter register. The PWM output becomes 1 and 0 when the Timer/Counter is reached to value of match register. The inverted PWM output is also 0 until dead zone counter is reached to value of dead zone counter register. Then inverted PWM output becomes 1 after dead zone counter is reached to the value of dead zone counter register.

Figure 34 PWM waveform with dead zone time

Figure 35 PWM waveform with dead zone counter

Capture event

Each PWM channel can capture its Timer/Counter value when an external input signal changes. Any channel could use any method of rising or falling edges. If capture interrupt is enabled, capture interrupt occurs when the external input signal is toggled. The Timer/Counter value is saved in Channel-n Capture Register(CHn_CR) and the capture register is not overwritten until capture interrupt is cleared. Figure 36 shows the capture event timing diagram. There is no interrupt clear, so second capture does not save during second rising edge detection.

Figure 36 Capture event with no interrupt clear

Figure 37 shows, also, capture event timing diagram with interrupt clear. The second capture is saved at the second rising edge detection because there is interrupt clear.

Figure 37 Capture event with interrupt clear

How to set the PWM

Figure 38 shows the PWM setting flow step by step.

Figure 38 The PWM setting flow

PWM Channel-0 Registers (Base address : 0x4000_5000)

Channel-0 interrupt register(PWMCH0IR)

Base address : 0x4000_5000

Address offset : 0x00

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CI | OI | MI |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R | R | R |

[0] MI – Match Interrupt

This bit is set by hardware and cleared by interrupt clear register.

0 : Match interrupt does not occur.

1 : Match interrupt occurs.

[1] OI – Overflow Interrupt

This bit is set by hardware and cleared by interrupt clear register.

0 : Overflow interrupt does not occur.

1 : Overflow interrupt occurs.

[2] CI – Capture Interrupt

This bit is set by hardware and cleared by interrupt clear register.

0 : Capture interrupt does not occur.

1 : Capture interrupt occurs.

Channel-0 interrupt enable register(PWMCH0IER)

Base address : 0x4000_5000

Address offset : 0x04

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CIE | OIE | MIE |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W |

[0] MIE – Match Interrupt Enabled.

0 : Match interrupt is not enabled.

1 : Match interrupt is enabled.

[1] OIE – Overflow Interrupt Enable.

0 : Overflow interrupt is not enabled.

1 : Overflow interrupt is enabled.

[2] CIE – Capture Interrupt Enable.

0 : Capture interrupt is not enabled.

1 : Capture interrupt is enabled.

Channel-0 interrupt clear register(PWMCH0ICR)

Base address : 0x4000_5000

Address offset : 0x08

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CIC | OIC | MIC |
|  |  |  |  |  |  |  |  |  |  |  |  |  | W | W | W |

This bit is set by software, cleared by hardware when a capture interrupt becomes 0.

[0] MIC – Match Interrupt

0 : No action.

1 : Match interrupt is cleared.

[1] OIC – Overflow Interrupt

0 : No action.

1 : Overflow Interrupt is cleared.

[2] CIC – Capture Interrupt Clear.

0 : No action.

1 : Capture Interrupt is cleared.

Channel-0 Timer/Counter Register (PWMCH0TCR)

Base address : 0x4000_5000

Address offset : 0x0C

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TCR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] TCR – Timer/Counter register

Timer/Counter register. These register hold the current values of the Timer/Counter(TC). The TC is incremented every PR cycles. When the TC is reached to value of match register, the match interrupt is occurred and PWM output waveform becomes 0. When the TC is reached to the value of limit register, the overflow interrupt is occurred, the TC is reset as 0.

Channel-0 Prescale Counter Register (PWMCH0PCR)

Base address : 0x4000_5000

Address offset : 0x10

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | PCR |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R |  |  |  |  |  |

[5:0] PC – Prescale Counter register

Prescale Counter register. These registers hold the current values of prescale counter(PC). The PC is incremented to the value stored in PR. When the PC is reached to PR, the TC is incremented and the PC is reset as 0.

Channel-0 Prescale Register (PWMCH0PR)

Base address : 0x4000_5000

Address offset : 0x14

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | PR |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |

[5 - 0] PR – Prescale Register

Prescale register. The PC is incremented when the PC is reached to the PR.

Channel-0 Match Register (PWMCH0MR)

Base address : 0x4000_5000

Address offset : 0x18

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] MR – Match Register

Match register. The MR can generate a match interrupt and PWM output waveform becomes 0 when the TC is reached to the MR. Match register should be smaller than limit register(LR). If not, match interrupt is not occurred and PWM output waveform is always 1.

Channel-0 Limit Register (PWMCH0LR)

Base address : 0x4000_5000

Address offset : 0x1C

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] LR – Limit Register

Limit Register. The LR can generate an overflow interrupt and PWM output waveform becomes 1 when the TC is reached to the LR. Match register should be smaller than limit register(LR). If not, match interrupt is not occurred and PWM output waveform is always 1.

Channel-0 Up-Down Mode Register (PWMCH0UDMR)

Base address : 0x4000_5000

Address offset : 0x20

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | UDM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] UDM – Up-Down mode

0 : TC runs up count.

1 : TC runs down count.

Channel-0 Timer/Counter Mode Register (PWMCH0TCMR)

Base address : 0x4000_5000

Address offset : 0x24

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | TCM |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] TCM – Timer/Counter mode

00 : Timer mode.

01 : Counter mode with counting driven by rising edge external input .

10 : Counter mode with counting driven by falling edge external input.

11 : Counter mode with counting driven by rising and falling edge external input.

Channel-0 PWM output Enable and External input Enable Register (PWMCH0PEEER)

Base address : 0x4000_5000

Address offset : 0x28

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PEEE |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PEEE – PWM output Enable and External input Enable

00 : PWM output is disable and external input is disable.

01 : PWM output is disable and external input is enable.

10 : PWM output is enable and external input is disable.

Channel-0 Capture Mode Register (PWMCH0CMR)

Base address : 0x4000_5000

Address offset : 0x2C

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | CM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] CM – Capture mode

0 : Timer/Counter is captured when external input signal is rising edge.

1 : Timer/Counter is captured when external input signal is falling edge.

Channel-0 Capture Register (PWMCH0CR)

Base address : 0x4000_5000

Address offset : 0x30

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] CR – Capture Register

Capture register. The CR is loaded with the value of the TC when external input signal is triggered.

Channel-0 Periodic Mode Register (PWMCH0PDMR)

Base address : 0x4000_5000

Address offset : 0x34

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | PDM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] PDM – Periodic Mode

0 : Periodic mode. When the TC is reached to the LR, the TC returns to 0 and then continues counting periodically.

1 : One-shot mode. When the TC is reached to the LR, the TC returns to 0 and then stops counting.

Channel-0 Dead Zone Enable Register (PWMCH0DZER)

Base address : 0x4000_5000

Address offset : 0x38

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | DZE |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] DZE – Dead Zone Enable

0 : Dead zone generation is disabled.

1 : Dead zone generation is enabled.

Channel-0 Dead Zone Counter Register (PWMCH0DZCR)

Base address : 0x4000_5000

Address offset : 0x3C

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | DZC |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |  |  |

[9:0] DZC – Dead Zone Counter value

Dead zone generation counter value register. If the DZE bit in DZER is 1, dead zone counter counts to this value and during this time, the two PWM output waveforms are all 0.

Register map

The following Table 17 summarizes the PWM Channel-0 registers.

Table 17 PWM channel 0 register map and reset values

PWM Channel-1 Registers (Base address : 0x4000_5100)

Channel-1 interrupt register(PWMCH1IR)

Base address : 0x4000_5100

Address offset : 0x00

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CI | OI | MI |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R | R | R |

[0] MI – Match Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Match interrupt does not occur.

1 : Match interrupt occurs.

[1] OI – Overflow Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Overflow interrupt does not occur.

1 : Overflow interrupt occurs.

[2] CI – Capture Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Capture interrupt does not occur.

1 : Capture interrupt occurs.

Channel-1 interrupt enable register(PWMCH1IER)

Base address : 0x4000_5100

Address offset : 0x04

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CIE | OIE | MIE |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W |

[0] MIE – Match Interrupt Enabled.

O : Match interrupt is not enabled.

1 : Match interrupt is enabled.

[1] OIE – Overflow Interrupt Enable.

O : Overflow interrupt is not enabled.

1 : Overflow interrupt is enabled.

[2] CIE – Capture Interrupt Enable.

O : Capture interrupt is not enabled.

1 : Capture interrupt is enabled.

Channel-1 interrupt clear register(PWMCH1ICR)

Base address : 0x4000_5100

Address offset : 0x08

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CIC | OIC | MIC |
|  |  |  |  |  |  |  |  |  |  |  |  |  | W | W | W |

This bit is set by software, cleared by hardware when a capture interrupt becomes 0.

[0] MIC – Match Interrupt

O : No action.

1 : Match interrupt is cleared.

[1] OIC – Overflow Interrupt

O : No action.

1 : Overflow Interrupt is cleared.

[2] CIC – Capture Interrupt Clear.

O : No action.

1 : Capture Interrupt is cleared.

Channel-1 Timer/Counter Register (PWMCH1TCR)

Base address : 0x4000_5100

Address offset : 0x0C

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TCR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] TCR – Timer/Counter register

Timer/Counter register. These register hold the current values of the Timer/Counter(TC). The TC is incremented every PR cycles. When the TC is reached to value of match register, the match interrupt is occurred and PWM output waveform becomes 0. When the TC is reached to the value of limit register, the overflow interrupt is occurred, the TC is reset as 0.

Channel-1 Prescale Counter Register (PWMCH1PCR)

Base address : 0x4000_5100

Address offset : 0x10

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | PCR |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R |  |  |  |  |  |

[5:0] PC – Prescale Counter register

Prescale Counter register. These registers hold the current values of prescale counter(PC). The PC is incremented to the value stored in PR. When the PC is reached to PR, the TC is incremented and the PC is reset as 0.

Channel-1 Prescale Register (PWMCH1PR)

Base address : 0x4000_5100

Address offset : 0x14

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | PR |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |

[5: 0] PR – Prescale Register

Prescale register. The PC is incremented when the PC is reached to the PR.

Channel-1 Match Register (PWMCH1MR)

Base address : 0x4000_5100

Address offset : 0x18

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] MR – Match Register

Match register. The MR can generate a match interrupt and PWM output waveform becomes 0 when the TC is reached to the MR. Match register should be smaller than limit register(LR). If not, match interrupt is not occurred and PWM output waveform is always 1.

Channel-1 Limit Register (PWMCH1LR)

Base address : 0x4000_5100

Address offset : 0x1C

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] LR – Limit Register

Limit Register. The LR can generate an overflow interrupt and PWM output waveform becomes 1 when the TC is reached to the LR. Match register should be smaller than limit register(LR). If not, match interrupt is not occurred and PWM output waveform is always 1.

Channel-1 Up-Down Mode Register (PWMCH1UDMR)

Base address : 0x4000_5100

Address offset : 0x20

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | UDM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] UDM – Up-Down mode

0 : TC runs up count.

1 : TC runs down count.

Channel-1 Timer/Counter Mode Register (PWMCH1TCMR)

Base address : 0x4000_5100

Address offset : 0x24

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | TCM |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] TCM – Timer/Counter mode

00 : Timer mode.

01 : Counter mode with counting driven by rising edge external input .

10 : Counter mode with counting driven by falling edge external input.

11 : Counter mode with counting driven by rising and falling edge external input.

Channel-1 PWM output Enable and External input Enable Register (PWMCH1PEEER)

Base address : 0x4000_5100

Address offset : 0x28

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PEEE |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PEEE – PWM output Enable and External input Enable

00 : PWM output is disable and external input is disable.

01 : PWM output is disable and external input is enable.

10 : PWM output is enable and external input is disable.

Channel-1 Capture Mode Register (PWMCH1CMR)

Base address : 0x4000_5100

Address offset : 0x2C

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | CM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] CM – Capture mode

0 : Timer/Counter is captured when external input signal is rising edge.

1 : Timer/Counter is captured when external input signal is falling edge.

Channel-1 Capture Register (PWMCH1CR)

Base address : 0x4000_5100

Address offset : 0x30

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] CR – Capture Register

Capture register. The CR is loaded with the value of the TC when external input signal is triggered.

Channel-1 Periodic Mode Register (PWMCH1PDMR)

Base address : 0x4000_5100

Address offset : 0x34

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | PDM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] PDM – Periodic Mode

0 : Periodic mode. When the TC is reached to the LR, the TC returns to 0 and then continues counting periodically.

1 : One-shot mode. When the TC is reached to the LR, the TC returns to 0 and then stops counting.

Channel-1 Dead Zone Enable Register (PWMCH1DZER)

Base address : 0x4000_5100

Address offset : 0x38

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | DZE |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] DZE – Dead Zone Enable

0 : Dead zone generation is disabled.

1 : Dead zone generation is enabled.

Channel-1 Dead Zone Counter Register (PWMCH1DZCR)

Base address : 0x4000_5100

Address offset : 0x3C

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | DZC |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |  |  |

[9:0] DZC – Dead Zone Counter value

Dead zone generation counter value register. If the DZE bit in DZER is 1, dead zone counter counts to this value and during this time, the two PWM output waveforms are all 0.

Register map

The following Table 18 summarizes the PWM Channel-1 registers.

Table 18 PWM channel 1 register map and reset values

PWM Channel-2 Registers (Base address : 0x4000_5200)

Channel-2 interrupt register(PWMCH2IR)

Base address : 0x4000_5200

Address offset : 0x00

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CI | OI | MI |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R | R | R |

[0] MI – Match Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Match interrupt does not occur.

1 : Match interrupt occurs.

[1] OI – Overflow Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Overflow interrupt does not occur.

1 : Overflow interrupt occurs.

[2] CI – Capture Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Capture interrupt does not occur.

1 : Capture interrupt occurs.

Channel-2 interrupt enable register(PWMCH2IER)

Base address : 0x4000_5200

Address offset : 0x04

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CIE | OIE | MIE |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W |

[0] MIE – Match Interrupt Enabled.

O : Match interrupt is not enabled.

1 : Match interrupt is enabled.

[1] OIE – Overflow Interrupt Enable.

O : Overflow interrupt is not enabled.

1 : Overflow interrupt is enabled.

[2] CIE – Capture Interrupt Enable.

O : Capture interrupt is not enabled.

1 : Capture interrupt is enabled.

Channel-2 interrupt clear register(PWMCH2ICR)

Base address : 0x4000_5200

Address offset : 0x08

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CIC | OIC | MIC |
|  |  |  |  |  |  |  |  |  |  |  |  |  | W | W | W |

This bit is set by software, cleared by hardware when a capture interrupt becomes 0.

[0] MIC – Match Interrupt

O : No action.

1 : Match interrupt is cleared.

[1] OIC – Overflow Interrupt

O : No action.

1 : Overflow Interrupt is cleared.

[2] CIC – Capture Interrupt Clear.

O : No action.

1 : Capture Interrupt is cleared.

Channel-2 Timer/Counter Register (PWMCH2TCR)

Base address : 0x4000_5200

Address offset : 0x0C

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TCR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] TCR – Timer/Counter register

Timer/Counter register. These register hold the current values of the Timer/Counter(TC). The TC is incremented every PR cycles. When the TC is reached to value of match register, the match interrupt is occurred and PWM output waveform becomes 0. When the TC is reached to the value of limit register, the overflow interrupt is occurred, the TC is reset as 0.

Channel-2 Prescale Counter Register (PWMCH2PCR)

Base address : 0x4000_5200

Address offset : 0x10

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | PCR |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R |  |  |  |  |  |

[5:0] PC – Prescale Counter register

Prescale Counter register. These registers hold the current values of prescale counter(PC). The PC is incremented to the value stored in PR. When the PC is reached to PR, the TC is incremented and the PC is reset as 0.

Channel-2 Prescale Register (PWMCH2PR)

Base address : 0x4000_5200

Address offset : 0x14

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | PR |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |

[5: 0] PR – Prescale Register

Prescale register. The PC is incremented when the PC is reached to the PR.

Channel-2 Match Register (PWMCH2MR)

Base address : 0x4000_5200

Address offset : 0x18

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] MR – Match Register

Match register. The MR can generate a match interrupt and PWM output waveform becomes 0 when the TC is reached to the MR. Match register should be smaller than limit register(LR). If not, match interrupt is not occurred and PWM output waveform is always 1.

Channel-2 Limit Register (PWMCH2LR)

Base address : 0x4000_5200

Address offset : 0x1C

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] LR – Limit Register

Limit Register. The LR can generate an overflow interrupt and PWM output waveform becomes 1 when the TC is reached to the LR. Match register should be smaller than limit register(LR). If not, match interrupt is not occurred and PWM output waveform is always 1.

Channel-2 Up-Down Mode Register (PWMCH2UDMR)

Base address : 0x4000_5200

Address offset : 0x20

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | UDM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] UDM – Up-Down mode

0 : TC runs up count.

1 : TC runs down count.

Channel-2 Timer/Counter Mode Register (PWMCH2TCMR)

Base address : 0x4000_5200

Address offset : 0x24

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | TCM |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] TCM – Timer/Counter mode

00 : Timer mode.

01 : Counter mode with counting driven by rising edge external input .

10 : Counter mode with counting driven by falling edge external input.

11 : Counter mode with counting driven by rising and falling edge external input.

Channel-2 PWM output Enable and External input Enable Register (PWMCH2PEEER)

Base address : 0x4000_5200

Address offset : 0x28

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PEEE |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PEEE – PWM output Enable and External input Enable

00 : PWM output is disable and external input is disable.

01 : PWM output is disable and external input is enable.

10 : PWM output is enable and external input is disable.

Channel-2 Capture Mode Register (PWMCH2CMR)

Base address : 0x4000_5200

Address offset : 0x2C

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | CM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] CM – Capture mode

0 : Timer/Counter is captured when external input signal is rising edge.

1 : Timer/Counter is captured when external input signal is falling edge.

Channel-2 Capture Register (PWMCH2CR)

Base address : 0x4000_5200

Address offset : 0x30

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] CR – Capture Register

Capture register. The CR is loaded with the value of the TC when external input signal is triggered.

Channel-2 Periodic Mode Register (PWMCH2PDMR)

Base address : 0x4000_5200

Address offset : 0x34

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | PDM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] PDM – Periodic Mode

0 : Periodic mode. When the TC is reached to the LR, the TC returns to 0 and then continues counting periodically.

1 : One-shot mode. When the TC is reached to the LR, the TC returns to 0 and then stops counting.

Channel-2 Dead Zone Enable Register (PWMCH2DZER)

Base address : 0x4000_5200

Address offset : 0x38

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | DZE |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] DZE – Dead Zone Enable

0 : Dead zone generation is disabled.

1 : Dead zone generation is enabled.

Channel-2 Dead Zone Counter Register (PWMCH2DZCR)

Base address : 0x4000_5200

Address offset : 0x3C

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | DZC |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |  |  |

[9:0] DZC – Dead Zone Counter value

Dead zone generation counter value register. If the DZE bit in DZER is 1, dead zone counter counts to this value and during this time, the two PWM output waveforms are all 0.

Register map

The following Table 19 summarizes the PWM Channel-2 registers.

Table 19 PWM channel 2 register map and reset values

PWM Channel-3 Registers (Base address : 0x4000_5300)

Channel-3 interrupt register(PWMCH3IR)

Base address : 0x4000_5300

Address offset : 0x00

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CI | OI | MI |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R | R | R |

[0] MI – Match Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Match interrupt does not occur.

1 : Match interrupt occurs.

[1] OI – Overflow Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Overflow interrupt does not occur.

1 : Overflow interrupt occurs.

[2] CI – Capture Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Capture interrupt does not occur.

1 : Capture interrupt occurs.

Channel-3 interrupt enable register(PWMCH3IER)

Base address : 0x4000_5300

Address offset : 0x04

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CIE | OIE | MIE |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W |

[0] MIE – Match Interrupt Enabled.

O : Match interrupt is not enabled.

1 : Match interrupt is enabled.

[1] OIE – Overflow Interrupt Enable.

O : Overflow interrupt is not enabled.

1 : Overflow interrupt is enabled.

[2] CIE – Capture Interrupt Enable.

O : Capture interrupt is not enabled.

1 : Capture interrupt is enabled.

Channel-3 interrupt clear register(PWMCH3ICR)

Base address : 0x4000_5300

Address offset : 0x08

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CIC | OIC | MIC |
|  |  |  |  |  |  |  |  |  |  |  |  |  | W | W | W |

This bit is set by software, cleared by hardware when a capture interrupt becomes 0.

[0] MIC – Match Interrupt

O : No action.

1 : Match interrupt is cleared.

[1] OIC – Overflow Interrupt

O : No action.

1 : Overflow Interrupt is cleared.

[2] CIC – Capture Interrupt Clear.

O : No action.

1 : Capture Interrupt is cleared.

Channel-3 Timer/Counter Register (PWMCH3TCR)

Base address : 0x4000_5300

Address offset : 0x0C

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TCR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] TCR – Timer/Counter register

Timer/Counter register. These register hold the current values of the Timer/Counter(TC). The TC is incremented every PR cycles. When the TC is reached to value of match register, the match interrupt is occurred and PWM output waveform becomes 0. When the TC is reached to the value of limit register, the overflow interrupt is occurred, the TC is reset as 0.

Channel-3 Prescale Counter Register (PWMCH3PCR)

Base address : 0x4000_5300

Address offset : 0x10

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | PCR |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R |  |  |  |  |  |

[5:0] PC – Prescale Counter register

Prescale Counter register. These registers hold the current values of prescale counter(PC). The PC is incremented to the value stored in PR. When the PC is reached to PR, the TC is incremented and the PC is reset as 0.

Channel-3 Prescale Register (PWMCH3PR)

Base address : 0x4000_5300

Address offset : 0x14

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | PR |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |

[5: 0] PR – Prescale Register

Prescale register. The PC is incremented when the PC is reached to the PR.

Channel-3 Match Register (PWMCH3MR)

Base address : 0x4000_5300

Address offset : 0x18

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] MR – Match Register

Match register. The MR can generate a match interrupt and PWM output waveform becomes 0 when the TC is reached to the MR. Match register should be smaller than limit register(LR). If not, match interrupt is not occurred and PWM output waveform is always 1.

Channel-3 Limit Register (PWMCH3LR)

Base address : 0x4000_5300

Address offset : 0x1C

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] LR – Limit Register

Limit Register. The LR can generate an overflow interrupt and PWM output waveform becomes 1 when the TC is reached to the LR. Match register should be smaller than limit register(LR). If not, match interrupt is not occurred and PWM output waveform is always 1.

Channel-3 Up-Down Mode Register (PWMCH3UDMR)

Base address : 0x4000_5300

Address offset : 0x20

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | UDM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] UDM – Up-Down mode

0 : TC runs up count.

1 : TC runs down count.

Channel-3 Timer/Counter Mode Register (PWMCH3TCMR)

Base address : 0x4000_5300

Address offset : 0x24

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | TCM |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] TCM – Timer/Counter mode

00 : Timer mode.

01 : Counter mode with counting driven by rising edge external input .

10 : Counter mode with counting driven by falling edge external input.

11 : Counter mode with counting driven by rising and falling edge external input.

Channel-3 PWM output Enable and External input Enable Register (PWMCH3PEEER)

Base address : 0x4000_5300

Address offset : 0x28

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PEEE |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PEEE – PWM output Enable and External input Enable

00 : PWM output is disable and external input is disable.

01 : PWM output is disable and external input is enable.

10 : PWM output is enable and external input is disable.

Channel-3 Capture Mode Register (PWMCH3CMR)

Base address : 0x4000_5300

Address offset : 0x2C

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | CM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] CM – Capture mode

0 : Timer/Counter is captured when external input signal is rising edge.

1 : Timer/Counter is captured when external input signal is falling edge.

Channel-3 Capture Register (PWMCH3CR)

Base address : 0x4000_5300

Address offset : 0x30

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] CR – Capture Register

Capture register. The CR is loaded with the value of the TC when external input signal is triggered.

Channel-3 Periodic Mode Register (PWMCH3PDMR)

Base address : 0x4000_5300

Address offset : 0x34

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | PDM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] PDM – Periodic Mode

0 : Periodic mode. When the TC is reached to the LR, the TC returns to 0 and then continues counting periodically.

1 : One-shot mode. When the TC is reached to the LR, the TC returns to 0 and then stops counting.

Channel-3 Dead Zone Enable Register (PWMCH3DZER)

Base address : 0x4000_5300

Address offset : 0x38

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | DZE |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] DZE – Dead Zone Enable

0 : Dead zone generation is disabled.

1 : Dead zone generation is enabled.

Channel-3 Dead Zone Counter Register (PWMCH3DZCR)

Base address : 0x4000_5300

Address offset : 0x3C

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | DZC |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |  |  |

[9:0] DZC – Dead Zone Counter value

Dead zone generation counter value register. If the DZE bit in DZER is 1, dead zone counter counts to this value and during this time, the two PWM output waveforms are all 0.

Register map

The following Table 20 summarizes the PWM Channel-3 registers.

Table 20 PWM channel 3 register map and reset values

PWM Channel-4 Registers (Base address : 0x4000_5400)

Channel-4 interrupt register(PWMCH4IR)

Base address : 0x4000_5400

Address offset : 0x00

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CI | OI | MI |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R | R | R |

[0] MI – Match Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Match interrupt does not occur.

1 : Match interrupt occurs.

[1] OI – Overflow Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Overflow interrupt does not occur.

1 : Overflow interrupt occurs.

[2] CI – Capture Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Capture interrupt does not occur.

1 : Capture interrupt occurs.

Channel-4 interrupt enable register(PWMCH4IER)

Base address : 0x4000_5400

Address offset : 0x04

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CIE | OIE | MIE |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W |

[0] MIE – Match Interrupt Enabled.

O : Match interrupt is not enabled.

1 : Match interrupt is enabled.

[1] OIE – Overflow Interrupt Enable.

O : Overflow interrupt is not enabled.

1 : Overflow interrupt is enabled.

[2] CIE – Capture Interrupt Enable.

O : Capture interrupt is not enabled.

1 : Capture interrupt is enabled.

Channel-4 interrupt clear register(PWMCH4ICR)

Base address : 0x4000_5400

Address offset : 0x08

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CIC | OIC | MIC |
|  |  |  |  |  |  |  |  |  |  |  |  |  | W | W | W |

This bit is set by software, cleared by hardware when a capture interrupt becomes 0.

[0] MIC – Match Interrupt

O : No action.

1 : Match interrupt is cleared.

[1] OIC – Overflow Interrupt

O : No action.

1 : Overflow Interrupt is cleared.

[2] CIC – Capture Interrupt Clear.

O : No action.

1 : Capture Interrupt is cleared.

Channel-4 Timer/Counter Register (PWMCH4TCR)

Base address : 0x4000_5400

Address offset : 0x0C

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TCR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] TCR – Timer/Counter register

Timer/Counter register. These register hold the current values of the Timer/Counter(TC). The TC is incremented every PR cycles. When the TC is reached to value of match register, the match interrupt is occurred and PWM output waveform becomes 0. When the TC is reached to the value of limit register, the overflow interrupt is occurred, the TC is reset as 0.

Channel-4 Prescale Counter Register (PWMCH4PCR)

Base address : 0x4000_5400

Address offset : 0x10

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | PCR |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R |  |  |  |  |  |

[5:0] PC – Prescale Counter register

Prescale Counter register. These registers hold the current values of prescale counter(PC). The PC is incremented to the value stored in PR. When the PC is reached to PR, the TC is incremented and the PC is reset as 0.

Channel-4 Prescale Register (PWMCH4PR)

Base address : 0x4000_5400

Address offset : 0x14

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | PR |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |

[5: 0] PR – Prescale Register

Prescale register. The PC is incremented when the PC is reached to the PR.

Channel-4 Match Register (PWMCH4MR)

Base address : 0x4000_5400

Address offset : 0x18

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] MR – Match Register

Match register. The MR can generate a match interrupt and PWM output waveform becomes 0 when the TC is reached to the MR. Match register should be smaller than limit register(LR). If not, match interrupt is not occurred and PWM output waveform is always 1.

Channel-4 Limit Register (PWMCH4LR)

Base address : 0x4000_5400

Address offset : 0x1C

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] LR – Limit Register

Limit Register. The LR can generate an overflow interrupt and PWM output waveform becomes 1 when the TC is reached to the LR. Match register should be smaller than limit register(LR). If not, match interrupt is not occurred and PWM output waveform is always 1.

Channel-4 Up-Down Mode Register (PWMCH4UDMR)

Base address : 0x4000_5400

Address offset : 0x20

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | UDM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] UDM – Up-Down mode

0 : TC runs up count.

1 : TC runs down count.

Channel-4 Timer/Counter Mode Register (PWMCH4TCMR)

Base address : 0x4000_5400

Address offset : 0x24

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | TCM |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] TCM – Timer/Counter mode

00 : Timer mode.

01 : Counter mode with counting driven by rising edge external input .

10 : Counter mode with counting driven by falling edge external input.

11 : Counter mode with counting driven by rising and falling edge external input.

Channel-4 PWM output Enable and External input Enable Register (PWMCH4PEEER)

Base address : 0x4000_5400

Address offset : 0x28

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PEEE |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PEEE – PWM output Enable and External input Enable

00 : PWM output is disable and external input is disable.

01 : PWM output is disable and external input is enable.

10 : PWM output is enable and external input is disable.

Channel-4 Capture Mode Register (PWMCH4CMR)

Base address : 0x4000_5400

Address offset : 0x2C

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | CM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] CM – Capture mode

0 : Timer/Counter is captured when external input signal is rising edge.

1 : Timer/Counter is captured when external input signal is falling edge.

Channel-4 Capture Register (PWMCH4CR)

Base address : 0x4000_5400

Address offset : 0x30

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] CR – Capture Register

Capture register. The CR is loaded with the value of the TC when external input signal is triggered.

Channel-4 Periodic Mode Register (PWMCH4PDMR)

Base address : 0x4000_5400

Address offset : 0x34

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | PDM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] PDM – Periodic Mode

0 : Periodic mode. When the TC is reached to the LR, the TC returns to 0 and then continues counting periodically.

1 : One-shot mode. When the TC is reached to the LR, the TC returns to 0 and then stops counting.

Channel-4 Dead Zone Enable Register (PWMCH4DZER)

Base address : 0x4000_5400

Address offset : 0x38

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | DZE |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] DZE – Dead Zone Enable

0 : Dead zone generation is disabled.

1 : Dead zone generation is enabled.

Channel-4 Dead Zone Counter Register (PWMCH4DZCR)

Base address : 0x4000_5400

Address offset : 0x3C

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | DZC |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |  |  |

[9:0] DZC – Dead Zone Counter value

Dead zone generation counter value register. If the DZE bit in DZER is 1, dead zone counter counts to this value and during this time, the two PWM output waveforms are all 0.

Register map

The following Table 21 summarizes the PWM Channel-4 registers.

Table 21 PWM channel 4 register map and reset values

PWM Channel-5 Registers (Base address : 0x4000_5500)

Channel-5 interrupt register(PWMCH5IR)

Base address : 0x4000_5500

Address offset : 0x00

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CI | OI | MI |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R | R | R |

[0] MI – Match Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Match interrupt does not occur.

1 : Match interrupt occurs.

[1] OI – Overflow Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Overflow interrupt does not occur.

1 : Overflow interrupt occurs.

[2] CI – Capture Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Capture interrupt does not occur.

1 : Capture interrupt occurs.

Channel-5 interrupt enable register(PWMCH5IER)

Base address : 0x4000_5500

Address offset : 0x04

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CIE | OIE | MIE |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W |

[0] MIE – Match Interrupt Enabled.

O : Match interrupt is not enabled.

1 : Match interrupt is enabled.

[1] OIE – Overflow Interrupt Enable.

O : Overflow interrupt is not enabled.

1 : Overflow interrupt is enabled.

[2] CIE – Capture Interrupt Enable.

O : Capture interrupt is not enabled.

1 : Capture interrupt is enabled.

Channel-5 interrupt clear register(PWMCH5ICR)

Base address : 0x4000_5500

Address offset : 0x08

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CIC | OIC | MIC |
|  |  |  |  |  |  |  |  |  |  |  |  |  | W | W | W |

This bit is set by software, cleared by hardware when a capture interrupt becomes 0.

[0] MIC – Match Interrupt

O : No action.

1 : Match interrupt is cleared.

[1] OIC – Overflow Interrupt

O : No action.

1 : Overflow Interrupt is cleared.

[2] CIC – Capture Interrupt Clear.

O : No action.

1 : Capture Interrupt is cleared.

Channel-5 Timer/Counter Register (PWMCH5TCR)

Base address : 0x4000_5500

Address offset : 0x0C

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TCR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] TCR – Timer/Counter register

Timer/Counter register. These register hold the current values of the Timer/Counter(TC). The TC is incremented every PR cycles. When the TC is reached to value of match register, the match interrupt is occurred and PWM output waveform becomes 0. When the TC is reached to the value of limit register, the overflow interrupt is occurred, the TC is reset as 0.

Channel-5 Prescale Counter Register (PWMCH5PCR)

Base address : 0x4000_5500

Address offset : 0x10

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | PCR |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R |  |  |  |  |  |

[5:0] PC – Prescale Counter register

Prescale Counter register. These registers hold the current values of prescale counter(PC). The PC is incremented to the value stored in PR. When the PC is reached to PR, the TC is incremented and the PC is reset as 0.

Channel-5 Prescale Register (PWMCH5PR)

Base address : 0x4000_5500

Address offset : 0x14

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | PR |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |

[5: 0] PR – Prescale Register

Prescale register. The PC is incremented when the PC is reached to the PR.

Channel-5 Match Register (PWMCH5MR)

Base address : 0x4000_5500

Address offset : 0x18

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] MR – Match Register

Match register. The MR can generate a match interrupt and PWM output waveform becomes 0 when the TC is reached to the MR. Match register should be smaller than limit register(LR). If not, match interrupt is not occurred and PWM output waveform is always 1.

Channel-5 Limit Register (PWMCH5LR)

Base address : 0x4000_5500

Address offset : 0x1C

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] LR – Limit Register

Limit Register. The LR can generate an overflow interrupt and PWM output waveform becomes 1 when the TC is reached to the LR. Match register should be smaller than limit register(LR). If not, match interrupt is not occurred and PWM output waveform is always 1.

Channel-5 Up-Down Mode Register (PWMCH5UDMR)

Base address : 0x4000_5500

Address offset : 0x20

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | UDM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] UDM – Up-Down mode

0 : TC runs up count.

1 : TC runs down count.

Channel-5 Timer/Counter Mode Register (PWMCH5TCMR)

Base address : 0x4000_5500

Address offset : 0x24

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | TCM |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] TCM – Timer/Counter mode

00 : Timer mode.

01 : Counter mode with counting driven by rising edge external input .

10 : Counter mode with counting driven by falling edge external input.

11 : Counter mode with counting driven by rising and falling edge external input.

Channel-5 PWM output Enable and External input Enable Register (PWMCH5PEEER)

Base address : 0x4000_5500

Address offset : 0x28

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PEEE |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PEEE – PWM output Enable and External input Enable

00 : PWM output is disable and external input is disable.

01 : PWM output is disable and external input is enable.

10 : PWM output is enable and external input is disable.

Channel-5 Capture Mode Register (PWMCH5CMR)

Base address : 0x4000_5500

Address offset : 0x2C

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | CM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] CM – Capture mode

0 : Timer/Counter is captured when external input signal is rising edge.

1 : Timer/Counter is captured when external input signal is falling edge.

Channel-5 Capture Register (PWMCH5CR)

Base address : 0x4000_5500

Address offset : 0x30

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] CR – Capture Register

Capture register. The CR is loaded with the value of the TC when external input signal is triggered.

Channel-5 Periodic Mode Register (PWMCH5PDMR)

Base address : 0x4000_5500

Address offset : 0x34

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | PDM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] PDM – Periodic Mode

0 : Periodic mode. When the TC is reached to the LR, the TC returns to 0 and then continues counting periodically.

1 : One-shot mode. When the TC is reached to the LR, the TC returns to 0 and then stops counting.

Channel-5 Dead Zone Enable Register (PWMCH5DZER)

Base address : 0x4000_5500

Address offset : 0x38

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | DZE |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] DZE – Dead Zone Enable

0 : Dead zone generation is disabled.

1 : Dead zone generation is enabled.

Channel-5 Dead Zone Counter Register (PWMCH5DZCR)

Base address : 0x4000_5500

Address offset : 0x3C

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | DZC |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |  |  |

[9:0] DZC – Dead Zone Counter value

Dead zone generation counter value register. If the DZE bit in DZER is 1, dead zone counter counts to this value and during this time, the two PWM output waveforms are all 0.

Register map

The following Table 22 summarizes the PWM Channel-5 registers.

Table 22 PWM channel 5 register map and reset values

PWM Channel-6 Registers (Base address : 0x4000_5600)

Channel-6 interrupt register(PWMCH6IR)

Base address : 0x4000_5600

Address offset : 0x00

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CI | OI | MI |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R | R | R |

[0] MI – Match Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Match interrupt does not occur.

1 : Match interrupt occurs.

[1] OI – Overflow Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Overflow interrupt does not occur.

1 : Overflow interrupt occurs.

[2] CI – Capture Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Capture interrupt does not occur.

1 : Capture interrupt occurs.

Channel-6 interrupt enable register(PWMCH6IER)

Base address : 0x4000_5600

Address offset : 0x04

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CIE | OIE | MIE |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W |

[0] MIE – Match Interrupt Enabled.

O : Match interrupt is not enabled.

1 : Match interrupt is enabled.

[1] OIE – Overflow Interrupt Enable.

O : Overflow interrupt is not enabled.

1 : Overflow interrupt is enabled.

[2] CIE – Capture Interrupt Enable.

O : Capture interrupt is not enabled.

1 : Capture interrupt is enabled.

Channel-6 interrupt clear register(PWMCH6ICR)

Base address : 0x4000_5600

Address offset : 0x08

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CIC | OIC | MIC |
|  |  |  |  |  |  |  |  |  |  |  |  |  | W | W | W |

This bit is set by software, cleared by hardware when a capture interrupt becomes 0.

[0] MIC – Match Interrupt

O : No action.

1 : Match interrupt is cleared.

[1] OIC – Overflow Interrupt

O : No action.

1 : Overflow Interrupt is cleared.

[2] CIC – Capture Interrupt Clear.

O : No action.

1 : Capture Interrupt is cleared.

Channel-6 Timer/Counter Register (PWMCH6TCR)

Base address : 0x4000_5600

Address offset : 0x0C

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TCR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] TCR – Timer/Counter register

Timer/Counter register. These register hold the current values of the Timer/Counter(TC). The TC is incremented every PR cycles. When the TC is reached to value of match register, the match interrupt is occurred and PWM output waveform becomes 0. When the TC is reached to the value of limit register, the overflow interrupt is occurred, the TC is reset as 0.

Channel-6 Prescale Counter Register (PWMCH6PCR)

Base address : 0x4000_5600

Address offset : 0x10

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | PCR |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R |  |  |  |  |  |

[5:0] PC – Prescale Counter register

Prescale Counter register. These registers hold the current values of prescale counter(PC). The PC is incremented to the value stored in PR. When the PC is reached to PR, the TC is incremented and the PC is reset as 0.

Channel-6 Prescale Register (PWMCH6PR)

Base address : 0x4000_5600

Address offset : 0x14

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | PR |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |

[5:0] PR – Prescale Register

Prescale register. The PC is incremented when the PC is reached to the PR.

Channel-6 Match Register (PWMCH6MR)

Base address : 0x4000_5600

Address offset : 0x18

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] MR – Match Register

Match register. The MR can generate a match interrupt and PWM output waveform becomes 0 when the TC is reached to the MR. Match register should be smaller than limit register(LR). If not, match interrupt is not occurred and PWM output waveform is always 1.

Channel-6 Limit Register (PWMCH6LR)

Base address : 0x4000_5600

Address offset : 0x1C

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] LR – Limit Register

Limit Register. The LR can generate an overflow interrupt and PWM output waveform becomes 1 when the TC is reached to the LR. Match register should be smaller than limit register(LR). If not, match interrupt is not occurred and PWM output waveform is always 1.

Channel-6 Up-Down Mode Register (PWMCH6UDMR)

Base address : 0x4000_5600

Address offset : 0x20

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | UDM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] UDM – Up-Down mode

0 : TC runs up count.

1 : TC runs down count.

Channel-6 Timer/Counter Mode Register (PWMCH6TCMR)

Base address : 0x4000_5600

Address offset : 0x24

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | TCM |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] TCM – Timer/Counter mode

00 : Timer mode.

01 : Counter mode with counting driven by rising edge external input .

10 : Counter mode with counting driven by falling edge external input.

11 : Counter mode with counting driven by rising and falling edge external input.

Channel-6 PWM output Enable and External input Enable Register (PWMCH6PEEER)

Base address : 0x4000_5600

Address offset : 0x28

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PEEE |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PEEE – PWM output Enable and External input Enable

00 : PWM output is disable and external input is disable.

01 : PWM output is disable and external input is enable.

10 : PWM output is enable and external input is disable.

Channel-6 Capture Mode Register (PWMCH6CMR)

Base address : 0x4000_5600

Address offset : 0x2C

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | CM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] CM – Capture mode

0 : Timer/Counter is captured when external input signal is rising edge.

1 : Timer/Counter is captured when external input signal is falling edge.

Channel-6 Capture Register (PWMCH6CR)

Base address : 0x4000_5600

Address offset : 0x30

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] CR – Capture Register

Capture register. The CR is loaded with the value of the TC when external input signal is triggered.

Channel-6 Periodic Mode Register (PWMCH6PDMR)

Base address : 0x4000_5600

Address offset : 0x34

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | PDM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] PDM – Periodic Mode

0 : Periodic mode. When the TC is reached to the LR, the TC returns to 0 and then continues counting periodically.

1 : One-shot mode. When the TC is reached to the LR, the TC returns to 0 and then stops counting.

Channel-6 Dead Zone Enable Register (PWMCH6DZER)

Base address : 0x4000_5600

Address offset : 0x38

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | DZE |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] DZE – Dead Zone Enable

0 : Dead zone generation is disabled.

1 : Dead zone generation is enabled.

Channel-6 Dead Zone Counter Register (PWMCH6DZCR)

Base address : 0x4000_5600

Address offset : 0x3C

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | DZC |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |  |  |

[9:0] DZC – Dead Zone Counter value

Dead zone generation counter value register. If the DZE bit in DZER is 1, dead zone counter counts to this value and during this time, the two PWM output waveforms are all 0.

Register map

The following Table 23 summarizes the PWM Channel-5 registers.

Table 23 PWM channel 6 register map and reset values

PWM Channel-7 Registers (Base address : 0x4000_5700)

Channel-7 interrupt register(PWMCH7IR)

Base address : 0x4000_5700

Address offset : 0x00

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CI | OI | MI |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R | R | R |

[0] MI – Match Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Match interrupt does not occur.

1 : Match interrupt occurs.

[1] OI – Overflow Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Overflow interrupt does not occur.

1 : Overflow interrupt occurs.

[2] CI – Capture Interrupt

This bit is set by hardware and cleared by interrupt clear register.

O : Capture interrupt does not occur.

1 : Capture interrupt occurs.

Channel-7 interrupt enable register(PWMCH7IER)

Base address : 0x4000_5700

Address offset : 0x04

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CIE | OIE | MIE |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W |

[0] MIE – Match Interrupt Enabled.

O : Match interrupt is not enabled.

1 : Match interrupt is enabled.

[1] OIE – Overflow Interrupt Enable.

O : Overflow interrupt is not enabled.

1 : Overflow interrupt is enabled.

[2] CIE – Capture Interrupt Enable.

O : Capture interrupt is not enabled.

1 : Capture interrupt is enabled.

Channel-7 interrupt clear register(PWMCH7ICR)

Base address : 0x4000_5700

Address offset : 0x08

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | CIC | OIC | MIC |
|  |  |  |  |  |  |  |  |  |  |  |  |  | W | W | W |

This bit is set by software, cleared by hardware when a capture interrupt becomes 0.

[0] MIC – Match Interrupt

O : No action.

1 : Match interrupt is cleared.

[1] OIC – Overflow Interrupt

O : No action.

1 : Overflow Interrupt is cleared.

[2] CIC – Capture Interrupt Clear.

O : No action.

1 : Capture Interrupt is cleared.

Channel-7 Timer/Counter Register (PWMCH7TCR)

Base address : 0x4000_5700

Address offset : 0x0C

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TCR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] TCR – Timer/Counter register

Timer/Counter register. These register hold the current values of the Timer/Counter(TC). The TC is incremented every PR cycles. When the TC is reached to value of match register, the match interrupt is occurred and PWM output waveform becomes 0. When the TC is reached to the value of limit register, the overflow interrupt is occurred, the TC is reset as 0.

Channel-7 Prescale Counter Register (PWMCH7PCR)

Base address : 0x4000_5700

Address offset : 0x10

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | PCR |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R |  |  |  |  |  |

[5:0] PC – Prescale Counter register

Prescale Counter register. These registers hold the current values of prescale counter(PC). The PC is incremented to the value stored in PR. When the PC is reached to PR, the TC is incremented and the PC is reset as 0.

Channel-7 Prescale Register (PWMCH7PR)

Base address : 0x4000_5700

Address offset : 0x14

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | PR |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |

[5:0] PR – Prescale Register

Prescale register. The PC is incremented when the PC is reached to the PR.

Channel-7 Match Register (PWMCH7MR)

Base address : 0x4000_5700

Address offset : 0x18

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] MR – Match Register

Match register. The MR can generate a match interrupt and PWM output waveform becomes 0 when the TC is reached to the MR. Match register should be smaller than limit register(LR). If not, match interrupt is not occurred and PWM output waveform is always 1.

Channel-7 Limit Register (PWMCH7LR)

Base address : 0x4000_5700

Address offset : 0x1C

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] LR – Limit Register

Limit Register. The LR can generate an overflow interrupt and PWM output waveform becomes 1 when the TC is reached to the LR. Match register should be smaller than limit register(LR). If not, match interrupt is not occurred and PWM output waveform is always 1.

Channel-7 Up-Down Mode Register (PWMCH7UDMR)

Base address : 0x4000_5700

Address offset : 0x20

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | UDM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] UDM – Up-Down mode

0 : TC runs up count.

1 : TC runs down count.

Channel-7 Timer/Counter Mode Register (PWMCH7TCMR)

Base address : 0x4000_5700

Address offset : 0x24

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | TCM |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] TCM – Timer/Counter mode

00 : Timer mode.

01 : Counter mode with counting driven by rising edge external input .

10 : Counter mode with counting driven by falling edge external input.

11 : Counter mode with counting driven by rising and falling edge external input.

Channel-7 PWM output Enable and External input Enable Register (PWMCH7PEEER)

Base address : 0x4000_5700

Address offset : 0x28

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | PEEE |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[1:0] PEEE – PWM output Enable and External input Enable

00 : PWM output is disable and external input is disable.

01 : PWM output is disable and external input is enable.

10 : PWM output is enable and external input is disable.

Channel-7 Capture Mode Register (PWMCH7CMR)

Base address : 0x4000_5700

Address offset : 0x2C

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | CM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] CM – Capture mode

0 : Timer/Counter is captured when external input signal is rising edge.

1 : Timer/Counter is captured when external input signal is falling edge.

Channel-7 Capture Register (PWMCH7CR)

Base address : 0x4000_5700

Address offset : 0x30

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] CR – Capture Register

Capture register. The CR is loaded with the value of the TC when external input signal is triggered.

Channel-7 Periodic Mode Register (PWMCH7PDMR)

Base address : 0x4000_5700

Address offset : 0x34

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | PDM |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] PDM – Periodic Mode

0 : Periodic mode. When the TC is reached to the LR, the TC returns to 0 and then continues counting periodically.

1 : One-shot mode. When the TC is reached to the LR, the TC returns to 0 and then stops counting.

Channel-7 Dead Zone Enable Register (PWMCH7DZER)

Base address : 0x4000_5700

Address offset : 0x38

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | DZE |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |

[0] DZE – Dead Zone Enable

0 : Dead zone generation is disabled.

1 : Dead zone generation is enabled.

Channel-7 Dead Zone Counter Register (PWMCH7DZCR)

Base address : 0x4000_5700

Address offset : 0x3C

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | DZC |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |  |  |

[9:0] DZC – Dead Zone Counter value

Dead zone generation counter value register. If the DZE bit in DZER is 1, dead zone counter counts to this value and during this time, the two PWM output waveforms are all 0.

Register map

The following Table 24 summarizes the PWM Channel-5 registers.

Table 24 PWM channel 7 register map and reset values

PWM Common Registers (Base address : 0x4000_5800)

Interrupt Enable Register (IER)

Base address : 0x4000_5800

Address offset : 0x00

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | IE7 | IE6 | IE5 | IE4 | IE3 | IE2 | IE1 | IE0 |
|  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

[0] IE0 – Channel 0 Interrupt Enable

0 : Channel 0 interrupt is disabled.

1 : Channel 0 interrupt is enabled.

[1] IE1 – Channel 1 Interrupt Enable

0 : Channel 1 interrupt is disabled.

1 : Channel 1 interrupt is enabled.

[2] IE2 – Channel 2 Interrupt Enable

0 : Channel 2 interrupt is disabled.

1 : Channel 2 interrupt is enabled.

[3] IE3 – Channel 3 Interrupt Enable

0 : Channel 3 interrupt is disabled.

1 : Channel 3 interrupt is enabled.

[4] IE4 – Channel 4 Interrupt Enable

0 : Channel 4 interrupt is disabled.

1 : Channel 4 interrupt is enabled.

[5] IE5 – Channel 5 Interrupt Enable

0 : Channel 5 interrupt is disabled.

1 : Channel 5 interrupt is enabled.

[6] IE6 – Channel 6 Interrupt Enable

0 : Channel 6 interrupt is disabled.

1 : Channel 6 interrupt is enabled.

[7] IE7 – Channel 7 Interrupt Enable

0 : Channel 7 interrupt is disabled.

1 : Channel 7 interrupt is enabled.

Start/Stop Register (SSR)

Base address : 0x4000_5800

Address offset : 0x04

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | SS7 | SS6 | SS5 | SS4 | SS3 | SS2 | SS1 | SS0 |
|  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

[0] SS0 – Channel 0 Timer/Counter Start or Stop.

0 : Timer/Counter stop.

1 : Timer/Counter start.

[1] SS1 – Channel 1 Timer/Counter Start or Stop.

0 : Timer/Counter stop.

1 : Timer/Counter start.

[2] SS2 – Channel 2 Timer/Counter Start or Stop.

0 : Timer/Counter stop.

1 : Timer/Counter start.

[3] SS3 – Channel 3 Timer/Counter Start or Stop.

0 : Timer/Counter stop.

1 : Timer/Counter start.

[4] SS4 – Channel 4 Timer/Counter Start or Stop.

0 : Timer/Counter stop.

1 : Timer/Counter start.

[5] SS5 – Channel 5 Timer/Counter Start or Stop.

0 : Timer/Counter stop.

1 : Timer/Counter start.

[6] SS6 – Channel 6 Timer/Counter Start or Stop.

0 : Timer/Counter stop.

1 : Timer/Counter start.

[7] SS7 – Channel 7 Timer/Counter Start or Stop.

0 : Timer/Counter stop.

1 : Timer/Counter start.

Pause Register (PSR)

Base address : 0x4000_5800

Address offset : 0x04

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | PS7 | PS6 | PS5 | PS4 | PS3 | PS2 | PS1 | PS0 |
|  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

The Timer/Counter is paused after TC is reached to value of limit register.

[0] PS0 – Channel 0 Timer/Counter Pause.

0 : Timer/Counter is not paused.

1 : Timer/Counter is paused.

[1] PS1 – Channel 1 Timer/Counter Pause.

0 : Timer/Counter is not paused.

1 : Timer/Counter is paused.

[2] PS2 – Channel 2 Timer/Counter Pause.

0 : Timer/Counter is not paused.

1 : Timer/Counter is paused.

[3] PS3 – Channel 3 Timer/Counter Pause.

0 : Timer/Counter is not paused.

1 : Timer/Counter is paused.

[4] PS0 – Channel 4 Timer/Counter Pause.

0 : Timer/Counter is not paused.

1 : Timer/Counter is paused.

[5] PS0 – Channel 5 Timer/Counter Pause.

0 : Timer/Counter is not paused.

1 : Timer/Counter is paused.

[6] PS0 – Channel 6 Timer/Counter Pause.

0 : Timer/Counter is not paused.

1 : Timer/Counter is paused.

[7] PS0 – Channel 7 Timer/Counter Pause.

0 : Timer/Counter is not paused.

1 : Timer/Counter is paused.

Register map

The following Table 25 summarizes the PWM Common registers.

Table 25 PWM common register map and reset values

- Dual timers

Introduction

The dual timer consists two programmable 32-bit or 16-bit Free-running counters(FRCs) that can generate interrupts when they reach 0. There are two dual timers and 4 FRCs. One dual timers has one interrupt handler, resulting in two interrupts of timers. Also one dual timer has one clock but two clock enable signals. Users can select one repetition modes one-shot or wrapping mode, and wrapping mode consists free-running and periodic mode. Two FRCs are one set so two FRCs has one clock, reset, and interrupt but each FRC has an individual clock enable.

Features

- One dual timer has two Free-Running Counters(FRCs).

- One dual timer has one interrupt handler and one clock.

- One dual timer has two clock enable signals.

- There are 2 dual timers.

- A 32-bit or a 16-bit down counter.

- One of the following repetition modes: one-shot and wrapping mode.

- One of the following wrapping modes: Free-running and periodic mode.

- There is a prescaler that can divide down the clock rate by 1, 16, or 256.

Figure 39 Block diagram of Dualtimer

Functional description

Clock and clock enable

The dual timers contain PCLK and TIMERCLK clock inputs. PCLK is the main APB system clock and is used by the register interface. TIMERCLK is the input to the prescale units and the decrementing counters. PCLK and TIMERCLK are synchronous.

The dual timers consist two programmable 32-bit Free-Running Counters(FRC) which operate independently. The two timers operate from one TIMERCLK but Each FRC is controlled independently by individual clock enable.

Timer size

Users can select FRC as 16-bit or 32-bit using the control register.

Prescaler

The timer has a prescaler that can divide down the enabled clock rate by 1, 16 or 256.

Repetition mode

There are two repetition mode: one-shot and wrapping mode. Wrapping mode has two modes: free-running and periodic mode.

One-shot mode

The counter generates an interrupt once. When the counter reaches 0, it halts until users reprogram it. Users can do this as below:

      - Clear the one-shot count bit in the control register, in which case the count proceeds according to the selection of wrapping mode(free-running or periodic mode).

      - Write a new value to the Load Value register.

Wrapping mode

Free-running mode

The counter wraps after reaching its zero value, and continues to count down from the maximum value. This is the default mode.

Periodic mode

The counter generates an interrupt at a constant interval, reloading the original value after wrapping past zero.

Interrupt

An interrupt is generated when the counter reaches 0 and is only cleared when the interrupt clear register is accessed.

The register holds the value until the interrupt is cleared.

Users can mask interrupts by writing 0 to the Interrupt Enable bit in the control register. Users can read the following from status registers:

- Raw interrupt status before masking.

- Final interrupt status after masking.

The interrupts from the individual timers after masking are logically ORed into a combined interrupt.

Operation

The operation of each timer is identical. The timer is loaded by writing to the load register and counts down to 0 if enabled. When a counter is already running, writing to the load register causes the counter to immediately restart at the new value. Writing to the background load value has no effect on the current count. In periodic mode, the counter continues to decrease to 0 and restart from the new load value.

An interrupt is generated when 0 is reached. Users can clear the interrupt by writing to the clear register. If users select one-shot mode, the counter halts when it reaches 0 until users deselect one-shot mode or write a new load value.

Otherwise, after reaching a zero count, if the timer is operating in free-running mode, it continues to decrease from its maximum value. If users select periodic mode, the timer reloads the count value from the load register and continues to decrease. In this mode, the counter effectively generates a periodic interrupt.

How to set the dual timers

Figure 40 The Dual timer setting flow

Dual timer0_0 Registers (Base address : 0x4000_1000)

Timer0_0 Load Register(DUALTIMER0_0TimerLoad)

Base address : 0x4000_1000

Address offset : 0x00

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TLR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] TLR – Timer Load Register

This register contains the value from which the counter is to decrement. This is the value used to reload the counter when Periodic mode is enabled, and the current count reaches 0.

Timer0_0 Value Register(DUALTIMER0_0TimerValue)

Base address : 0x4000_1000

Address offset : 0x04

Reset value : 0xFFFF_FFFF

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TVR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] TVR – Timer Value Register

This register provides the current value of the decrementing counter.

Timer0_0 Control Register(DUALTIMER0_0TimerControl)

Base address : 0x4000_1000

Address offset : 0x08

Reset value : 0x0000_0020

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | TE | TM | IE | res | TP |  | TS | OC |
|  |  |  |  |  |  |  |  | R/W | R/W | R/W |  | R/W |  | R/W | R/W |

[0] OC – One-shot Count

0 : Wrapping mode, default.

1 : One-shot mode.

[1] TS – Timer Size

0 : 16-bit counter, default.

1 : 32-bit counter.

[3:2] TP – Timer Prescale.

00 : 0 stages of prescale, clock is divided by 1, default.

01 : 4 stages of prescale, clock is divided by 16.

10 : 8 stages of prescale, clock is divided by 256.

11 : Undefined, do not use.

[5] IE – Interrupt Enable.

0 : Timer Interrupt disable.

1 : Timer Interrupt enabled, default.

[6] TM – Timer Mode.

0 : Timer is in free-running mode, default.

1 : Timer is in periodic mode.

[7] TE – Timer Enable.

0 : Timer disabled, default.

1 : Timer enabled.

Timer0_0 Interrupt Clear Register (DUALTIMER0_0TimerIntClr)

Base address : 0x4000_1000

Address offset : 0x0C

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | TIC |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | W |

[0] TIC – Interrupt Clear

Set to this register clears the interrupt output from the counter.

Timer0_0 Raw Interrupt Status Register (DUALTIMER0_0TimerRIS)

Base address : 0x4000_1000

Address offset : 0x10

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | RIS |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |

[0] RIS – Raw Interrupt Status Register

This register indicates the raw interrupt status from the counter. This value is ANDed with the timer interrupt enable bit from the Timer Control Register to create the masked interrupt, that is passed to the interrupt output pin.

Timer0_0 Masked Interrupt Status Register (DUALTIMER0_0TimerMIS)

Base address : 0x4000_1000

Address offset : 0x14

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | MIS |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |

[0] MIS – Masked Interrupt Status Register

This register indicates the masked interrupt status from the counter. This value is the logical AND of the raw interrupt status with the timer interrupt enable bit from the Timer Control Register, and is the same value that is passed to the interrupt output pin.

Timer0_0 Background Load Register (DUALTIMER0_0TimerBGLoad)

Base address : 0x4000_1000

Address offset : 0x18

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BGL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] BGL – Background Load Register

This register contains the value from which the counter is to decrement. This is the value used to reload the counter when Periodic mode is enabled, and the current count reaches 0.

This register provides an alternative method of accessing the TimerLoad Register. The difference is that writes to TimerBGLoad Register do not cause the counter to immediately restart from the new value.

Reading from this register returns the same value returned from TimerLoad Register.

Register map

The following Table 26 summarizes the Dual timer 0_0 registers.

Table 26 Dual timer 0_0 register map and reset values

Dual timer0_1 Registers (Base address : 0x4000_1020)

Timer0_1 Load Register(DUALTIMER0_1TimerLoad)

Base address : 0x4000_1020

Address offset : 0x00

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TLR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] TLR – Timer Load Register

This register contains the value from which the counter is to decrement. This is the value used to reload the counter when Periodic mode is enabled, and the current count reaches 0.

Timer0_1 Value Register(DUALTIMER0_1TimerValue)

Base address : 0x4000_1020

Address offset : 0x04

Reset value : 0xFFFF_FFFF

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TVR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] TVR – Timer Value Register

This register provides the current value of the decrementing counter.

Timer0_1 Control Register(DUALTIMER0_1TimerControl)

Base address : 0x4000_1020

Address offset : 0x08

Reset value : 0x0000_0020

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | TE | TM | IE | res | TP |  | TS | OC |
|  |  |  |  |  |  |  |  | R/W | R/W | R/W |  | R/W |  | R/W | R/W |

[0] OC – One-shot Count

0 : Wrapping mode, default.

1 : One-shot mode.

[1] TS – Timer Size

0 : 16-bit counter, default.

1 : 32-bit counter.

[3:2] TP – Timer Prescale.

00 : 0 stages of prescale, clock is divided by 1, default.

01 : 4 stages of prescale, clock is divided by 16.

10 : 8 stages of prescale, clock is divided by 256.

11 : Undefined, do not use.

[5] IE – Interrupt Enable.

0 : Timer Interrupt disable.

1 : Timer Interrupt enabled, default.

[6] TM – Timer Mode.

0 : Timer is in free-running mode, default.

1 : Timer is in periodic mode.

[7] TE – Timer Enable.

0 : Timer disabled, default.

1 : Timer enabled.

Timer0_1 Interrupt Clear Register (DUALTIMER0_1TimerIntClr)

Base address : 0x4000_1020

Address offset : 0x0C

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | IC |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | W |

[0] IC – Interrupt Clear

Set to the this register clears the interrupt output from the counter.

Timer0_1 Raw Interrupt Status Register (DUALTIMER0_1TimerRIS)

Base address : 0x4000_1020

Address offset : 0x10

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | RIS |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |

[0] RIS – Raw Interrupt Status Register

This register indicates the raw interrupt status from the counter. This value is ANDed with the timer interrupt enable bit from the Timer Control Register to create the masked interrupt, that is passed to the interrupt output pin.

Timer0_1 Masked Interrupt Status Register (DUALTIMER0_1TimerMIS)

Base address : 0x4000_1020

Address offset : 0x14

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | MIS |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |

[0] MIS – Masked Interrupt Status Register

This register indicates the masked interrupt status from the counter. This value is the logical AND of the raw interrupt status with the timer interrupt enable bit from the Timer Control Register, and is the same value that is passed to the interrupt output pin.

Timer0_1 Background Load Register (DUALTIMER0_1TimerBGLoad)

Base address : 0x4000_1020

Address offset : 0x18

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BGL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] BGL – Background Load Register

This register contains the value from which the counter is to decrement. This is the value used to reload the counter when Periodic mode is enabled, and the current count reaches 0.

This register provides an alternative method of accessing the TimerLoad Register. The difference is that writes to TimerBGLoad Register do not cause the counter to immediately restart from the new value.

Reading from this register returns the same value returned from TimerLoad Register.

Register map

The following Table 27 summarizes the Dual timer 0_1 registers.

Table 27 Dual timer 0_1 register map and reset values

Dual Timer 0 Clock Enable Register (Base address : 0x4000_1080)

Timer0_0 Clock Enable Register (TIMCLKEN0_0)

Base address : 0x4000_1080

Address offset : 0x00

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | CE |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[0] CE – Clock Enable Register

0 : Clock disable

1 : Clock enable

Timer0_1 Clock Enable Register (TIMCLKEN0_1)

Base address : 0x4000_1080

Address offset : 0x20

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | CE |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[0] CE – Clock Enable Register

0 : Clock disable

1 : Clock enable

Register map

The following Table 28 summarizes the Dual timer 0 registers.

Table 28 Dual timer 0 clock enable register map and reset values

Dual timer1_0 Registers (Base address : 0x4000_2000)

Timer1_0 Load Register(DUALTIMER1_0TimerLoad)

Base address : 0x4000_2000

Address offset : 0x00

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TLR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] TLR – Timer Load Register

This register contains the value from which the counter is to decrement. This is the value used to reload the counter when Periodic mode is enabled, and the current count reaches 0.

Timer1_0 Value Register(DUALTIMER1_0TimerValue)

Base address : 0x4000_2000

Address offset : 0x04

Reset value : 0xFFFF_FFFF

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TVR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] TVR – Timer Value Register

This register provides the current value of the decrementing counter.

Timer1_0 Control Register(DUALTIMER1_0TimerControl)

Base address : 0x4000_2000

Address offset : 0x08

Reset value : 0x0000_0020

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | TE | TM | IE | res | TP |  | TS | OC |
|  |  |  |  |  |  |  |  | R/W | R/W | R/W |  | R/W |  | R/W | R/W |

[0] OC – One-shot Count

0 : Wrapping mode, default.

1 : One-shot mode.

[1] TS – Timer Size

0 : 16-bit counter, default.

1 : 32-bit counter.

[3:2] TP – Timer Prescale.

00 : 0 stages of prescale, clock is divided by 1, default.

01 : 4 stages of prescale, clock is divided by 16.

10 : 8 stages of prescale, clock is divided by 256.

11 : Undefined, do not use.

[5] IE – Interrupt Enable.

0 : Timer Interrupt disable.

1 : Timer Interrupt enabled, default.

[6] TM – Timer Mode.

0 : Timer is in free-running mode, default.

1 : Timer is in periodic mode.

[7] TE – Timer Enable.

0 : Timer disabled, default.

1 : Timer enabled.

Timer1_0 Interrupt Clear Register (DUALTIMER1_0TimerIntClr)

Base address : 0x4000_2000

Address offset : 0x0C

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | IC |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | W |

[0] IC – Interrupt Clear

Set to the this register clears the interrupt output from the counter.

Timer1_0 Raw Interrupt Status Register (DUALTIMER1_0TimerRIS)

Base address : 0x4000_2000

Address offset : 0x10

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | RIS |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |

[0] RIS – Raw Interrupt Status Register

This register indicates the raw interrupt status from the counter. This value is ANDed with the timer interrupt enable bit from the Timer Control Register to create the masked interrupt, that is passed to the interrupt output pin.

Timer1_0 Masked Interrupt Status Register (DUALTIMER1_0TimerMIS)

Base address : 0x4000_2000

Address offset : 0x14

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | MIS |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |

[0] MIS – Masked Interrupt Status Register

This register indicates the masked interrupt status from the counter. This value is the logical AND of the raw interrupt status with the timer interrupt enable bit from the Timer Control Register, and is the same value that is passed to the interrupt output pin.

Timer1_0 Background Load Register (DUALTIMER1_0TimerBGLoad)

Base address : 0x4000_2000

Address offset : 0x18

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BGL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] BGL – Background Load Register

This register contains the value from which the counter is to decrement. This is the value used to reload the counter when Periodic mode is enabled, and the current count reaches 0.

This register provides an alternative method of accessing the TimerLoad Register. The difference is that writes to TimerBGLoad Register do not cause the counter to immediately restart from the new value.

Reading from this register returns the same value returned from TimerLoad Register.

Register map

The following Table 29 summarizes the Dual timer 1_0 registers.

Table 29 Dual timer 1_0 register map and reset values

Dual timer1_1 Registers (Base address : 0x4000_2020)

Timer1_1 Load Register(DUALTIMER1_1TimerLoad)

Base address : 0x4000_2020

Address offset : 0x00

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TLR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] TLR – Timer Load Register

This register contains the value from which the counter is to decrement. This is the value used to reload the counter when Periodic mode is enabled, and the current count reaches 0.

Timer1_1 Value Register(DUALTIMER1_1TimerValue)

Base address : 0x4000_2020

Address offset : 0x04

Reset value : 0xFFFF_FFFF

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TVR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] TVR – Timer Value Register

This register provides the current value of the decrementing counter.

Timer1_1 Control Register(DUALTIMER1_1TimerControl)

Base address : 0x4000_2020

Address offset : 0x08

Reset value : 0x0000_0020

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | TE | TM | IE | res | TP |  | TS | OC |
|  |  |  |  |  |  |  |  | R/W | R/W | R/W |  | R/W |  | R/W | R/W |

[0] OC – One-shot Count

0 : Wrapping mode, default.

1 : One-shot mode.

[1] TS – Timer Size

0 : 16-bit counter, default.

1 : 32-bit counter.

[3:2] TP – Timer Prescale.

00 : 0 stages of prescale, clock is divided by 1, default.

01 : 4 stages of prescale, clock is divided by 16.

10 : 8 stages of prescale, clock is divided by 256.

11 : Undefined, do not use.

[5] IE – Interrupt Enable.

0 : Timer Interrupt disable.

1 : Timer Interrupt enabled, default.

[6] TM – Timer Mode.

0 : Timer is in free-running mode, default.

1 : Timer is in periodic mode.

[7] TE – Timer Enable.

0 : Timer disabled, default.

1 : Timer enabled.

Timer1_1 Interrupt Clear Register (DUALTIMER1_1TimerIntClr)

Base address : 0x4000_2020

Address offset : 0x0C

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | IC |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | W |

[0] IC – Interrupt Clear

Set to the this register clears the interrupt output from the counter.

Timer1_1 Raw Interrupt Status Register (DUALTIMER1_1TimerRIS)

Base address : 0x4000_2020

Address offset : 0x10

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | RIS |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |

[0] RIS – Raw Interrupt Status Register

This register indicates the raw interrupt status from the counter. This value is ANDed with the timer interrupt enable bit from the Timer Control Register to create the masked interrupt, that is passed to the interrupt output pin.

Timer1_1 Masked Interrupt Status Register (DUALTIMER1_1TimerMIS)

Base address : 0x4000_2020

Address offset : 0x14

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | MIS |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |

[0] MIS – Masked Interrupt Status Register

This register indicates the masked interrupt status from the counter. This value is the logical AND of the raw interrupt status with the timer interrupt enable bit from the Timer Control Register, and is the same value that is passed to the interrupt output pin.

Timer1_1 Background Load Register (DUALTIMER1_1TimerBGLoad)

Base address : 0x4000_2020

Address offset : 0x18

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BGL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] BGL – Background Load Register

This register contains the value from which the counter is to decrement. This is the value used to reload the counter when Periodic mode is enabled, and the current count reaches 0.

This register provides an alternative method of accessing the TimerLoad Register. The difference is that writes to TimerBGLoad Register do not cause the counter to immediately restart from the new value.

Reading from this register returns the same value returned from TimerLoad Register.

Register map

The following Table 30 summarizes the Dual timer 1_1 registers.

Table 30 Dual timer 1_1 register map and reset values

Dual Timer 1 Clock Enable Register (Base address : 0x4000_2080)

Timer1_0 Clock Enable Register (TIMCLKEN1_0)

Base address : 0x4000_2080

Address offset : 0x00

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | CE |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[0] CE – Clock Enable Register

0 : Clock disable

1 : Clock enable

Timer1_1 Clock Enable Register (TIMCLKEN1_1)

Base address : 0x4000_2080

Address offset : 0x20

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | CE |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |

[0] CE – Clock Enable Register

0 : Clock disable

1 : Clock enable

Register map

The following Table 31 summarizes the Dual timer 1 registers.

Table 31 Dual timer 1 clock enable register map and reset values

- Watchdog timer

Introduction

The watchdog is based on a 32-bit down-counter that is initialized from the Reload Register, WDTLoad. The watchdog generates a regular interrupt depending on a programmed value. The counter decreases by one on each positive clock edge of watchdog clock.

The watchdog monitors the interrupt and asserts a reset request signal when the counter reaches 0 and the counter is stopped. On the next enabled watchdog clock edge, the counter is reloaded from the WDTLoad Register and the countdown sequence continues. The watchdog reasserts the reset signal if the interrupt is not cleared by the time the counter next reaches 0.

The watchdog applies a reset to a system in the event of a software failure to provide a way to recover from software crashes. Users can enable or disable the watchdog unit as required.

Features

- 32-bit down counter.

- Internally resets chip if not periodically reloaded.

- The watchdog timer has lock register to prevent rogue software from disabling the watchdog timer functionality.

- The watchdog timer clock(WDTCLK) and system clock(PCLK) are synchronous.

Functional description

Clock

The watchdog timer contains PCLK and WDTCLK clock inputs.

PCLK is the main APB system clock and is used by the register interface.

Interrupt and reset request

An interrupt is generated when the counter reaches 0 and is only cleared when the interrupt clear register is accessed.

The register holds the value until the interrupt is cleared.

Reset request is asserted when the counter reaches 0 repeatedly and is not reprogrammed.

Users can mask interrupts by writing 0 to the Interrupt Enable bit in the control register. Users can read the following from status registers:

    - Raw interrupt status, before masking.

    - Final interrupt status, after masking.

Figure 41 Watchdog timer operation flow diagram

Watchdog timer Registers (Base address : 0x4000_0000)

Watchdog timer Load Register(WDTLoad)

Address offset : 0x000

Reset value : 0xFFFF_FFFF

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WLR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] WLR – Watchdog timer Load Register.

This register contains the value from which the counter is to decrement. When this register is written to, the count is immediately restarted from the new value. The minimum valid value for WDTLoad is 1.

Watchdog timer Value Register(WDTValue)

Address offset : 0x004

Reset value : 0xFFFF_FFFF

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WVR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[31:0] WVR – Watchdog timer Value Register.

This register gives the current value of the decrementing counter.

Watchdog timer Control Register(WDTControl)

Address offset : 0x008

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | REN | IEN |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W |

[0] IEN – Interrupt Enable.

0 : Disable the counter and the interrupt.

1 : Enable the counter and the interrupt. Reloads the counter from the value in WDTLoad, after previously being disabled.

[1] REN – Reset Request Enable.

0 : Disable watchdog reset output.

1 : Enable watchdog reset output.

Watchdog timer Interrupt Clear Register (WDTIntClr)

Address offset : 0x00C

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | WIC |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | W |

[0] WIC – Watchdog timer Interrupt Clear

A write of 1 to this register clears the watchdog interrupt, and reloads the counter from the value in WDTLoad.

Watchdog timer Raw Interrupt Status Register (WDTRIS)

Address offset : 0x010

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | RIS |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |

[0] RIS – Watchdog timer Raw Interrupt Status.

This register indicates the raw interrupt status from the counter. This value is ANDed with the interrupt enable bit from the control register to create the masked interrupt, that is passed to the interrupt output pin.

Watchdog timer Masked Interrupt Status Register (WDTMIS)

Address offset : 0x014

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| res | res | res | res | res | res | res | res | res | res | res |  | res |  | res |  | res |  | MIS |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |

[0] MIS – Watchdog timer Masked Interrupt Status.

This register indicates the masked interrupt status from the counter. This value is the logical AND of the raw interrupt status with the interrupt enable(IEN) bit from the control register, and is the same value that is passed to the interrupt output pin.

Watchdog timer Lock Register(WDTLock)

Address offset : 0xC00

Reset value : 0x0000_0000

| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ERW |  |  |  |  |  |  |  |  |  |  |  |  |  |  | WES |
| W |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |

This register disables write accesses to all other registers. This is to prevent rogue software from disabling the watchdog functionality. Writing a value of 0x1ACCE551 enables write access to all other registers. Writing any other value disables write accesses. A read from this register returns only the bottom bit.

[0] WES – Register Write Enable status.

0 : Indicates that write access is enabled, not locked. Default.

1 : Indicates that write access is disabled, locked.

[31:1] ERW – Enable Register Writes

Enable write access to all other registers by writing 0x1ACCE551. Disable write access by writing any other value.

Register map

The following Table 32 summarizes the Watchdog timer registers.

Table 32 Watchdog Timer register map and reset values

- Real-time Clock(RTC)

Introduction

The real-timer clock (RTC) is an independent BCD timer/counter. The RTC provides a timer-of-day clock/calendar with programmable alarm interrupt.

Features

Calendar with year, month, day, weekday, hours, minutes, and seconds

Programmable alarm with interrupt function

Functional description

Figure 46 shows the RTC block diagram.

Figure 42. RTC block diagram

RTC clock

RTC Clock (RTCCLK) can be selected among several clocks (32768Hz oscillator, MCLK, RCLK, OCLK). Please refer to the Clock Reset Generator chapter for configuring the clock.

If the 32768Hz oscillator clock is used, the divider generates 1 Hz clock internally.

If the DIVRST (Bit[1] of RTC Control Register ) value is high, the RTC Divider is cleared. If the DIVRST value is low, the divider operates.

RTC interrupt

RTC has two kinds of interrupt source, Counter Interrupt and Alarm Interrupt.

Interrupts are enabled in the NVIC using the appropriate Interrupt Set Enable register.

There are following Counter Interrupts: Second, Minute, Hour, Day (Day of Week), Date (Day of Month) and Year.

Each bit of RTCINTM (Interrupt Mask Register) can disable or enable interrupt for each Counter Interrupt.

Alarm Interrupt can be generated when the Alarm matches with Counter.

RTC Counter and Calendar

When user writes 1 to bit 0 of control register, then the RTC counter runs according to the RTC Clock.

Timer value should be written as the BCD format.

The RTC counter operates for Calendar function including Day, Month, Year and Leap Year calculation.

RTC Setting flow

RTC setting sequences are shown in flow chart.

Figure 47 shows RTC setting flow for Counter Function.

Figure 48 shows RTC setting flow for Alarm function.

Figure 43. RTC setting flow for Counter Function

Figure 44. RTC setting flow for Alarm function

RTC Registers (Base Address : 0x4000_E000)

In these descriptions, for most of the registers the Reset Value shows "NC", meaning that these registers are Not Changed by a Reset. Software must initialize these registers between power-on and setting the RTC into operation.

RTC control register (RTCCON)

Address offset: 0x0000

Reset value: 0x0000_0000

| 31 |  | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  |  |  | 22 | 21 |  | 20 |  |  | 19 |  |  | 18 | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  |  |  | res | res |  | res |  |  | res |  |  | res | res |  | res |
| 15 | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  |  | 4 |  | 3 | 2 |  |  | 1 |  |  | 0 |  |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | INTEN |  |  | res |  | res | res |  |  | DIVRST |  |  | CLKEN |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |  | R/W |  |  | R/W |  |

[0] CLKEN – Clock Enable

This bit written by S/W to enable or disable clock

0 : The time counters are disabled (clock stop)

1 : The time counters are enabled (clock enable)

[1] DIVRST – RTC Divider Reset

This bit set and cleared by S/W to reset RTC divider.

0 : No effect

1 : When 1, divider reset and remain reset until RTCCON[1] is changed to 0.

[5] INTEN – RTC Interrupt Enable

This bit set and cleared by S/W to enable or disable RTC interrupt.

0 : RTC Interrupt is disabled

1 : RTC Interrupt is enabled

RTC Interrupt Mask register (RTCINTE)

Address offset: 0x0004

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | AINT | IMMON | IMDAY | IMDATE | IMHOUR | IMMIN | IMSEC |
|  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

[0] IMSEC – RTC Second Interrupt Enable

This bit set and cleared by S/W to enable or disable RTC Second interrupt.

0 : No effect

1 : an increment of the Second value generates an interrupt

[1] IMMIN – RTC Minute Interrupt Enable

This bit set and cleared by S/W to enable or disable RTC Minute interrupt.

0 : No effect

1 : an increment of the Minute value generates an interrupt

[2] IMHOUR – RTC Hour Interrupt Enable

This bit set and cleared by S/W to enable or disable RTC Hour interrupt.

0 : No effect

1 : an increment of the Hour value generates an interrupt

[3] IMDATE – RTC Date Interrupt Enable

This bit set and cleared by S/W to enable or disable RTC Date interrupt.

0 : No effect

1 : an increment of the Date value generates an interrupt

[4] IMDAY – RTC Day Interrupt Enable

This bit set and cleared by S/W to enable or disable RTC Day interrupt.

0 : No effect

1 : an increment of the Day value generates an interrupt

[5] IMMON – RTC Month Interrupt Enable

This bit set and cleared by S/W to enable or disable RTC Month interrupt.

0 : No effect

1 : an increment of the Month value generates an interrupt

[6] AINT – RTC Alarm Interrupt Enable

This bit set and cleared by S/W to enable or disable RTC Alarm interrupt.

0 : No effect

1 : Alarm interrupt enabled

RTC Interrupt Pending register (RTCINTP)

Address offset: 0x0008

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | RTCALF | RTCCIF |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W |

Bit [0] RTCCIF – RTC Counter Interrupt pending flag

When one, the Counter Increment Interrupt block generated an interrupt. Writing a one to this bit clears the counter increment interrupt.

Read 0 : No effect

Read 1 : an increment of the Second value generates an interrupt

Write 0 : No effect

Write 1 : clear interrupt

Bit [1] RTCALF – RTC Alarm interrupt pending flag

When one, the alarm registers generated an interrupt. Writing a one to this bit clears the alarm interrupt.

Read 0 : No effect

Read 1 : an increment of the Minute value generates an interrupt

Write 0 : No effect

Write 1 : clear interrupt

RTC Alarm Mask register (RTCAMR)

Address offset: 0x000C

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | AMRYEAR | AMRMON | AMRDATE | AMRDAY | AMRHOUR | AMRMIN | AMRSEC |
|  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

[0] AMRSEC – RTC Alarm Mask for Second

This bit set and cleared by S/W to enable or disable comparison for Second

0 : No effect

1 : the Second value is compared to the Predetermining Second for the alarm

[1] AMRMIN – RTC Alarm Mask for Minute

This bit set and cleared by S/W to enable or disable comparison for Minute

0 : No effect

1 : the Minute value is compared to the Predetermining Minute for the alarm

[2] AMRHOUR – RTC Alarm Mask for Hour

This bit set and cleared by S/W to enable or disable comparison for Minute

0 : No effect

1 : the Hour value is compared to the Predetermining Hour for the alarm

[3] AMRDAY – RTC Alarm Mask for Day

This bit set and cleared by S/W to enable or disable comparison for Day of Week

0 : No effect

1 : the Day value is compared to the Predetermining Day for the alarm

[4] AMRDATE – RTC Alarm Mask for Date

This bit set and cleared by S/W to enable or disable comparison for Date of Month

0 : No effect

1 : the Date value is compared to the Predetermining Date for the alarm

[5] AMRMON – RTC Alarm Mask for Month

This bit set and cleared by S/W to enable or disable comparison for Date of Month

0 : No effect

1 : the Month value is compared to the Predetermining Month for the alarm

[6] AMRYEAR – RTC Alarm Mask for Year

This bit set and cleared by S/W to enable or disable comparison for Date of Month

0 : No effect

1 : the Date value is compared to the Predetermining Date for the alarm

RTC BCD Second register (BCDSEC)

Address offset: 0x0010

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | Res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | BCDSEC |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

Bit [6:0] BCDSEC – RTC Seconds value (0 to 59)

RTC BCD Minute register (BCDMIN)

Address offset: 0x0014

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 |  | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res |  | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res |  | res | BCDMIN |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

Bit [6:0] BCDMIN – RTC Minute value (0 to 59)

RTC BCD Hour register (BCDHOUR)

Address offset: 0x0018

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | BCDHOUR |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W |

Bit [5:0] BCDHOUR – RTC Hour value (0 to 23)

RTC BCD Day register (BCDDAY)

Address offset: 0x001C

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | BCDDAY |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W |

Bit [3:0] BCDDAY – RTC Day of Week value (1 to 7)

RTC BCD Date register (BCDDATE)

Address offset: 0x0020

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | BCDDATE |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W |

Bit [5:0] BCDDATE – RTC Day of Month value (1 to 28, 29, 30, or 31)

RTC BCD Month register (BCDMON)

Address offset: 0x0024

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | BCDMON |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W |

Bit [4:0] BCDMON – RTC Month value (1 to 12)

RTC BCD Year register (BCDYEAR)

Address offset: 0x0028

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 |  | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res |  | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 |  | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
|  | BCDYEAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

Bit [15:0] BCDYEAR – RTC Year value (0 to 4095)

RTC Predetermining Second register (PRESEC)

Address offset: 0x002C

Reset value : 0xXXXX_XXXX, NC

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | PRESEC |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

Bit [6:0] PRESEC – RTC PREDETERMINING Seconds value (0 to 59)

RTC Predetermining Minute register (PREMIN)

Address offset: 0x0030

Reset value : 0xXXXX_XXXX, NC

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | PREMIN |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

Bit [6:0] PREMIN – RTC Predetermining Minute value (0 to 59)

RTC Predetermining Hour register (PREHOUR)

Address offset: 0x0034

Reset value : 0xXXXX_XXXX, NC

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | PREHOUR |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W |

Bit [5:0] PREHOUR – RTC Predetermining Hour value (0 to 23)

RTC Predetermining Day register (PREDAY)

Address offset: 0x0038

Reset value : 0xXXXX_XXXX, NC

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | PREDAY |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W |

Bit [3:0] PREDAY – RTC Predetermining Day of Week value (1 to 7)

RTC Predetermining Date register (PREDATE)

Address offset: 0x003C

Reset value : 0xXXXX_XXXX, NC

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | PREDATE |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W |

Bit [5:0] PREDATE – RTC Predetermining Day of Month value (1 to 28, 29, 30, or 31)

RTC Predetermining Month register (PREMON)

Address offset: 0x0040

Reset value : 0xXXXX_XXXX, NC

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | PREMON |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W |

Bit [4:0] PREMON – RTC Predetermining Month value (1 to 12)

RTC Predetermining Year register (PREYEAR)

Address offset: 0x0044

Reset value : 0xXXXX_XXXX, NC

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| PREYEAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

Bit [15:0] PREYEAR – RTC Predetermining Year value (0 to 4095)

RTC Consolidated Time0 register (RTCTIME0)

Address offset: 0x0048

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | CBCDDAY |  |  |  | res | res | CBCDHOUR |  |  |  |  |  |
|  |  |  |  | R | R | R | R |  |  | R | R | R | R | R | R |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | CBCDMIN |  |  |  |  |  |  | res | CBCDSEC |  |  |  |  |  |  |
|  | R | R | R | R | R | R | R |  | R | R | R | R | R | R | R |

Bit [6:0] CBCDSEC – RTC Consolidated Second value (0 to 59)

Bit [14:8] CBCDMIN – RTC Consolidated Minute value (0 to 59)

Bit [21:16] CBCDHOUR – RTC Consolidated Hour value (0 to 23)

Bit [27:24] CBCDDAY – RTC Consolidated Day of Week value (1 to 7)

RTC Consolidated Time1 register (RTCTIME1)

Address offset: 0x004C

Reset value : 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CBCDYEAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R | R | R | R | R | R | R | R | R | R | R | R | R | R | R | R |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | CBCDMON |  |  |  |  | res | res | CBCDDATE |  |  |  |  |  |
|  |  |  | R | R | R | R | R |  |  | R | R | R | R | R | R |

Bit [5:0] CBCDDATE – RTC Consolidated Day of Month value (1 to 28, 29, 30, or 31)

Bit [12:8] CBCDMON – RTC Consolidated Month value (1 to 12)

Bit [31:16] CBCDYEAR – RTC Consolidated Year value (0 to 4095)

Register map

Table 33 RTC register map and reset values

- UART(Universal Asynchronous Receive Transmit)

Introduction

The UART supports synchronous one-way communication, half-duplex single wire communication, and multiprocessor communications(CTS/RTS).

Features

- Serial-to-parallel conversion on data received from a peripheral device

- Parallel-to-serial conversion on data transmitted to the peripheral device

- Data size of 5,6,7 and 8 its

- One or two stop bits

- Even, odd, stick, or no-parity bit generation and detection

- Support of hardware flow control

- Programmable FIFO disabling for 1-byte depth.

- Programmable use of UART or IrDA SIR input/output

- False start bit detection

Functional description

UART bidirectional communication requires a minimum of two pins: RX, TX

The frame are comprised of:

- An Idle Line prior to transmission or reception

- A start bit

- A data word (8 or 9 bits) least significant bit first

- 1, 1.5, 2 Stop bits indicating that the frame is complete

- The USART interface uses a baud rate generator

- A status register (UART1_RISR)

- data registers (UART1DR)

- A baud rate register (UART1_IBRD,UART1_FBRD)

Figure 45. UART0,1 Block diagram

Figure 46 shows the UART character frame

Figure 46. UART character frame

Baud rate calculation

UARTx can operate with or without using the Fractional Divider. The baud rate divisor is a 22-bit number consisting the UARTxIBRD(16-bit integer) and the UARTxFBRD(6-bit fractional).

This is used by the baud rate generator to determine the bit period.

- Baud Rate Divisor =UARTCLK(16*baud rate)= BRDI+ BRDF

Figure 47. UART divider flow chart

Figure 48 show how to set the UART Initial value.

Figure 48. UART Initial setting flow chart

Data transmission

Data transmitted is stored in a 32-byte FIFOs. Transmit data is written into the transmit FIFO for transmission. If UART is enabled, it causes a data frame to start transmitting with parameters indicated in the UARTxLCR_H.

Data continues to transmit until there is no data left in the transmit FIFO. The BUSY bit of UARTxFR is ‘1’ as soon as data is written to the transmit FIFO, which means the FIFO is not empty, and remains as ‘1’ while data is being transmitted.

Data receive

Received data is stored in the 32-byte FIFOs. When a start bit has been received, it begins running and data is sampled on the eighth cycle of that counter in UART mode. A valid stop bit is confirmed if UARTRXD is ‘1’. When a full word is received, the data is stored in the receive FIFO. Error bit is stored in bit[10:8] of UARTxCR and overrun is stored in bit[11] of UARTxCR.

Figure 49. Transmit and Receive data flow chart

Hardware flow control

Figure 50. Hardware flow control description

The RTS flow control is enabled by setting the RTSen of UARTxCR. If RTS is enabled, the data flow is controlled as follows.

When the receiver FIFO level reaches the programmed trigger level, nUARTRTS(pin) is asserted(to a low value). nUARTRTS is reasserted(to a low level) once the receiver FIFO has reached the previous trigger level. The reasserted of nUARTRTS signals to the sending UART to continue transmitting data.

The CTS flow control is enabled, the transmitter can only transmit data when nUARTCTS is asserted. When nUARTCTR is re-asserted(to a low) the transmitter sends the next byte. To stop the transmitter from sending the following byte, nUARTCTS must be released before the middle of the last stop bit that is currently being sent.

Figure 51. CTS Functional Timing

Figure 52 shows how software should use the RTS/CTR.

Figure 52. Algorithm for setting CTS/RTS flowchart

UART0 Registers(Base address: 0x4000_C000)

UART0DR (UART0 Data Register)

Address offset: 0x000

Reset value: 0x0000_0000

The UART0DR is the data register.

The write operation initiates transmission from the UART. The data is prefixed with a start bit, appended with the appropriate parity bit(if parity is enabled), and a stop bit. The resultant word is then transmitted.

The received data byte is read by performing reads from the UARTDR register along with the corresponding status information.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | OE | BE | PE | FE | DATA |  |  |  |  |  |  |  |
|  |  |  |  | R | R | R | R | R/W |  |  |  |  |  |  |  |

[11] OE – Overrun error

0: data is empty

1: data is received and the receive FIFO is already full.

[10] BE – Break error

1: if a break condition was detected, indicating that the received data input was held LOW of longer than a full-word transmission time(defined as start, data, parity and stop bits)

[9] PE – Parity error

1: it indicates that the parity of the received, it indicates that the parity of the received data character does not match the parity that the EPS and SPS bits in the line control register, UARTLCR_H

[8] FE – Framing error

1: it indicates that the received

[7:0] DATA – Receive (READ)/Transmit (WRITE) data

UART0RSR/ECR (UART0 Receive Status Register/Error Clear Register)

Address offset: 0x004

Reset value: 0x0000_0000

The UART0RSR/ECR is the receive status register/error clear register.

Receive status can also be read from the UART0RSR register.

A write to the UART0ECR register clears the framing, parity, break, and overrun errors.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | OE | BE | PE | FE |

[3] OE – Overrun error

This bit is set to 1 if data is received and the FIFO is already full.

This bit is cleared to 0 by a write to UART0ECR

[2] BE – Break error

This bit is cleared to 0 by a write to UART0ECR

[1] PE – Parity error

When set to 1, it indicates that the parity of the received data character does not match

the parity that the EPS and SPS bits in the line control register, UARTLCR_H select

This bit is cleared to 0 by a write to UART0ECR

[0] FE – Framing error

When set to 1, in indicates that the received character didn’t have a valid stop bit

This bit is cleared to 0 by a write to UART0ECR

UART0FR (UART0 Flag Register)

Address offset: 0x0018

Reset value: 0bx11000xxx

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | RI | TXFE | RXFF | TXFF | RXFE | BUSY | DCD | DSR | CTS |
|  |  |  |  |  |  |  | R | R | R | R | R | R | R | R | R |

[8] RI – Ring indicator

This bit is the complement of the UART ring indicator, UART0RI.

1: When nUART0RI is LOW

[7] TXFE – Transmit FIFO empty

This bit depends on the state of the FEN bit in the line control register, UARTLCR_H.

0: The bit is set when transmit holding register is empty.

1: The bit is set when transmit FIFO is empty

[6] RXFF – Receive FIFO full

This bit depends on the state of the FEN bit in the line control register, UARTLCR_H.

0: The bit is set when the receive holding register is full

1: The bit is set when the receive FIFO is full

[5] TXFF – Transmit FIFO full

This bit depends on the state of the FEN bit in the line control register, UARTLCR_H.

0: The bit is set when transmit holding register is full.

1: The bit is set when transmit FIFO is full.

[4] RXFE – Receive FIFO empty

This bit depends on the state of the FEN bit in the line control register, UARTLCR_H.

0: The bit is set when the receive holding register is empty.

1: The bit is set when the receive FIFO is empty.

[3] BUSY – UART busy

1: the UART is busy transmitting data

[2] DCD – Data carrier detect

This bit is the complement of the UART data carrier detect, UART0DCD, status input

1: The bit is the complement of the UART data carrier detect

[1] DSR – Data set ready

This bit is the complement of the UART data set ready, UART0DSR, status input

1: The bit is the complement of the UART data set ready

[0] CTS – Clear to send

This bit is the complement of the UART clear to send, UART0CTS, status input

1: The bit is the complement of the UART clear to send

UART0ILPR (UART0 IrDA Low-Power Counter Register)

Address offset: 0x0020

Reset value: 0x00

The UARTILPR Register is the IrDA low-power counter register

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | ILPDVSR |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |

[7:0] ILPDVSR – 8-bit low-power divisor value

These bits are cleared to 0 at reset.

ILPDVSR=(FUARTCLK/FIrLPBaud16)

Where, FIrLPBaud16 is nominally 1.8432MHz

The divisor is 1.42MHz < FIrLPBaud16 < 2.12MHz, results in a low-power pulse duration of 1.41 – 2.11us.

UART0IBRD (UART0 Integer Baud Rate Register)

Address offset: 0x0024

Reset value: 0x00

The UART0IBRD Register is the integer part of the baud rate divisor value.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| BAUD DIVINT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| w |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[15:0] BAUD DIVINT – The integer baud rate divisor.

These bits are cleared to 0 on reset

UART0FBRD (UART0 Fractional Baud Rate Register)

Address offset: 0x0028

Reset value: 0x00

The UART0FBRD register is the fractional part of the baud rate divisor value.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | BAUD DIVFRAC |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | w |  |  |  |  |  |

[5:0] BAUD DIVFRAC – The fractional baud rate divisor.

These bits are cleared to 0 on reset

The baud rate divisor is calculated as follows:

Baud rate divisor BAUDDIV = (FUARTCLK/(16×Baud rate))

Where, FUARTCLK is the UART reference clock frequency.

The BAUDDIV is comprised of the integer value (BAUD DIVINT) and the fractional value (BAUD DIVFRAC).

Example 1

If the required baud rate is 115200 and UARTCLK = 8MHz then:

Baud rate divisor = (8×106)/(16 ×115200)  = 4.340278

This means BRDI = 4 and BRDF = 0.340278

(Therefore, UART0IBRD = 4)

Therefore, UART0FBRD = inerger[0.340278 ×64+0.5] = 22

Generated baud rate divider = UART0IBRD +(UART0FBRD/64) = 4.34375

Generated baud rate = (8 ×106)/(16×4.34375) = 115107.914

Error = (115108-115200)/(115200)×100 = -0.07861%

When UartCLK  = 8MHz

| Integer divisor | Fractional divisor | Required bit rate(bps) | Generated bit rate(bps) | Error% |
| --- | --- | --- | --- | --- |
| 0x2 | 0x0B | 230400 | 2.171875 | -0.07994 |
| 0x4 | 0x16 | 115200 | 4.34375 | -0.07994 |
| 0x6 | 0x21 | 76800 | 6.515625 | -0.07994 |
| 0x8 | 0x2C | 57600 | 8.6875 | -0.07994 |
| 0x22 | 0x2E | 14400 | 34.71875 | 0.010001 |

UART0LCR_H (UART0 Line Control Register)

Address offset: 0x002C

Reset value: 0x00

The UART0LCR_H register is the line control register. This register accesses bits 29 to 22 of

the UART line control register, UART0LCR.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | SPS | WLEN |  | FEN | STP2 | EPS | PEN | BRK |
|  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |

[7] SPS – Stick parity select

0: stick parity is disable

1: either:

The parity bit is transmitted and checked as a 1 when EPS bit set ‘0’

The parity bit is transmitted and checked as a 0 when EPS bit set ‘1’

[6:5] WLEN – Word length

| 00 | 01 | 10 | 11 |
| --- | --- | --- | --- |
| 5 bits | 6 bits | 7 bits | 8 bits |

[4] FEN – Enable FIFO

0: The FIFO become 1-byte-deep holding register.

1: The transmit and receive FIFO buffers are enable (FIFO mode)

[3] STP2 – Two stop bit select

1: Two stop bits are transmitted at the end of the frame

[2] EPS – Even parity select

0: odd parity.

1: even parity

[1] PEN – Parity enable

0: parity is disabled and no parity bit added to the data frame

1: parity checking and generations is enabled

[0] BRK – Send break

0: For normal use, the bit must be cleared to 0

1: The low-level is continually output on the UARTTXD output

| PEN | EPS | SPS | Parity bit(Transmitted or checked) |
| --- | --- | --- | --- |
| 0 | X | X | Not transmitted or checked |
| 1 | 1 | 0 | Even parity |
| 1 | 0 | 0 | Odd parity |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 0 |

UART0CR (UART0 Control register)

Address offset: 0x0030

Reset value: 0x0300

The UART0CR register is the control register

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| CTSEn | RTSEn | Out2 | Out1 | RTS | DTR | RXE | TXE | res |  |  |  |  | SIRLP | SIREN | UARTEN |

[15] CTSEn – CTS hardware flow control enable.

1: CTS hardware flow control is enable. Data is only transmitted when the UART0CTS signal is asserted.

[14] RTSEn – RTS hardware flow control enable

1: RTS hardware flow control is enable. Data is only requested when there is space in the receive FIFO for it to be received.

[13] Out2

This bit is the complement of the UART Out2 (nUARTOut2) modem status output.

That is, when the bit is programmed to 1, the output is 0.

For DTE this can be used as “Ring Indicator” (RI).

[12] Out1

This bit is the complement of the UART Out1 (nUARTOut1) modem status output.

That is, when the bit is programmed to 1 the output is 0.

For DTE this can be used as “Data Carrier Detect”(DCD).

[11] RTS – Request to send

This bit is the complement of the UART request to send, UART0RTS, modem status output.

That is, when the bit is programmed to 1 then UART0RTS is LOW.

[10] DTS – Data transmit ready

This bit is the complement of the UART data transmit ready, UART0DTR, modem status output.

That is, when the bit is programmed to 1 then UART0DTR is LOW.

[9] RXE – Receive enable

If this bit is set to 1, the receive section of the UART is enabled.

Data reception occurs for either UART signals or SIR signals depending on the setting of the SIREN bit. When the UART is disabled in the middle of reception, it completes the current character before stopping.

[8] TXE – Transmit enable

1: The transmit section of the UART is enabled,

[7:3] Reserved

[2] SIRLP – SIR low-power IrDA mode

0: low-level bits are transmitted as an active high pulse with a width of 3/16th of the bit period.

1: low-level bits are transmitted with a pulse width that is 3 times the period of the

IrLPBaud16 input signal, regardless of the selected bit rate.

[1] SIREN – SIR enable

0: IrDA SIR ENDEC is disable

1: IrDA SIR ENDEC is enable

[0] UARTEN –UART enable

0: UART is disabled

1: UART is enabled

Program the control registers as follows:

1. Disable the UART.

2. Wait for the end of transmission or reception of the current character.

3. Flush the transmit FIFO by setting the FEN bit to 0 in the Line Control Register,

UARTLCR_H.

4. Reprogram the UARTCR Register.

5. Enable the UART.

UART0IFLS (UART0 Interrupt FIFO Level Select Register)

Address offset: 0x0034

Reset value: 0x12

The UARTIFLS register is the interrupt FIFO level select register.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | RXIFLSEL |  |  | TXIFLSEL |  |  |
|  |  |  |  |  |  |  |  |  |  | W |  |  | W |  |  |

[5:3] RXIFLSEL – Receive interrupt FIFO level select

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  | 7/8 full | 3/4 full | 1/2 full | 1/4 full | 1/8 full |

[2:0] TXIFLSEL – Transmit interrupt FIFO level select

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  | 7/8 full | 3/4 full | 1/2 full | 1/4 full | 1/8 full |

UART0IMSC (UART0 Interrupt Mask Set/Clear Register)

Address offset: 0x0038

Reset value: 0x00

The UART0IMSC register is the interrupt mask set/clear interrupts. When a bit of UART0IMSC is ‘1’ and the corresponding bit of interrupt register is ‘1’, an interrupt will be issued.

In other words, if a bit of UART0IMSC is ‘0’, an interrupt will not be issued even if the corresponding bit of interrupt register is ‘1’.

| 31 | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 | 20 |  | 19 |  |  | 18 |  | 17 |  | 16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res | res |  | res |  |  | res |  | res |  | res |  |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  |  | 5 |  | 4 | 3 |  | 2 |  | 1 |  | 0 |
| res |  | Res |  | res |  | res |  | res |  | OEIM |  | BEIM |  | PEIM |  | FEIM |  | RTIM |  |  | TXIM |  | RXIM | DSRMIM |  | DCDMIM |  | CTSMIM |  | RIMIM |
|  |  |  |  |  |  |  |  |  |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  |  | R/W |  | R/W | R/W |  | R/W |  | R/W |  | R/W |

[10] OEIM – Overrun error interrupt mask

0: Disable UART0OEINTR

1: Enable UART0OEINTR

[9] BEIM – Break error interrupt mask

0: Disable UART0BEINTR

1: Enable UART0BEINTR

[8] PEIM – Parity error interrupt mask

0: Disable UART0EINTR

1: Enable UART0EINTR

[7] FEIM – Framing error interrupt mask

0: Disable UART0FEINTR

1: Enable UART0FEINTR

[6] RTIM – Receive timeout interrupt mask

0: Disable UART0RTINTR

1: Enable UART0RTINTR

[5] TXIM – Transmit interrupt mask

0: Disable UART0TXINTR

1: Enable UART0TXINTR

[4] RXIM – Receive interrupt mask

0: Disable UART0RXINTR

1: Enable UART0RXINTR

[3] DSRMIM – nUART0DSR modem interrupt mask

0: Disable UART0DSRINTR

1: Enable UART0DSRINTR

[2] DCDMIM – nUART0DCD modem interrupt mask

0: Disable UART0DCDINTR

1: Enable UART0DCDINTR

[1] CTSMIM – nUART0CTS modem interrupt mask

0: Disable UART0CRSINTR

1: Enable UART0CRSINTR

[0] RIMIM – nUART0RI modem interrupt mask

0: Disable UART0RIINTR

1: Enable UART0RIINTR

UART0RIS (UART0 Raw Interrupt Status Register)

Address offset: 0x003C

Reset value: 0x00-

The UART0RIS register indicates the raw interrupt status register.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | OE RIS | BE RIS | PE RIS | FE RIS | RT RIS | TX RIS | RX RIS | DRRRMIS | DCDRMIS | CTSRMIS | RI RMIS |
|  |  |  |  |  | R | R | R | R | R | R | R | R | R | R | R |

[10] OERIS – Overrun error interrupt status

It indicates state of the UART0OEINTR interrupt.

[9] BERIS – Break error interrupt status

It indicates state of the UART0BEINTR interrupt.

[8] PERIS – Parity error interrupt status

It indicates state of the UART0PEINTR interrupt.

[7] FERIS – Framing error interrupt status

It indicates state of the UART0FEINTR interrupt.

[6] RTRIS – Receive timeout interrupt status

It indicates state of the UART0RTINTR interrupt.

[5] TXRIS – Transmit interrupt status

It indicates state of the UART0TXINTR interrupt.

[4] RXRIS – Receive interrupt status

It indicates state of the UART0RXINTR interrupt.

[3] DSRRMIS – nUART0DSR modem interrupt status

It indicates state of the UART0DSRINTR interrupt.

[2] DCDRMIS – nUART0DCD modem interrupt status

It indicates state of the UART0DCDINTR interrupt.

[1] CTSRMIS – nUART0CTS modem interrupt status

It indicates state of the UART0CTSINTR interrupt.

[0] RIRMIS – nUART0RI modem interrupt status

It indicates state of the UART0RIINTR interrupt.

UART0MIS (UART0 Masked Interrupt Status Register)

Address offset: 0x0040

Reset value: 0x00-

The UART0MIS register is the masked interrupt status register.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | OE MIS | BE MIS | PE MIS | FE MIS | RT MIS | TX MIS | RX MIS | DRRMMIS | DCDMMIS | CTS MMIS | RI MMIS |
|  |  |  |  |  | R | R | R | R | R | R | R | R | R | R | R |

[10] OEMIS – Overrun error masked interrupt status

It indicates state of the UART0OEINTR interrupt.

[9] BEMIS – Break error masked interrupt status

It indicates state of the UART0BEINTR interrupt.

[8] PEMIS – Parity error masked interrupt status

It indicates state of the UART0PEINTR interrupt.

[7] FEMIS – Framing error masked interrupt status

It indicates state of the UART0FEINTR interrupt.

[6] RTMIS – Receive timeout masked interrupt status

It indicates state of the UART0RTINTR interrupt.

[5] TXMIS – Transmit masked interrupt status

It indicates state of the UART0TXINTR interrupt.

[4] RXMIS – Receive masked interrupt status

It indicates state of the UART0RXINTR interrupt.

[3] DSRMMIS – nUART0DSR modem masked interrupt status

It indicates state of the UART0DSRINTR interrupt.

[2] DCDMMIS – nUART0DCD modem masked interrupt status

It indicates state of the UART0DCDINTR interrupt.

[1] CTSMMIS – nUART0CTS modem masked interrupt status

It indicates state of the UART0CTSINTR interrupt.

[0] RIMMIS – nUART0RI modem masked interrupt status

It indicates state of the UART0RIINTR interrupt.

UART0ICR (UART0 Interrupt Clear Register)

Address offset: 0x0044

Reset value: -

The UART0ICR register is the interrupt clear register and is write-only.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | OEIC | BEIC | PEIC | FEIC | RTIC | TXIC | RXIC | DSRMIC | DCDMIC | CTSMIC | RI MIC |
|  |  |  |  |  | R | R | R | R | R | R | R | R | R | R | R |

[10] OEIC – Overrun error interrupt clear

Clear the UART0OEINTR interrupt.

[9] BEIC – Break error interrupt clear

Clear the UART0BEINTR interrupt.

[8] PEIC – Parity error interrupt clear

Clear the UART0PEINTR interrupt.

[7] FEIC – Framing error interrupt clear

Clear the UART0FEINTR interrupt.

[6] RTIC – Receive timeout interrupt clear

Clear the UART0RTINTR interrupt.

[5] TXIC – Transmit interrupt clear

Clear the UART0TXINTR interrupt.

[4] RXIC – Receive interrupt clear

Clear the UART0RXINTR interrupt.

[3] DSRMIC – nUART0DSR modem interrupt clear

Clear the UART0DSRINTR interrupt.

[2] DCDMIC – nUART0DCD modem interrupt clear

Clear the UART0DCDINTR interrupt.

[1] CTSMIC – nUART0CTS modem interrupt clear

Clear the UART0CTSINTR interrupt.

[0] RIMIC – nUART0RI modem interrupt clear

Clear the UART0RIINTR interrupt.

Register map

The following Table 34 summarizes the UART0 registers.

Table 34 UART0 register map and reset values

UART1 Registers(Base address: 0x4000_D000)

UART1DR (UART1 Data Register)

Address offset: 0x000

Reset value: 0x0000_0000

The UART1DR is the data register.

The write operation initiates transmission from the UART. The data is prefixed with a start bit, appended with the appropriate parity bit(if parity is enabled), and a stop bit. The resultant word is then transmitted.

The received data byte is read by performing reads from the UARTDR register along with the corresponding status information.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | OE | BE | PE | FE | DATA |  |  |  |  |  |  |  |
|  |  |  |  | R | R | R | R | R/W |  |  |  |  |  |  |  |

[11] OE – Overrun error

0: data is empty

1: data is received and the receive FIFO is already full.

[10] BE – Break error

1: if a break condition was detected, indicating that the received data input was held LOW of longer than a full-word transmission time(defined as start, data, parity and stop bits)

[9] PE – Parity error

1: it indicates that the parity of the received, it indicates that the parity of the received data character does not match the parity that the EPS and SPS bits in the line control register, UARTLCR_H

[8] FE – Framing error

1: it indicates that the received

[7:0] DATA – Receive (READ)/Transmit (WRITE) data

UART1RSR/ECR (UART1 Receive Status Register/Error Clear Register)

Address offset: 0x004

Reset value: 0x0000_0000

The UART1RSR/ECR is the receive status register/error clear register.

Receive status can also be read from the UART1RSR register.

A write to the UART1ECR register clears the framing, parity, break, and overrun errors.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | OE | BE | PE | FE |

[3] OE – Overrun error

This bit is set to 1 if data is received and the FIFO is already full.

This bit is cleared to 0 by a write to UART1ECR

[2] BE – Break error

This bit is cleared to 0 by a write to UART1ECR

[1] PE – Parity error

When set to 1, it indicates that the parity of the received data character does not match

the parity that the EPS and SPS bits in the line control register, UARTLCR_H select

This bit is cleared to 0 by a write to UART1ECR

[0] FE – Framing error

When set to 1, in indicates that the received character didn’t have a valid stop bit

This bit is cleared to 0 by a write to UART1ECR

UART1FR (UART1 Flag Register)

Address offset: 0x0018

Reset value: 0bx11000xxx

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | RI | TXFE | RXFF | TXFF | RXFE | BUSY | DCD | DSR | CTS |
|  |  |  |  |  |  |  | R | R | R | R | R | R | R | R | R |

[8] RI – Ring indicator

This bit is the complement of the UART ring indicator, UART1RI.

1: When nUART1RI is LOW

[7] TXFE – Transmit FIFO empty

This bit depends on the state of the FEN bit in the line control register, UARTLCR_H.

0: The bit is set when transmit holding register is empty.

1: The bit is set when transmit FIFO is empty

[6] RXFF – Receive FIFO full

This bit depends on the state of the FEN bit in the line control register, UARTLCR_H.

0: The bit is set when the receive holding register is full

1: The bit is set when the receive FIFO is full

[5] TXFF – Transmit FIFO full

This bit depends on the state of the FEN bit in the line control register, UARTLCR_H.

0: The bit is set when transmit holding register is full.

1: The bit is set when transmit FIFO is full.

[4] RXFE – Receive FIFO empty

This bit depends on the state of the FEN bit in the line control register, UARTLCR_H.

0: The bit is set when the receive holding register is empty.

1: The bit is set when the receive FIFO is empty.

[3] BUSY – UART busy

1: the UART is busy transmitting data

[2] DCD – Data carrier detect

This bit is the complement of the UART data carrier detect, UART1DCD, status input

1: The bit is the complement of the UART data carrier detect

[1] DSR – Data set ready

This bit is the complement of the UART data set ready, UART1DSR, status input

1: The bit is the complement of the UART data set ready

[0] CTS – Clear to send

This bit is the complement of the UART clear to send, UART1CTS, status input

1: The bit is the complement of the UART clear to send

UART1ILPR (UART1 IrDA Low-Power Counter Register)

Address offset: 0x0020

Reset value: 0x00

The UARTILPR Register is the IrDA low-power counter register

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | ILPDVSR |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |

[7:0] ILPDVSR – 8-bit low-power divisor value

These bits are cleared to 0 at reset

ILPDVSR=(FUARTCLK/FIrLPBaud16)

Where, FIrLPBaud16 is nominally 1.8432MHz

The divisor is 1.42MHz < FIrLPBaud16 < 2.12MHz, results in a low-power pulse duration of 1.41 – 2.11us.

UART1IBRD (UART1 Integer Baud Rate Register)

Address offset: 0x0024

Reset value: 0x00

The UART1IBRD Register is the integer part of the baud rate divisor value.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| BAUD DIVINT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| w |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[15:0] BAUD DIVINT – The integer baud rate divisor.

These bits are cleared to 0 on reset

UART1FBRD (UART1 Fractional Baud Rate Register)

Address offset: 0x0028

Reset value: 0x00

The UART1FBRD register is the fractional part of the baud rate divisor value.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | BAUD DIVFRAC |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | w |  |  |  |  |  |

[5:0] BAUD DIVFRAC – The fractional baud rate divisor.

These bits are cleared to 0 on reset

The baud rate divisor is calculated as follows:

Baud rate divisor BAUDDIV = (FUARTCLK/(16×Baud rate))

Where, FUARTCLK is the UART reference clock frequency.

The BAUDDIV is comprised of the integer value (BAUD DIVINT) and the fractional value (BAUD DIVFRAC).

Example 1

If the required baud rate is 115200 and UARTCLK = 8MHz then:

Baud rate divisor = (8×106)/(16 ×115200)  = 4.340278

This means BRDI = 4 and BRDF = 0.340278

(Therefore, UART1IBRD = 4)

Therefore, UART1FBRD = inerger[0.340278 ×64+0.5] = 22

Generated baud rate divider = UART1IBRD +(UART1FBRD/64) = 4.34375

Generated baud rate = (8 ×106)/(16×4.34375) = 115107.914

Error = (115108-115200)/(115200)×100 = -0.07861%

When UartCLK  = 8MHz

| Integer divisor | Fractional divisor | Required bit rate(bps) | Generated bit rate(bps) | Error% |
| --- | --- | --- | --- | --- |
| 0x2 | 0x0B | 230400 | 2.171875 | -0.07994 |
| 0x4 | 0x16 | 115200 | 4.34375 | -0.07994 |
| 0x6 | 0x21 | 76800 | 6.515625 | -0.07994 |
| 0x8 | 0x2C | 57600 | 8.6875 | -0.07994 |
| 0x22 | 0x2E | 14400 | 34.71875 | 0.010001 |

UART1LCR_H (UART1 Line Control Register)

Address offset: 0x002C

Reset value: 0x00

The UART1LCR_H register is the line control register. This register accesses bits 29 to 22 of

the UART line control register, UART1LCR.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | SPS | WLEN |  | FEN | STP2 | EPS | PEN | BRK |
|  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |

[7] SPS – Stick parity select

0: stick parity is disable

1: either:

The parity bit is transmitted and checked as a 1 when EPS bit set ‘0’

The parity bit is transmitted and checked as a 0 when EPS bit set ‘1’

[6:5] WLEN – Word length

| 00 | 01 | 10 | 11 |
| --- | --- | --- | --- |
| 5 bits | 6 bits | 7 bits | 8 bits |

[4] FEN – Enable FIFO

0: The FIFO become 1-byte-deep holding register.

1: The transmit and receive FIFO buffers are enable (FIFO mode)

[3] STP2 – Two stop bit select

1: Two stop bits are transmitted at the end of the frame

[2] EPS – Even parity select

0: odd parity.

1: even parity

[1] PEN – Parity enable

0: parity is disabled and no parity bit added to the data frame

1: parity checking and generations is enabled

[0] BRK – Send break

0: For normal use, the bit must be cleared to 0

1: The low-level is continually output on the UARTTXD output

| PEN | EPS | SPS | Parity bit(Transmitted or checked) |
| --- | --- | --- | --- |
| 0 | X | X | Not transmitted or checked |
| 1 | 1 | 0 | Even parity |
| 1 | 0 | 0 | Odd parity |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 0 |

UART1CR (UART1 Control register)

Address offset: 0x0030

Reset value: 0x0300

The UART1CR register is the control register

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| CTSEn | RTSEn | Out2 | Out1 | RTS | DTR | RXE | TXE | res |  |  |  |  | SIRLP | SIREN | UARTEN |

[15] CTSEn – CTS hardware flow control enable.

1: CTS hardware flow control is enable. Data is only transmitted when the UART1CTS signal is asserted.

[14] RTSEn – RTS hardware flow control enable

1: RTS hardware flow control is enable. Data is only requested when there is space in the receive FIFO for it to be received.

[13] Out2

This bit is the complement of the UART Out2 (nUARTOut2) modem status output.

That is, when the bit is programmed to 1, the output is 0.

For DTE this can be used as “Ring Indicator” (RI).

[12] Out1

This bit is the complement of the UART Out1 (nUARTOut1) modem status output.

That is, when the bit is programmed to 1 the output is 0.

For DTE this can be used as “Data Carrier Detect”(DCD).

[11] RTS – Request to send

This bit is the complement of the UART request to send, UART1RTS, modem status output.

That is, when the bit is programmed to 1 then UART1RTS is LOW.

[10] DTS – Data transmit ready

This bit is the complement of the UART data transmit ready, UART1DTR, modem status output.

That is, when the bit is programmed to 1 then UART1DTR is LOW.

[9] RXE – Receive enable

If this bit is set to 1, the receive section of the UART is enabled.

Data reception occurs for either UART signals or SIR signals depending on the setting of the SIREN bit. When the UART is disabled in the middle of reception, it completes the current character before stopping.

[8] TXE – Transmit enable

1: The transmit section of the UART is enabled,

[7:3] Reserved

[2] SIRLP – SIR low-power IrDA mode

0: low-level bits are transmitted as an active high pulse with a width of 3/16th of the bit period.

1: low-level bits are transmitted with a pulse width that is 3 times the period of the

IrLPBaud16 input signal, regardless of the selected bit rate.

[1] SIREN – SIR enable

0: IrDA SIR ENDEC is disable

1: IrDA SIR ENDEC is enable

[0] UARTEN – UART enable

0: UART is disabled

1: UART is enabled

Program the control registers as follows:

1. Disable the UART.

2. Wait for the end of transmission or reception of the current character.

3. Flush the transmit FIFO by setting the FEN bit to 0 in the Line Control Register,

UARTLCR_H on page 3-12.

4. Reprogram the UARTCR Register.

5. Enable the UART.

UART1IFLS (UART1 Interrupt FIFO Level Select Register)

Address offset: 0x0034

Reset value: 0x12

The UARTIFLS register is the interrupt FIFO level select register.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | RXIFLSEL |  |  | TXIFLSEL |  |  |
|  |  |  |  |  |  |  |  |  |  | W |  |  | W |  |  |

[5:3] RXIFLSEL – Receive interrupt FIFO level select

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  | 7/8 full | 3/4 full | 1/2 full | 1/4 full | 1/8 full |

[2:0] TXIFLSEL – Transmit interrupt FIFO level select

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  | 7/8 full | 3/4 full | 1/2 full | 1/4 full | 1/8 full |

UART1IMSC (UART1 Interrupt Mask Set/Clear Register)

Address offset: 0x0038

Reset value: 0x00

The UART1IMSC register is the interrupt mask set/clear interrupts. When a bit of UART1IMSC is ‘1’ and the corresponding bit of interrupt register is ‘1’, an interrupt will be issued.

In other words, if a bit of UART1IMSC is ‘0’, an interrupt will not be issued even if the corresponding bit of interrupt register is ‘1’.

| 31 | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 | 20 |  | 19 |  |  | 18 |  | 17 |  | 16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res | res |  | res |  |  | res |  | res |  | res |  |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  |  | 5 |  | 4 | 3 |  | 2 |  | 1 |  | 0 |
| res |  | Res |  | res |  | res |  | res |  | OEIM |  | BEIM |  | PEIM |  | FEIM |  | RTIM |  |  | TXIM |  | RXIM | DSRMIM |  | DCDMIM |  | CTSMIM |  | RIMIM |
|  |  |  |  |  |  |  |  |  |  | R/W |  | R/W |  | R/W |  | R/W |  | R/W |  |  | R/W |  | R/W | R/W |  | R/W |  | R/W |  | R/W |

[10] OEIM – Overrun error interrupt mask

0: Disable UART1OEINTR

1: Enable UART1OEINTR

[9] BEIM – Break error interrupt mask

0: Disable UART1BEINTR

1: Enable UART1BEINTR

[8] PEIM – Parity error interrupt mask

0: Disable UART1EINTR

1: Enable UART1EINTR

[7] FEIM – Framing error interrupt mask

0: Disable UART1FEINTR

1: Enable UART1FEINTR

[6] RTIM – Receive timeout interrupt mask

0: Disable UART1RTINTR

1: Enable UART1RTINTR

[5] TXIM – Transmit interrupt mask

0: Disable UART1TXINTR

1: Enable UART1TXINTR

[4] RXIM – Receive interrupt mask

0: Disable UART1RXINTR

1: Enable UART1RXINTR

[3] DSRMIM – nUART1DSR modem interrupt mask

0: Disable UART1DSRINTR

1: Enable UART1DSRINTR

[2] DCDMIM – nUART1DCD modem interrupt mask

0: Disable UART1DCDINTR

1: Enable UART1DCDINTR

[1] CTSMIM – nUART1CTS modem interrupt mask

0: Disable UART1CRSINTR

1: Enable UART1CRSINTR

[0] RIMIM – nUART1RI modem interrupt mask

0: Disable UART1RIINTR

1: Enable UART1RIINTR

UART1RIS (UART1 Raw Interrupt Status Register)

Address offset: 0x003C

Reset value: 0x00-

The UART1RIS register indicates the raw interrupt status register.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | OE RIS | BE RIS | PE RIS | FE RIS | RT RIS | TX RIS | RX RIS | DRRRMIS | DCDRMIS | CTSRMIS | RI RMIS |
|  |  |  |  |  | R | R | R | R | R | R | R | R | R | R | R |

[10] OERIS – Overrun error interrupt status

It indicates state of the UART1OEINTR interrupt.

[9] BERIS – Break error interrupt status

It indicates state of the UART1BEINTR interrupt.

[8] PERIS – Parity error interrupt status

It indicates state of the UART1PEINTR interrupt.

[7] FERIS – Framing error interrupt status

It indicates state of the UART1FEINTR interrupt.

[6] RTRIS – Receive timeout interrupt status

It indicates state of the UART1RTINTR interrupt.

[5] TXRIS – Transmit interrupt status

It indicates state of the UART1TXINTR interrupt.

[4] RXRIS – Receive interrupt status

It indicates state of the UART1RXINTR interrupt.

[3] DSRRMIS – nUART1DSR modem interrupt status

It indicates state of the UART1DSRINTR interrupt.

[2] DCDRMIS – nUART1DCD modem interrupt status

It indicates state of the UART1DCDINTR interrupt.

[1] CTSRMIS – nUART1CTS modem interrupt status

It indicates state of the UART1CTSINTR interrupt.

[0] RIRMIS – nUART1RI modem interrupt status

It indicates state of the UART1RIINTR interrupt.

UART1MIS (UART1 Masked Interrupt Status Register)

Address offset: 0x0040

Reset value: 0x00-

The UART1MIS register is the masked interrupt status register.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | OE MIS | BE MIS | PE MIS | FE MIS | RT MIS | TX MIS | RX MIS | DRRMMIS | DCDMMIS | CTS MMIS | RI MMIS |
|  |  |  |  |  | R | R | R | R | R | R | R | R | R | R | R |

[10] OEMIS – Overrun error masked interrupt status

It indicates state of the UART1OEINTR interrupt.

[9] BEMIS – Break error masked interrupt status

It indicates state of the UART1BEINTR interrupt.

[8] PEMIS – Parity error masked interrupt status

It indicates state of the UART1PEINTR interrupt.

[7] FEMIS – Framing error masked interrupt status

It indicates state of the UART1FEINTR interrupt.

[6] RTMIS – Receive timeout masked interrupt status

It indicates state of the UART1RTINTR interrupt.

[5] TXMIS – Transmit masked interrupt status

It indicates state of the UART1TXINTR interrupt.

[4] RXMIS – Receive masked interrupt status

It indicates state of the UART1RXINTR interrupt.

[3] DSRMMIS – nUART1DSR modem masked interrupt status

It indicates state of the UART1DSRINTR interrupt.

[2] DCDMMIS – nUART1DCD modem masked interrupt status

It indicates state of the UART1DCDINTR interrupt.

[1] CTSMMIS – nUART1CTS modem masked interrupt status

It indicates state of the UART1CTSINTR interrupt.

[0] RIMMIS – nUART1RI modem masked interrupt status

It indicates state of the UART1RIINTR interrupt.

UART1ICR (UART1 Interrupt Clear Register)

Address offset: 0x0044

Reset value: -

The UART1ICR register is the interrupt clear register and is write-only.

| 31 | 30 |  | 29 |  | 28 |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  |  |  | 22 |  | 21 |  | 20 |  | 19 |  | 18 |  | 17 |  | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res |  | res |  | res |  | res |  | res |  | res |  | res |  | res |  |  |  | res |  | res |  | res |  | res |  | res |  | res |  | res |
| 15 |  | 14 |  | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| res |  | res |  | res |  | res |  | res |  | OEIC |  | BEIC |  | PEIC |  | FEIC |  | RTIC |  | TXIC |  | RXIC |  | DSRMIC |  | DCDMIC |  | CTSMIC |  | RI MIC |  |
|  |  |  |  |  |  |  |  |  |  | R |  | R |  | R |  | R |  | R |  | R |  | R |  | R |  | R |  | R |  | R |  |

[10] OEIC – Overrun error interrupt clear

Clear the UART1OEINTR interrupt.

[9] BEIC – Break error interrupt clear

Clear the UART1BEINTR interrupt.

[8] PEIC – Parity error interrupt clear

Clear the UART1PEINTR interrupt.

[7] FEIC – Framing error interrupt clear

Clear the UART1FEINTR interrupt.

[6] RTIC – Receive timeout interrupt clear

Clear the UART1RTINTR interrupt.

[5] TXIC – Transmit interrupt clear

Clear the UART1TXINTR interrupt.

[4] RXIC – Receive interrupt clear

Clear the UART1RXINTR interrupt.

[3] DSRMIC – nUART1DSR modem interrupt clear

Clear the UART1DSRINTR interrupt.

[2] DCDMIC – nUART1DCD modem interrupt clear

Clear the UART1DCDINTR interrupt.

[1] CTSMIC – nUART1CTS modem interrupt clear

Clear the UART1CTSINTR interrupt.

[0] RIMIC – nUART1RI modem interrupt clear

Clear the UART1RIINTR interrupt.

Register map

The following Table 35 summarizes the UART1 registers.

Table 35 UART1 register map and reset values

Universal Asynchronous Receive Transmit(UART2)

Introduction

The UART2 supports Asynchronous one-way communication, half-duplex single wire communication, and without CTS/RTS. UART2 is called the simple UART.

Feature

- Serial-to-parallel conversion on data received from a peripheral device

- Parallel-to-serial conversion on data transmitted to the peripheral device

Functional description

UART bidirectional communication requires a minimum of two pins: RX, TX

The frame are comprised of:

- An Idle Line prior to transmission or reception

- The UART interface uses a baud rate generator

- A status register (UART2_SR)

- data registers (UART2DR)

- A baud rate register (UART2_B,UART1_FBRD)

Baud rate calculation

UART2 can operate with or without using the Fractional Divider. The baud rate divisor is a 20-bit number consisting the UART2BDR.

This is used by the baud rate generator to determine the bit period.

- Baud Rate Divisor =UARTCLK(baud rate)

Figure 53. UART2 divider flow chart

Figure 54 show how to set the UART Initial value.

Figure 54. UART2 Initial setting flow chart

UART2 Registers(Base address: 0x4000_6000)

UART2DR (UART2 Data Register)

Address offset: 0x000

Reset value: 0x0000_0000

The UART2DR is the data register.

The write operation initiates transmission from the UART2. The received data byte is read by performing reads from the UART2DR register along with the corresponding status information.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | DATA |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | R/W |  |  |  |  |  |  |  |

[7:0] DATA – Receive (READ)/Transmit (WRITE) data

UART2SR (UART2 Status Register)

Address offset: 0x004

Reset value: 0x0000_0000

The UART2SR is the status register. RXBF and TXBF is read only.

A write to the UART2SR register clears the TX/RX overrun errors.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | RXO | TXO | RXBF | TXBF |
|  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R | R |

[3] RXO –Receive Overrun

[2] TXO – Transmit Overrun

[1] RXBF – Receive buffer full

Read Only

0: The bit is set when the receive holding register is full

1: The bit is set when the receive buffer is full

[0] TXBF – Transmit buffer full

Read Only

0: The bit is set when transmit holding register is full.

1: The bit is set when transmit buffer is full.

UART2CR (UART2 Control Register)

Address offset: 0x008

Reset value: 0x0000_0000

The UART2CR is the control register.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Res | Res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Res | Res | res | res | res | res | res | res | res | res | ROIE | TOIE | RXIE | TXIE | RXE | TXE |
|  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W |

[5] ROIE – Receive Overrun Interrupt Enable

[4] TOIE – Transmit Overrun Interrupt Enable

[3] RXIE – Receive Interrupt Enable

[2] TXIE – Transmit Interrupt Enable

[1] RXE – Receive enable

If this bit is set to 1, the receive section of the UART2 is enabled.

When the UART2 is disabled in the middle of reception, it completes the current character before stopping.

[0] TXE – Transmit enable

If this bit is set to 1, the transmit section of the UART2 is enabled

UART2ISR/ICR (UART2 Interrupt Status/Interrupt Clear Register)

Address offset: 0x00C

Reset value: 0x0000_0000

The UART2ISR is the interrupt status register.

A write to the UART2ISR register clear to interrupts.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | RXOI | TXOI | RXI | TXI |
|  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W |

[3] RXOI –Receive Overrun Interrupt

This bit depends on the state of the ROIE bit in the control register, UART2CR

[2] TXOI – Transmit Overrun Interrupt

This bit depends on the state of the TOIE bit in the control register, UART2CR

[1] RXI – Receive Interrupt

This bit depends on the state of the RXIE bit in the control register, UART2CR

[0] TXI – Transmit Interrupt

This bit depends on the state of the TXIE bit in the control register, UART2CR

UART2BDR (UART2 Baud Rate Divider Register)

Address offset: 0x010

Reset value: 0x0000_0000

The UART2BDR r is the integer part of the baud rate divisor value.

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | Res | res | res | res | res | res | res | res | res | res | res |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | R/W |  |  |  |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| BAUDDIV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[19:0] Baud rate divider

The minimum number is 16. These bits are cleared to 0 on reset

Example 1

If the required baud rate is 115200 and UARTCLK = 8MHz then:

Baud rate divisor BAUDDIV = (FUARTCLK/Baud rate)

Baud rate divisor = (8×106)/(115200)  = 69.4

This means BRD= 69

Register map

The following Table 36 summarizes the UART2 registers.

Table 36 UART2 register map and reset values

- Synchronous Serial Port (SSP)

Introduction

The SSP block is an IP provided by ARM (PL022 “PrimeCell® Synchronous Serial Port”).

Additional details about its functional blocks may be found in “ARM PrimeCell® Synchronous Serial Port (PL022) Technical Reference Manual”.

Features

•   The SSP is a master or slave interface that enables synchronous serial communication with slave or master peripherals having one of the following:

  - A MOTOROLA SPI-compatible interface

  - A TEXAS INSTRUMENTS synchronous serial interface

  - A National Semiconductor MICROWIRE® interface.

•   The SPI interface operates as a master or slave interface. It supports bit rates up to 2 MHz and higher in both master and slave configurations. The SPI has the following features:

  - Parallel-to-serial conversion on data written to an internal 16-bit wide, 8-       location deep transmit FIFO

  - Serial-to-parallel conversion on received data, buffering it in a 16-bit wide, 8-location deep receive FIFO

  - Programmable data frame size from 4 to 16 bits

  - Programmable clock bit rate and prescaler. The input clock may be divided by a factor of 2 to 254 in steps of two to provide the serial output clock

  - Programmable clock phase and polarity.

Functional description

Figure 55 shows the SSP block diagram.

Figure 55. SSP block diagram

Clock prescaler

When configured as a master, an internal prescaler is used to provide the serial output clock. The prescaler may be programmed through the SSPCPSR register to divide the SSPCLK by a factor of 2 to 254 in two steps. As the least significant bit of the SSPCPSR register is not used, division by an odd number is impossible and this ensures a symmetrical (equal mark space ratio) clock is generated.

The output of this prescaler is further divided by a factor 1 to 256 through the programming of the SSPCR0 control register, to give a final master output clock.

Transmit FIFO

The common transmit FIFO is a 16-bit wide, 8-locations deep, First-In, First-Out (FIFO) memory buffer. CPU data written across the AMBA APB interface are stored in the buffer until it is read out by the transmit logic.

When configured as a master or a slave, parallel data is written into the transmit FIFO prior to serial conversion and is transmitted to the attached slave or master through the SSPTXD pin.

Receive FIFO

The common receive FIFO is a 16-bit wide, 8-locations deep, first-in, first-out memory buffer.

Received data from the serial interface are stored in the buffer until it is read out by the CPU across the AMBA APB interface.

When configured as a master or slave, serial data received through the SSPRXD pin is registered prior to parallel loading into the attached slave or master receive FIFO.

Interrupt generation logic

The PrimeCell SSP generates four individual maskable, active-HIGH interrupts. A combined interrupt output is also generated as an OR function of the individual interrupt requests.

Users can use the single combined interrupt with a system interrupt controller that provides another level of masking on a per-peripheral basis. This enables use of modular device drivers that always know where to find the interrupt source control register bits.

Users can also use the individual interrupt requests with a system interrupt controller that provides masking for the outputs of each peripheral. In this way, a global interrupt controller service routine can read the entire set of sources from one wide register in the system interrupt controller. This is attractive when the time to read from the peripheral registers is significant compared to the CPU clock speed in a real-time system.

The peripheral supports both methods above.

The transmit and receive dynamic data-flow interrupts, SSPTXINTR and SSPRXINTR, are separated from the status interrupts so that data can be read or written in response to the FIFO trigger levels.

DMA interface

The PrimeCell SSP provides an interface to connect to the DMA controller. The PrimeCell SSP DMA control register, SSPDMACR controls the DMA operation of the PrimeCell SSP.

Receive – The DMA interface includes the following signals for receive:

- SSPRXDMASREQ

  - Single-character DMA transfer request asserted by the SSP. This signal is asserted when the receive FIFO contains at least one character.

- SSPRXDMABREQ

  - Burst DMA transfer request, asserted by the SSP. This signal is asserted when the receive FIFO contains four or more characters.

- SSPRXDMACLR

  - DMA request clear asserted by the DMA controller to clear the receive request signals. If DMA burst transfer is requested, the clear signal is asserted during the transfer of the last data in the burst.

Transmit – The DMA interface includes the following signals for transmit:

- SSPTXDMASREQ

  - Single-character DMA transfer request asserted by the SSP. This signal is asserted when there is at least one empty location in the transmit FIFO.

- SSPTXDMABREQ

  - Burst DMA transfer request asserted by the SSP. This signal is asserted when the transmit FIFO contains four characters or fewer.

- SSPTXDMACLR

  - DMA request clear asserted by the DMA controller to clear the transmit request signals. If a DMA burst transfer is requested, the clear signal is asserted during the transfer of the last data in the burst.

The burst transfer and single transfer request signals are not mutually exclusive. They can both be asserted at the same time. For example, when there is more data than the watermark level of four in the receive FIFO, the burst transfer request and the single transfer request are asserted.

When the amount of data left in the receive FIFO is less than the watermark level, the single request only is asserted. This is useful for situations when the number of characters left to be received in the stream is less than a burst.

For example, if 19 characters must be received, the DMA controller then transfers four bursts of four characters and three single transfers to complete the stream.

The PrimeCell SSP does not assert the burst request for the remaining three characters.

Each request signal remains asserted until the relevant DMA clear signal is asserted. After the request clear signal is de-asserted, a request signal can become active again depending on the conditions that previous sections describe. All request signals are de-asserted if the PrimeCell SSP is disabled or the DMA enable signal is cleared.

Table 37 shows the trigger points for DMABREQ of both the transmit and receive FIFOs.

Table 37 DMA trigger points for the transmit and receive FIFOs.

|  | Burst length |  |
| --- | --- | --- |
| Watermark level | Transmit, number of empty locations | Receive, number of filled locations |
| 1/2 | 4 | 4 |

Figure 56 shows the timing diagram for both a single transfer request, and a burst transfer request, with the appropriate DMA clear signal. The signals are all synchronous to PCLK.

Figure 56. DMA transfer waveforms

Interface reset

The PrimeCell SSP is reset by the global reset signal, PRESETn, and a block-specific reset signal, nSSPRST. An external reset controller must use PRESETn to assert nSSPRST asynchronously and negate it synchronously to SSPCLK. PRESETn must be asserted LOW for a period long enough to reset the slowest block in the on-chip system, and then taken HIGH again. The PrimeCell SSP requires PRESETn to be asserted LOW for at least one period of PCLK.

Configuring the SSP

The Following reset, the PrimeCell SSP logic is disabled and must be configured when in this state.

It is necessary to program control registers SSPCR0 and SSPCR1 to configure the peripheral as a master or slave operating under one of the following protocols:

• Motorola SPI

• Texas Instruments SSI

• National Semiconductor.

The bit rate, derived from the external SSPCLK, requires the programming of the clock prescale register SSPCPSR.

Enable PrimeCell SSP operation

You can either prime the transmit FIFO, by writing up to eight 16-bit values when the PrimeCell SSP is disabled, or permit the transmit FIFO service request to interrupt the CPU. Once enabled, transmission or reception of data begins on the transmit, SSPTXD, and receive, SSPRXD, pins.

Clock ratios

There is a constraint on the ratio of the frequencies of PCLK to SSPCLK. The frequency of SSPCLK must be less or equal to that of PCLK. This ensures that control signals from the SSPCLK domain to the PCLK domain are guaranteed to get synchronized before one frame duration:

FSSPCLK <= FPCLK.

In the slave mode of operation, the SSPCLKIN signal from the external master is double-synchronized and then delayed to detect an edge. It takes three SSPCLKs to detect an edge on SSPCLKIN. SSPTXD has less setup time to the falling edge of SSPCLKIN on which the master is sampling the line.

The setup and hold times on SSPRXD, with reference to SSPCLKIN, must be more conservative to ensure that it is at the right value when the actual sampling occurs within the SSPMS. To ensure correct device operation, SSPCLK must be at least 12 times faster than the maximum expected frequency of SSPCLKIN.

The frequency selected for SSPCLK must accommodate the desired range of bit clock rates.

The ratio of minimum SSPCLK frequency to SSPCLKOUT maximum frequency in the case of the slave mode is 12, and for the master mode, it is two.

To generate a maximum bit rate of 1.8432Mbps in the master mode, the frequency of SSPCLK must be at least 3.6864MHz. With an SSPCLK frequency of 3.6864MHz, the SSPCPSR register must be programmed with a value of 2, and the SCR[7:0] field in the SSPCR0 register must be programmed with a value of 0.

To work with a maximum bit rate of 1.8432Mbps in the slave mode, the frequency of SSPCLK must be at least 22.12MHz. With an SSPCLK frequency of 22.12MHz, the SSPCPSR register can be programmed with a value of 12 and the SCR[7:0] field in the SSPCR0 register can be programmed with a value of 0. Similarly, the ratio of SSPCLK maximum frequency to SSPCLKOUT minimum frequency is 254 x 256.

The minimum frequency of SSPCLK is calculated by the following equations, both of which must be satisfied:

FSSPCLK(min) => 2 x FSSPCLKOUT(max), for master mode

FSSPCLK(min) => 12 x FSSPCLKIN(max), for slave mode.

The maximum frequency of SSPCLK is calculated by the following equations, both of which must be satisfied:

FSSPCLK(max) <= 254 x 256 x FSSPCLKOUT(min), for master mode

FSSPCLK(max) <= 254 x 256 x FSSPCLKIN(min), for slave mode.

Programming the SSPCR0 Control Register

The SSPCR0 register is used to:

• program the serial clock rate

• select one of the three protocols

• select the data word size, where applicable.

The Serial Clock Rate (SCR) value in conjunction with the SSPCPSR clock prescale divisor value, CPSDVSR, is used to derive the PrimeCell SSP transmit and receive bit rate from the external SSPCLK.

The frame format is programmed through the FRF bits and the data word size through the DSS bits.

Bit phase and polarity applicable to Motorola SPI format only are programmed through the SPH and SPO bits.

Programming the SSPCR1 Control Register

The SSPCR1 register is used to:

• select master or slave mode

• enable a loop back test feature

• enable the PrimeCell SSP peripheral.

To configure the PrimeCell SSP as a master, clear the SSPCR1 register master or slave selection bit, MS, to 0. This is the default value on reset.

Setting the SSPCR1 register MS bit to 1 configures the PrimeCell SSP as a slave. When

configured as a slave, enabling or disabling of the PrimeCell SSP SSPTXD signal is provided through the SSPCR1 slave mode SSPTXD output disable bit, SOD. You can use this in some multi-slave environments where masters might parallel broadcast.

Set the Synchronous Serial Port Enable (SSE) bit to 1 to enable the operation of the PrimeCell SSP.

Bit rate generation

The serial bit rate is derived by dividing down the input clock SSPCLK. The clock is first divided by an even prescale value CPSDVSR in the range of 2-254, and is programmed in SSPCPSR. The clock is divided again by a value in the range of 1-256, that is 1 + SCR, where SCR is the value programmed in SSPCR0.

The following equation defines the frequency of the output signal bit clock, SSPCLKOUT:

FSSPCLKOUT=FSSPCLKCPSDVR×(1+SCR)

For example, if SSPCLK is 3.6864MHz, and CPSDVSR = 2, then SSPCLKOUT has a

frequency range of 7.2kHz-1.8432MHz.

Recommendation

The SSPCLKOUT can be 24MHz as Max frequency when SSPCLK is 48MHz. But 20MHz is recommended as SSPCLKOUT Max frequency for stable CLK output.

Frame format

Each data frame is between 4-16 bits long depending on the size of data programmed and is transmitted starting with the MSB. Users can select the following basic frame types:

• Texas Instruments synchronous serial

• Motorola SPI

• National Semiconductor Microwire.

For all formats, the serial clock SSPCLKOUT is held inactive while the PrimeCell SSP is idle and transitions at the programmed frequency only during active transmission or reception of data. The idle state of SSPCLKOUT is utilized to provide a receive timeout indication that occurs when the receive FIFO still contains data after a timeout period.

For Motorola SPI and National Semiconductor Microwire frame formats, the serial frame SSPFSSOUT pin is active-LOW and is asserted and pulled-down during the entire transmission of the frame.

For Texas Instruments synchronous serial frame format, the SSPFSSOUT pin is pulsed for one serial clock period starting at its rising edge prior to the transmission of each frame. For this frame format, both the PrimeCell SSP and the off-chip slave device drive their output data on the rising edge of SSPCLKOUT and latch data from the other device on the falling edge.

Unlike the full-duplex transmission of the other two frame formats, the National Semiconductor Microwire format uses a special master-slave messaging technique which operates at half-duplex. In this mode, an 8-bit control message is transmitted to the off-chip slave when a frame begins. During this transmit, the SSP receives no incoming data. After the message has been sent, the off-chip slave decodes it and responds with the requested data after waiting one serial clock after the last bit of the 8-bit control message has been sent. The returned data can be 4-16 bits in length making the total frame length in the range of 13-25 bits.

Texas Instruments synchronous serial frame format

Figure 57 shows the Texas Instruments synchronous serial frame format for a single

transmitted frame.

Figure 57. Texas Instruments synchronous serial frame format, single transfer

In this mode, SSPCLKOUT and SSPFSSOUT are forced LOW and the transmit data line SSPTXD is tristated whenever the PrimeCell SSP is idle. When the bottom entry of the transmit FIFO contains data, SSPFSSOUT is pulsed HIGH for one SSPCLKOUT period. The value to be transmitted is also transferred from the transmit FIFO to the serial shift register of the transmit logic. On the next rising edge of SSPCLKOUT, the MSB of the 4-bit to 16-bit data frame is shifted out on the SSPTXD pin. In a similar way, the MSB of the received data is shifted onto the SSPRXD pin by the off-chip serial slave device.

Both the PrimeCell SSP and the off-chip serial slave device then clock each data bit into their serial shifter on the falling edge of each SSPCLKOUT. The received data is transferred from the serial shifter to the receive FIFO on the first rising edge of PCLK after the LSB has been latched.

Figure 58 shows the Texas Instruments synchronous serial frame format when back-to-back frames are transmitted.

Figure 58. Texas Instruments synchronous serial frame format, continuous transfers

Motorola SPI frame format

The Motorola SPI interface is a four-wire interface where the SSPFSSOUT signal behaves as a slave select. The main feature of the Motorola SPI format is that you can program the inactive state and phase of the SSPCLKOUT signal using the SPO and SPH bits of the SSPSCR0 control register.

SPO, clock polarity

When the SPO clock polarity control bit is LOW, it produces a steady state LOW value on the SSPCLKOUT pin. If the SPO clock polarity control bit is HIGH, a steady state HIGH value is placed on the SSPCLKOUT pin when data is not being transferred.

SPH, clock phase

The SPH control bit selects the clock edge that captures data and enables it to change state. It has the most impact on the first bit transmitted by either permitting or not permitting a clock transition before the first data capture edge.

When the SPH phase control bit is LOW, data is captured on the first clock edge transition.

When the SPH clock phase control bit is HIGH, data is captured on the second clock edge transition.

Figure 59 and Figure 60 show single and continuous transmission signal sequences for Motorola SPI format with SPO=0, SPH=0.

Figure 59 Motorola SPI frame format, single transfer, with SPO=0 and SPH=0

Figure 60 shows a continuous transmission signal sequence for Motorola SPI frame format with SPO=0, SPH=0.

Figure 60 Motorola SPI frame format, continuous transfers, with SPO=0 and SPH=0

In this configuration, during idle periods:

• the SSPCLKOUT signal is forced LOW

• the SSPFSSOUT signal is forced HIGH

• the transmit data line SSPTXD is arbitrarily forced LOW

• the nSSPOE pad enable signal is forced HIGH, making the transmit pad high impedance

• when the PrimeCell SSP is configured as a master, the nSSPCTLOE line is driven LOW, enabling the SSPCLKOUT pad, active-LOW enable

• when the PrimeCell SSP is configured as a slave, the nSSPCTLOE line is driven HIGH, disabling the SSPCLKOUT pad, active-LOW enable.

If the PrimeCell SSP is enabled and there is valid data within the transmit FIFO, the start of transmission is signified by the SSPFSSOUT master signal being driven LOW. This causes the slave data to be enabled onto the SSPRXD input line of the master. The nSSPOE line is driven LOW, enabling the master SSPTXD output pad.

One half SSPCLKOUT period later, valid master data is transferred to the SSPTXD pin. Now that both the master and slave data have been set, the SSPCLKOUT master clock pin goes HIGH after one additional half SSPCLKOUT period.

The data is now captured on the rising and propagated on the falling edges of the SSPCLKOUT signal.

In the case of a single word transmission after all bits of the data word have been transferred, the SSPFSSOUT line is returned to its idle HIGH state one SSPCLKOUT period after the last bit has been captured.

However, in the case of continuous back-to-back transmissions, the SSPFSSOUT signal must be pulsed HIGH between each data word transfer. This is because the slave select pin freezes the data in its serial peripheral register and does not permit it to be altered if the SPH bit is logic zero. Therefore, the master device must raise the SSPFSSIN pin of the slave device between each data transfer to enable the serial peripheral data write. On completion of the continuous transfer, the SSPFSSOUT pin is returned to its idle state one SSPCLKOUT period after the last bit has been captured.

Figure 61 shows the transfer signal sequence for Motorola SPI format with SPO=0, SPH=1, and it covers both single and continuous transfers.

Figure 61 Motorola SPI frame format, single and continuous transfers, with SPO=0 and SPH=1

In this configuration, during idle periods:

• the SSPCLKOUT signal is forced LOW

• The SSPFSSOUT signal is forced HIGH

• the transmit data line SSPTXD is arbitrarily forced LOW

• the nSSPOE pad enable signal is forced HIGH, making the transmit pad high impedance

• when the PrimeCell SSP is configured as a master, the nSSPCTLOE line is driven LOW, enabling the SSPCLKOUT pad, active-LOW enable

• when the PrimeCell SSP is configured as a slave, the nSSPCTLOE line is driven HIGH, disabling the SSPCLKOUT pad, active-LOW enable.

If the PrimeCell SSP is enabled and there is valid data within the transmit FIFO, the start of transmission is signified by the SSPFSSOUT master signal being driven LOW. The nSSPOE line is driven LOW, enabling the master SSPTXD output pad. After an additional one half SSPCLKOUT period, both master and slave valid data is enabled onto their respective transmission lines. At the same time, the SSPCLKOUT is enabled with a rising edge transition.

Data is then captured on the falling edges and propagated on the rising edges of the

SSPCLKOUT signal.

In the case of a single word transfer after all bits have been transferred, the SSPFSSOUT line is returned to its idle HIGH state one SSPCLKOUT period after the last bit has been captured.

For continuous back-to-back transfers, the SSPFSSOUT pin is held LOW between successive data words and termination is the same as that of the single word transfer.

Figure 62 shows a single transmission signal sequence for Motorola SPI format with SPO=1, SPH=0.

Figure 62 Motorola SPI frame format, single transfer, with SPO=1 and SPH=0

Figure 63 shows a continuous transmission signal sequence for Motorola SPI format with SPO=1, SPH=0. In Figure 9, Q is an undefined signal.

Figure 63. Motorola SPI frame format, continuous transfers, with SPO=1 and SPH=0

In this configuration, during idle periods:

• the SSPCLKOUT signal is forced HIGH

• the SSPFSSOUT signal is forced HIGH

• the transmit data line SSPTXD is arbitrarily forced LOW

• the nSSPOE pad enable signal is forced HIGH, making the transmit pad high impedance

• when the PrimeCell SSP is configured as a master, the nSSPCTLOE line is driven LOW, enabling the SSPCLKOUT pad, active-LOW enable

• when the PrimeCell SSP is configured as a slave, the nSSPCTLOE line is driven HIGH, disabling the SSPCLKOUT pad, active-LOW enable.

If the PrimeCell SSP is enabled and there is valid data within the transmit FIFO, the start of transmission is signified by the SSPFSSOUT master signal being driven LOW, and this causes slave data to be immediately transferred onto the SSPRXD line of the master. The nSSPOE line is driven LOW, enabling the master SSPTXD output pad.

One half period later, valid master data is transferred to the SSPTXD line. Now that both the master and slave data have been set, the SSPCLKOUT master clock pin becomes LOW after one additional half SSPCLKOUT period. This means that data is captured on the falling edges and be propagated on the rising edges of the SSPCLKOUT signal.

In the case of a single word transmission after all bits of the data word are transferred, the SSPFSSOUT line is returned to its idle HIGH state one SSPCLKOUT period after the last bit has been captured.

However, in the case of continuous back-to-back transmissions, the SSPFSSOUT signal must be pulsed HIGH between each data word transfer. This is because the slave select pin freezes the data in its serial peripheral register and does not permit it to be altered if the SPH bit is logic zero. Therefore, the master device must raise the SSPFSSIN pin of the slave device between each data transfer to enable the serial peripheral data write. On completion of the continuous transfer, the SSPFSSOUT pin is returned to its idle state one SSPCLKOUT period after the last bit has been captured.

Figure 64 shows the transfer signal sequence for Motorola SPI format with SPO=1, SPH=1, and it covers both single and continuous transfers. In Figure 64. Motorola SPI frame format, single and continuous transfers, with SPO=1 and SPH=1, Q is an undefined signal.

Figure 64. Motorola SPI frame format, single and continuous transfers, with SPO=1 and SPH=1

In this configuration, during idle periods:

• the SSPCLKOUT signal is forced HIGH

• the SSPFSSOUT signal is forced HIGH

• the transmit data line SSPTXD is arbitrarily forced LOW

• the nSSPOE pad enable signal is forced HIGH, making the transmit pad high impedance

• when the PrimeCell SSP is configured as a master, the nSSPCTLOE line is driven LOW, enabling the SSPCLKOUT pad, active-LOW enable

• when the PrimeCell SSP is configured as a slave, the nSSPCTLOE line is driven HIGH,

disabling the SSPCLKOUT pad, active-LOW enable.

If the PrimeCell SSP is enabled and there is valid data within the transmit FIFO, the start of transmission is signified by the SSPFSSOUT master signal being driven LOW. The nSSPOE line is driven LOW, enabling the master SSPTXD output pad. After an additional one half SSPCLKOUT period, both master and slave data are enabled onto their respective transmission lines. At the same time, the SSPCLKOUT is enabled with a falling edge transition. Data is then captured on the rising edges and propagated on the falling edges of the SSPCLKOUT signal.

After all bits have been transferred in the case of a single word transmission, the SSPFSSOUT line is returned to its idle HIGH state one SSPCLKOUT period after the last bit has been captured.

For continuous back-to-back transmissions, the SSPFSSOUT pin remains in its active-LOW state, until the final bit of the last word has been captured, and then returns to its idle state as the previous section describes.

For continuous back-to-back transfers, the SSPFSSOUT pin is held LOW between successive data words and termination is the same as that of the single word transfer.

National Semiconductor Microwire frame format

Figure 65 shows the National Semiconductor Microwire frame format for a single frame.

Figure 65. National Semiconductor Microwire frame format, single transfer

Microwire format is very similar to the SPI format except that transmission is half-duplex instead of full-duplex using a master-slave message passing technique. Each serial transmission begins with an 8-bit control word that is transmitted from the PrimeCell SSP to the off-chip slave device. During this transmission, the PrimeCell SSP receives no incoming data. After the message has been sent, the off-chip slave decodes it and responds with the required data after waiting one serial clock after the last bit of the 8-bit control message has been sent. The returned data is 4 to 16 bits in length, making the total frame length in the range of 13-25 bits.

In this configuration, during idle periods:

• SSPCLKOUT is forced LOW

• SSPFSSOUT is forced HIGH

• the transmit data line, SSPTXD, is arbitrarily forced LOW

• the nSSPOE pad enable signal is forced HIGH, making the transmit pad high impedance.

A transmission is triggered by writing a control byte to the transmit FIFO. The falling edge of SSPFSSOUT causes the value contained in the bottom entry of the transmit FIFO to be transferred to the serial shift register of the transmit logic and the MSB of the 8-bit control frame to be shifted out onto the SSPTXD pin. SSPFSSOUT remains LOW for the duration of the frame transmission. The SSPRXD pin remains tristated during this transmission.

The off-chip serial slave device latches each control bit into its serial shifter on the rising edge of each SSPCLKOUT. After the last bit is latched by the slave device, the control byte is decoded during a one clock wait-state and the slave responds by transmitting data back to the PrimeCell SSP. Each bit is driven onto the SSPRXD line on the falling edge of SSPCLKOUT. The PrimeCell SSP in turn latches each bit on the rising edge of SSPCLKOUT. At the end of the frame for single transfers, the SSPFSSOUT signal is pulled HIGH one clock period after the last bit has been latched in the receive serial shifter, which causes the data to be transferred to the receive FIFO.

The off-chip slave device can tristate the receive line either on the falling edge of SSPCLKOUT after the LSB has been latched by the receive shifter or when the SSPFSSOUT pin goes HIGH.

For continuous transfers, data transmission begins and ends in the same manner as a single transfer. However, the SSPFSSOUT line is continuously asserted, held LOW, and transmission of data occurs back-to-back. The control byte of the next frame follows directly after the LSB of the received data from the current frame. Each of the received values is transferred from the receive shifter on the falling edge SSPCLKOUT, after the LSB of the frame has been latched into the PrimeCell SSP.

Figure 66 shows the National Semiconductor Microwire frame format when back-to-back frames are transmitted.

Figure 66. National Semiconductor Microwire frame format, continuous transfers

Master and Slave configurations

Figure 67 shows how a PrimeCell SSP (PL022) configured as master, interfaces to a Motorola SPI slave. The SPI Slave Select (SS) signal is permanently tied LOW and configures it as a slave. Similar to the above operation, the master can broadcast to the slave through the master PrimeCell SSP SSPTXD line. In response, the slave drives its SPI MISO port onto the SSPRXD line of the master.

Figure 67. PrimeCell SSP master coupled to an SPI slave

Figure 68 shows a Motorola SPI configured as a master and interfaced to an instance of a PrimeCell SSP (PL022) configured as a slave. In this case, the slave Select Signal (SS) is permanently tied HIGH to configure it as a master. The master can broadcast to the slave through the master SPI MOSI line. In response, the slave drives its nSSPOE signal LOW.

This enables its SSPTXD data onto the MISO line of the master.

Figure 68. SPI master coupled to a PrimeCell SSP slave

SSP Flow chart

Figure 69 shows how to setting TI or Microwire mode.

Figure 69. how to setting TI or Microwire mode flow chart

Figure 70 shows how to setting SPI mode.

Figure 70. how to setting SPI mode flow chart

SSP0 Registers (Base Address : 0x4000_A000)

This section describes the SSP0 registers.

SSP0 Control register 0 (SSP0CR0)

Address offset: 0x0000

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 |  | 24 |  | 23 |  |  | 22 |  |  | 21 |  | 20 |  | 19 |  |  |  | 18 |  | 17 |  |  | 16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res |  | res |  | res |  |  | res |  |  | res |  | res |  | res |  |  |  | res |  | res |  |  | res |  |
| 15 | 14 | 13 | 12 | 11 | 10 |  | 9 |  | 8 |  | 7 |  |  | 6 |  |  | 5 |  | 4 |  | 3 |  |  |  | 2 |  | 1 |  |  | 0 |  |
| SCR |  |  |  |  |  |  |  |  |  | SPH |  |  | SPO |  |  | FRF |  |  |  | DSS |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  | R/W |  |  | R/W |  |  | R/W |  |  |  | R/W |  |  |  |  |  |  |  |  |  |  |

[3:0] DSS – Data size select:

0000 : reserved, undefined operation

0001 : reserved, undefined operation

0010 : reserved, undefined operation

0011 : 4-bit data

0100 : 5-bit data

0101 : 6-bit data

0110 : 7-bit data

0111 : 8-bit data

1000 : 9-bit data

1001 : 10-bit data

1010 : 11-bit data

1011 : 12-bit data

1100 : 13-bit data

1101 : 14-bit data

1111 : 15-bit data

[5:4] FRF – Frame Format

00 : Motorola SPI frame format

01 : TI synchronous serial frame format

10 : National Microwire frame format

11 : Reserved, undefined operation

[6] SPO – SSPCLKOUT polarity

This is applicable to Motorola SPI frame format only.

[7] SPH – SSPCLKOUT phase

This is applicable to Motorola SPI frame format only.

[15:8] SCR – Serial clock rate

The value SCR is used to generate the transmit and receive bit rate of the SSP. The bit rate is:

fSSPCLK / (CPSDVSR * (1 + SCR))

where

CPSDVSR is an even value from 2-254, programmed through the SSPCPSR register and SCR is a value from 0-255.

SSP0 Control register 1 (SSP0CR1)

Address offset: 0x0004

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | SOD | MS | SSE | LBM |
|  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W |

[0] LBM – Loop back mode:

0 : normal serial port operation enabled

1 : output of transmit serial shifter is connected to input of receive serial shifter internally

[1] SSE – Synchronous serial port enable:

0 : SSP0 operation disabled.

1 : SSP0 operation enabled.

[2] MS – Master or Slave mode select:

0 : device configured as master, default.

1 : device configured as slave.

[3] SOD – Slave-mode output disable.

This bit is relevant only in the slave mode, MS = 1. In multiple-slave systems, it is possible for a SSP0 master to broadcast a message to all slaves in the system while ensuring that only one slave drives data onto its serial output line. In such systems the RXD lines from multiple slaves could be tied together.

To operate in such systems, the SOD bit can be set if the SSP0 slave is not supposed to drive the SSPTXD line:

0 : SSP0 can drive the SSPTXD output in slave mode.

1 : SSP0 must not drive the SSPTXD output in slave mode.

SSP0 Data register (SSP0DR)

Address offset: 0x0008

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[15:0] DATA – Transmit/Receive FIFO:

Read: Read: receive FIFO.

Write: transmit FIFO.

You must right-justify data when the SSP0 is programmed for a data size that is less than 16 bits. Unused bits at the top are ignored by transmit logic. The receive logic automatically right-justifies.

SSP0 Status register (SSP0SR)

Address offset: 0x000C

Reset value: 0x0000_0003

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | BSY | RFF | RNE | TNF | TFE |
|  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W |

[0] TFE – Transmit FIFO empty, RO

0 : Transmit FIFO is not empty.

1 : Transmit FIFO is empty.

[1] TNF – Transmit FIFO not full, RO:

0 : Transmit FIFO is full.

1 : Transmit FIFO is not full.

[2] RNE – Receive FIFO not empty, RO:

0 : Receive FIFO is empty.

1 : Receive FIFO is not empty.

[3] RFF – Receive FIFO full, RO:

0 : Receive FIFO is not full.

1 : Receive FIFO is full.

[4] BSY – SSP busy flag, RO:

0 : SSP is idle.No effect

1 : SSP is currently transmitting and/or receiving a frame or the transmit FIFO is not empty.

SSP0 Clock prescale register (SSP0CPSR)

Address offset: 0x0010

Reset value: 0x0000_00000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | CPSDVSR |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

[7:0] CPSDVSR – Clock prescale divisor

This must be an even number from 2-254, depending on the frequency of SSPCLK. The least significant bit always returns zero on reads.

SSP0 Interrupt mask set or clear register (SSP0IMSC)

Address offset: 0x0014

Reset value: 0x0000_00000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | TXIM | RXIM | RTIM | RORIM |
|  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W |

[0] RORIM – Receive overrun interrupt mask:

0 : Receive FIFO written to while full condition interrupt is masked.

1 : Receive FIFO written to while full condition interrupt is not masked.

[1] RTIM – Receive timeout interrupt mask:

0 : Receive FIFO not empty and no read prior to timeout period interrupt is masked.

1 : Receive FIFO not empty and no read prior to timeout period interrupt is not masked.

[2] RXIM – Receive FIFO interrupt mask:

0 : Receive FIFO half full or less condition interrupt is masked.

1 : Receive FIFO half full or less condition interrupt is not masked.

[3] TXIM – Transmit FIFO interrupt mask:

0 : Transmit FIFO half empty or less condition interrupt is masked.

1 : Transmit FIFO half empty or less condition interrupt is not masked.

SSP0 Raw interrupt status register (SSP0RIS)

Address offset: 0x0018

Reset value: 0x0000_00004

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | TXRIS | RXRIS | RTRIS | RORRIS |
|  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W |

[0] RORRIS – Gives the raw interrupt state, prior to masking, of the SSPRORINTR interrupt

[1] RTRIS – Gives the raw interrupt state, prior to masking, of the SSPRTINTR interrupt

[2] RXRIS – Gives the raw interrupt state, prior to masking, of the SSPRXINTR interrupt

[3] TXRIS – Gives the raw interrupt state, prior to masking, of the SSPTXINTR interrupt

SSP0 Masked interrupt status register, (SSP0MIS)

Address offset: 0x001C

Reset value: 0x0000_00000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | TXMIS | RXMIS | RTMIS | RORMIS |
|  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W |

[0] RORMIS – Gives the receive over run masked interrupt status, after masking, of the SSPRORINTR interrupt

[1] RTMIS – Gives the receive timeout masked interrupt state, after masking, of the SSPRTINTR interrupt

[2] RXMIS – Gives the receive FIFO masked interrupt state, after masking, of the SSPRXINTR interrupt

[3] TXMIS – Gives the transmit FIFO masked interrupt state, after masking, of the SSPTXINTR interrupt

SSP0 Interrupt clear register (SSP0ICR)

Address offset: 0x0020

Reset value: 0x0000_00000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | RTIC | RORIC |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W |

[0] RORICS – Clears the SSPRORINTR interrupt

[1] RTIC – Clears the SSPRTINTR interrupt

SSP0 DMA control register, (SSP0DMACR)

Address offset: 0x0024

Reset value: 0x0000_00000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | TXDMAE | RXDMAE |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W |

[0] RXDMAE – Receive DMA Enable:

0 : DMA for the receive FIFO is disabled.

1 : DMA for the receive FIFO is enabled.

[1] TXDMAE – Transmit DMA Enable:

0 : DMA for the transmit FIFO is disabled.

1 : DMA for the transmit FIFO is enabled.

Register map

The following Table 38 summarizes the SSP0 registers.

Table 38 SSP0 register map and reset values

SSP1 Registers (Base Address : 0x4000_B000)

This section describes the SSP0 registers.

SSP1 Control register 0 (SSP1CR0)

Address offset: 0x0000

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 |  | 24 |  | 23 |  |  | 22 |  |  | 21 |  | 20 |  | 19 |  |  |  | 18 |  | 17 |  |  | 16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res |  | res |  | res |  |  | res |  |  | res |  | res |  | res |  |  |  | res |  | res |  |  | res |  |
| 15 | 14 | 13 | 12 | 11 | 10 |  | 9 |  | 8 |  | 7 |  |  | 6 |  |  | 5 |  | 4 |  | 3 |  |  |  | 2 |  | 1 |  |  | 0 |  |
| SCR |  |  |  |  |  |  |  |  |  | SPH |  |  | SPO |  |  | FRF |  |  |  | DSS |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  | R/W |  |  | R/W |  |  | R/W |  |  |  | R/W |  |  |  |  |  |  |  |  |  |  |

[3:0] DSS – Data size select:

0000 : reserved, undefined operation

0001 : reserved, undefined operation

0010 : reserved, undefined operation

0011 : 4-bit data

0100 : 5-bit data

0101 : 6-bit data

0110 : 7-bit data

0111 : 8-bit data

1000 : 9-bit data

1001 : 10-bit data

1010 : 11-bit data

1011 : 12-bit data

1100 : 13-bit data

1101 : 14-bit data

1111 : 15-bit data

[5:4] FRF – Frame Format

00 : Motorola SPI frame format

01 : TI synchronous serial frame format

10 : National Microwire frame format

11 : Reserved, undefined operation

[6] SPO – SSPCLKOUT polarity

This is applicable to Motorola SPI frame format only.

[7] SPH – SSPCLKOUT phase

This is applicable to Motorola SPI frame format only.

[15:8] SCR – Serial clock rate

The value SCR is used to generate the transmit and receive bit rate of the SSP. The bit rate is:

fSSPCLK / (CPSDVSR * (1 + SCR))

where

CPSDVSR is an even value from 2-254, programmed through the SSPCPSR register and SCR is a value from 0-255.

SSP1 Control register 1 (SSP1CR1)

Address offset: 0x0004

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | SOD | MS | SSE | LBM |
|  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W |

[0] LBM – Loop back mode:

0 : normal serial port operation enabled

1 : output of transmit serial shifter is connected to input of receive serial shifter internally

[1] SSE – Synchronous serial port enable:

0 : SSP1 operation disabled.

1 : SSP1 operation enabled.

[2] MS – Master or Slave mode select:

0 : device configured as master, default.

1 : device configured as slave.

[3] SOD – Slave-mode output disable.

This bit is relevant only in the slave mode, MS = 1. In multiple-slave systems, it is possible for a SSP1 master to broadcast a message to all slaves in the system while ensuring that only one slave drives data onto its serial output line. In such systems the RXD lines from multiple slaves could be tied together.

To operate in such systems, the SOD bit can be set if the SSP1 slave is not supposed to drive the SSPTXD line:

0 : SSP1 can drive the SSPTXD output in slave mode.

1 : SSP1 must not drive the SSPTXD output in slave mode.

SSP1 Data register (SSP1DR)

Address offset: 0x0008

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[15:0] DATA – Transmit/Receive FIFO:

Read: Read: receive FIFO.

Write: transmit FIFO.

You must right-justify data when the SSP1 is programmed for a data size that is less than 16 bits. Unused bits at the top are ignored by transmit logic. The receive logic automatically right-justifies.

SSP1 Status register (SSP1SR)

Address offset: 0x000C

Reset value: 0x0000_0003

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | BSY | RFF | RNE | TNF | TFE |
|  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W |

[0] TFE – Transmit FIFO empty, RO

0 : Transmit FIFO is not empty.

1 : Transmit FIFO is empty.

[1] TNF – Transmit FIFO not full, RO:

0 : Transmit FIFO is full.

1 : Transmit FIFO is not full.

[2] RNE – Receive FIFO not empty, RO:

0 : Receive FIFO is empty.

1 : Receive FIFO is not empty.

[3] RFF – Receive FIFO full, RO:

0 : Receive FIFO is not full.

1 : Receive FIFO is full.

[4] BSY – SSP busy flag, RO:

0 : SSP is idle.No effect

1 : SSP is currently transmitting and/or receiving a frame or the transmit FIFO is not empty.

SSP1 Clock prescale register (SSP1CPSR)

Address offset: 0x0010

Reset value: 0x0000_00000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | CPSDVSR |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

[7:0] CPSDVSR – Clock prescale divisor

This must be an even number from 2-254, depending on the frequency of SSPCLK. The least significant bit always returns zero on reads.

SSP1 Interrupt mask set or clear register (SSP1IMSC)

Address offset: 0x0014

Reset value: 0x0000_00000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | TXIM | RXIM | RTIM | RORIM |
|  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W |

[0] RORIM – Receive overrun interrupt mask:

0 : Receive FIFO written to while full condition interrupt is masked.

1 : Receive FIFO written to while full condition interrupt is not masked.

[1] RTIM – Receive timeout interrupt mask:

0 : Receive FIFO not empty and no read prior to timeout period interrupt is masked.

1 : Receive FIFO not empty and no read prior to timeout period interrupt is not masked.

[2] RXIM – Receive FIFO interrupt mask:

0 : Receive FIFO half full or less condition interrupt is masked.

1 : Receive FIFO half full or less condition interrupt is not masked.

[3] TXIM – Transmit FIFO interrupt mask:

0 : Transmit FIFO half empty or less condition interrupt is masked.

1 : Transmit FIFO half empty or less condition interrupt is not masked.

SSP1 Raw interrupt status register (SSP1RIS)

Address offset: 0x0018

Reset value: 0x0000_00004

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | TXRIS | RXRIS | RTRIS | RORRIS |
|  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W |

[0] RORRIS – Gives the raw interrupt state, prior to masking, of the SSPRORINTR interrupt

[1] RTRIS – Gives the raw interrupt state, prior to masking, of the SSPRTINTR interrupt

[2] RXRIS – Gives the raw interrupt state, prior to masking, of the SSPRXINTR interrupt

[3] TXRIS – Gives the raw interrupt state, prior to masking, of the SSPTXINTR interrupt

SSP1 Masked interrupt status register, (SSP1MIS)

Address offset: 0x001C

Reset value: 0x0000_00000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | TXMIS | RXMIS | RTMIS | RORMIS |
|  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W | R/W | R/W |

[0] RORMIS – Gives the receive over run masked interrupt status, after masking, of the SSPRORINTR interrupt

[1] RTMIS – Gives the receive timeout masked interrupt state, after masking, of the SSPRTINTR interrupt

[2] RXMIS – Gives the receive FIFO masked interrupt state, after masking, of the SSPRXINTR interrupt

[3] TXMIS – Gives the transmit FIFO masked interrupt state, after masking, of the SSPTXINTR interrupt

SSP1 Interrupt clear register (SSP1ICR)

Address offset: 0x0020

Reset value: 0x0000_00000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | RTIC | RORIC |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W |

[0] RORICS – Clears the SSPRORINTR interrupt

[1] RTIC – Clears the SSPRTINTR interrupt

SSP1 DMA control register, (SSP1DMACR)

Address offset: 0x0024

Reset value: 0x0000_00000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | res | res |
| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| res | res | res | res | res | res | res | res | res | res | res | res | res | res | TXDMAE | RXDMAE |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | R/W | R/W |

[0] RXDMAE – Receive DMA Enable:

0 : DMA for the receive FIFO is disabled.

1 : DMA for the receive FIFO is enabled.

[1] TXDMAE – Transmit DMA Enable:

0 : DMA for the transmit FIFO is disabled.

1 : DMA for the transmit FIFO is enabled.

Register map

The following Table 39 summarizes the SSP1 registers.

Table 39 SSP1 register map and reset values

# Document History Information

| Version | Date | Descriptions |
| --- | --- | --- |
| Ver. 1.0.0 | 18SEP2017 | Initial Release |
| Ver. 1.0.1 | 12OCT2017 | Edit Flash Chapter. |
| Ver. 1.0.2 | 26OCT2017 | Edit GPIO,PADCON Register and added Open Drain function |
| Ver. 1.0.3 | 27OCT2017 | TCKCNTR, RTR, Sn_RTR, Sn_KATMR Additional description added. |
| Ver. 1.0.4 | 09NOV2017 | Edit PADCON register description CS,DS inverted value. |
| Ver. 1.0.5 | 17NOV2017 | Edit UARTCR register description about UARTEN. |
| Ver. 1.0.6 | 29NOV2017 | Edit WZTOE Sn_KATMR description about timer trigger. |
| Ver. 1.0.7 | 26JAN2018 | Edit typo about PWM preodic mode |
| Ver. 1.0.8 | 12FAB2018 | Edit Base Address about RTC (4000_4000 -> 4000_E000) |
| Ver. 1.0.9 | 05MAR2018 | Edit UART2 description & register map. |
| Ver. 1.1.0 | 11APR2018 | Edit Systick description & PADCON register table fix Edit Register Table about SSP0, SSP1 (Add FRF) Edit DSS about SSP0, SSP1 Edit Register map about SSP0, SSP1 (Add FRF) |
| Ver. 1.1.1 | 27JUL2018 | Fixed figure 4. CRG Diagram (Input Information of WDOGCLK, TIMCLK0, TIMCLK1). Flash memory lock function Additional description added (11.1) |

Copyright Notice

Copyright 2018 WIZnet Co., Ltd. All Rights Reserved.

Technical Support: https://forum.wizwiki.io/

Sales & Distribution: sales@wiznet.io

For more information, visit our website at http://www.wiznet.io/

REXT value depends on the crystal characteristics